3GPP TSG RAN WG4 Meeting #115                                                     	      R4-2505459

Malta, April 19 – 23, 2025

Title:	Mitigating UE-to-UE Cross Link Interference (CLI)

Agenda item:	 7.25.2	General aspects (including RAN4 aspects for SBFD system parameters)

Code:	[NR_duplex_evo-Core]

Source:	Charter Communications

Document for:	 Discussion/Agreement

# Introduction

In Rel 18 study, SBFD performance was studied, and it was concluded in TR 38.858 [1] that SBFD will cause UE-to-UE interference caused by the SBFD UEs inflicting the legacy TDD UEs of a network operating in the adjacent channel. Rel. 19 normative phase started with the specific objective in the WID [2] for adjacent channel fair coexistence between the two operators.

As a result of our RAN4 TDOCs presented in February and in April,  there was some consensus that UE-tp-UE CLI is a real problem and that reducing the power will mitigate this interference.

In the February meeting in Athens, we presented [6] how a single operator owning both networks could mitigate this UE-to-UE CLI, by reducing the pMax of the SBFD UE by XdB. The RAN4 vice chair decision was to wait until after RAN1 approves this solution.

During that April RAN4 meeting companies asked for a method to identify the aggressor UE of the neighboring network and then to request the aggressor’s SBFD gNb to reduce the maximum transmit power of that specific aggressor UE.

RAN1 decision: RAN1 discussed the need to reduce pMax for SBFD UE in April but deferred the decision to send an LS from RAN1 to RAN4, asking RAN4 for more data. Since this May is the last meeting for RAN1 on Rel 19 SBFD, we cannot continue with this ping-pong discussion between RAN4 and RAN1 and propose to move on with adding a 2nd pMax for SBFD UEs.

The open question is how to apply this pMax reduction only to UE’s causing interference.  This is a RAN1 and RAN3 issue to address by coordinating and scheduling the UEs so UEs at cell edge that are assigned to SBFD would invoke the pMax reduction.  Solving this will mitigate the CLI problem:

- On one hand, limiting the pMax can help to address UE to UE CLI. Meanwhile, On the other hand, some operators including VDF and CMCC raised the SBFD UL coverage concerns if UL tx power is reduced.

a.	Note:  UE’s causing interference or victims suffering from UE to UE interference will experience UL

coverage issues as a result of such interference.

b. 	Note:  Charter and other companies pointed that repeated transmissions can enhance the lost coverage during SBFD symbols/slots.

- In the WID, also as part of the conclusion in R18 SI, UE RF requirement impacts are excluded.

- How to handle legacy UE, if it does not have CLI measurement capability? Specifically identifying the aggressor UEs of the neighboring SBFD network causing UE-to-UE CLI?

- How to address gNB-gNB info sharing to coordinate UE scheduling? This should be defined in RAN1 and in RAN3.

In this meeting, we present our proposal for mitigating UE-to-UE CLI.

# Discussion

It was agreed that UE-to-UE CLI is worst at cell edge, when the two UEs are in a cluster location (same area), as shown in Rel. 18 study, TR 38.858 [1] section 11.3.1-1.

The next diagram shows that DUD (40%, 20%, 40%) is optimal to mitigate this issue, as its IM7 noise is reduced much more then IM3 and IM5. Additionally, its IM3 and IM5 do not reach the adjacent channel.

DUD is much better than DU, since DU protects only the left side carrier, but not the right-side carrier. Also, DU creates IM3, which is much more powerful noise than IM7.  Therefore, a single operator owning both networks could mitigate this UE-to-UE CLI, by using DUD (40%, 20%, 40%), and by reducing pMax of the SBFD UE by XdB.

## Limiting Maximum UE Transmit power in SBFD symbols/slots.

As mentioned above, the benefit of allowing UEs to transmit at the maximum power level during the SBFD symbols/slots may not be beneficial to the UEs and may increase UE-to-UE CLI in the adjacent networks. This UE-to-UE CLI is worse when the UEs of adjacent legacy TDD network are at their cell edge and SBFD-Aware UEs are close to it as shown in Rel. 18 study, TR 38.858 [1] section 11.3.1-1.

To limit this increase in UE-to-UE CLI in the adjacent legacy TDD networks, it will be more efficient to the limit maximum transmit power of UEs during SBFD symbols/slots, while keeping the maximum transmit power of UEs during the legacy UL symbols/slots same as in case of legacy TDD systems as shown in the following figure:

The figure above shows that SBFD aware UEs will transmit at a higher power level than the legacy TDD UEs. This higher power is caused by the interference factor, caused by gNB-to-gNB interference, where the aggressor is TDD, and the victim is SBFD. To overcome this interference at the SBFD gNB receiver, the UE will need to boost its power. This may cause UE-to-UE CLI, where now the aggressor is the SBFD UE and the victim is TDD UE. To mitigate this UE-to-UE CLI in the adjacent networks the maximum UE transmit power should be limited to a value lower than the maximum transmit power allowed in the legacy TDD networks. Based on the above figure we propose

Transmit power=minpMax, Ptx,  &non-SBFD symbols/slotsminpMaxSBFD, Ptx,  &SBFD symbols/slots

Where:

Ptx = transmit power as defined in 38.213 [3] for different Channels (PUSCH/PUCCH/RACH/SRS)

pMax = UE configured maximum output power as specified in [8-1, 38.101-1], [8-2, 38.101-2], and [8-3. 38.101-3]

pMaxSBFD = pMax – XdB where XdB could be a backoff as determined by gNB supporting SBFD.

Limiting the UE transmit power during SBFD symbols/slots will help in mitigating UE-to-UE CLI in the adjacent legacy TDD networks. It will also save the SBFD system in unsuccessful retransmissions of screaming UEs at cell edge which cannot benefit from SBFD anyway. The gNB will have full cell range for TDD symbols, and smaller range for SBFD symbols.

The SBFD operator may set XdB = 0 to completely turn off this proposal. Alternatively, the operator may set XdB to a value that will turn off only 5% or 10% of the SBFD UE population by using a dynamic algorithm which is based on Tx power headroom reports from the UEs, as we propose at the end of this TDOC.

Proposal 1: Reduce pMax of the SBFD UE power by XdB.

Proposal 2: Send LS from RAN4 to RAN1 asking to approve our agreement

Proposal 3: A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%).

# Conclusions

Based on the discussion above we propose the following:

Proposal 1: Reduce pMax of the SBFD UE power by XdB.

Proposal 2: Send LS from RAN4 to RAN1 asking to approve our agreement

Proposal 3: A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%).

# Reference

[1] 3GPP TR 38.858, “Study on Evolution of NR Duplex Operation”

[2] RP-242772 Revised WID - Evolution of NR duplex operation Sub-band full duplex (SBFD)

[3] 3GPP TS 38.213, “Physical Layer Procedures for Control”

[4] 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception: Part 1: Range 1 Standalone”

[5] 3GPP TS 38.101-2, “User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”

[5] 3GPP TS 38.101-3, “User Equipment (UE) radio transmission and reception: Part 3: Range 1 and Range 2 Interworking operation with other radios”

[6] Charter TDOCs R4-2500474, R4-2503235