3GPP TSG-RAN-WG4 Meeting #114	R4-2505464

# St Julian's, Malta, 19th – 23rd May, 2025

# Title:                 	TP for TR37.898: addition of PC2 to DC_8_n41-n77 and DC_3_n3-n77

# Source:	SoftBank Corp.

# Agenda Item:	6.7.2

# Document for:	Approval

- Introduction

This contribution provides a text proposal for TR37.898[8] adding PC2 to the following EN-DC combinations.

- DC_8_n41-n77 with UL DC_8_n77

- DC_3_n3-n77 with UL DC_3_n77

- Reference

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- 3GPP RP-241678: “Rel-19 High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s)”.

[5] 3GPP TS38.101-3: “User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 19)”. V19.1.0

[6] 3GPP TR37.716-21-21: “E-UTRA-NR Dual Connectivity (EN-DC) of x bands (x=1,2,3,4) LTE inter-band CA (x DL/1 UL) and 2 bands NR Inter-band CA (2 DL/1 UL) band combinations (Release 16)”. V16.0.0

[7] 3GPP TR37.863-01-01: “E-UTRA (Evolved Universal Terrestrial Radio Access) – NR Dual Connectivity (EN-DC) of LTE 1Down Link (DL) / 1 Up Link (UL) and 1 NR band (Release 15)”. V15.4.0

[8] 3GPP TR37.898: “High power UE (power class 1.5 or 2) for Dual Connectivity (DC) combinations of LTE band(s) and NR band(s); (Release 19)”. V0.3.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted –

- 5.x DC_8-41_n77

- 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A-41A_n77(2A)14 DC_8A-41C_n77(2A)14 | DC_8A_n77A14 |
| NOTE1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

- 5.x.2	Maximum output power for DC

The maximum output power requirement for PC2 UL DC_8A_n77A is already specified in Table 6.2B.1.3-1 in TS38.101-3[5]. So, this section can be omitted.

- 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Interference impact is identified as follows:

-	2nd order IMD generated by dual uplink of band 8 and band n77 may fall into own Rx of band 41.

The MSD value for PC3 is defined in TS38.101-3[5] as follows:

Table 5.x.3-2: MSD test points for SCell due to dual uplink operation for PC3 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-41A_n77A DC_8A-41C_n77A | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | 41 | N/A | 5 | N/A | 2650 | 28.0 | IMD2 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | N/A |

The MSD value for PC2 is proposed as follows:

Table 5.x.3-2: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-41A_n77A DC_8A-41C_n77A | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | 41 | N/A | 5 | N/A | 2650 | 34.0 | IMD2 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | N/A |

- 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

- 5.x DC_3_n3-n77

- 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n3A-n77A5 | DC_3A_n77A14 DC_3A_n3A2 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. |  |

- 5.x.2	Maximum output power for DC

The maximum output power requirement for PC2 UL DC_3A_n77A is already specified in Table 6.2B.1.3-1 in TS38.101-3[5]. So, this section can be omitted.

- 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. The co-existence studies for PC3 are already performed for DC_3A_n3A-n77A in TR37.716-21-21[6].

- 	Uplink DC configuration is DC_3A_n77A: Harmonic problem already discussed and solved in DC_3A_n77A, CA_n3A-n77A. IMD problem already discussed and solved in DC_3A_n77A.

The co-existence studies of DC_3A_n3A and DC_3A_n77A are already captured in TR37.863-01-01[7].

- The 2nd harmonic interference from Band 3 or Band n3 UL will fall into Band n77 DL frequency range.

- The 2nd IMD and 4th IMD will be impact to Band n77 due to dual uplink transmission by DC 3A_n3A.

These issues occurred due to uplink Band 3 or Band n3. Hence, additional MSD due to high power uplink n77 is not needed.

- 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

-- Unaffected parts omitted --

-- End of TP –