# 3GPP TSG-RAN WG4 Meeting #115                                                              R4-2505469

Malta, MT, 19th – 23rd May, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                      Views on modification of existing Rx requirements for SBFD

Agenda Item:       7.25.3.3

Document for:     Discussion

# Introduction

In the RAN4 #114bis meeting, RAN4 agreed to further study, SBFD in-band blocking.

# Discussion

## In-band blocking for FR1 WA BS

In the RAN4 #114bis agreed to further study BS receiver in-band blocking with following agreement and WF [1].

- FFS on this issue - Option 1: Define the in-band blocking requirement with the same grid-shift [TBD] for both SBFD-TDD adjacent operator scenario and SBFD-SBFD adjacent operator scenario. Additional emission requirement is included in this release - Option 2: Define the in-band blocking requirement with grid-shift 20-50% for SBFD-TDD adjacent operator scenario, while additional emission requirement is not considered in Rel-19. If a decision is made to require SBFD on adjacent operators in a band, a future WI will address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario

In our previous contributions [2]–[4], we provided in-band blocking requirements for the SBFD-TDD adjacent operator scenario, including results for 10% and 100% grid shifts (GS). In this contribution, we expand upon that work by presenting additional simulation results for a 20% GS. We propose a minimum grid shift of 10% to better reflect practical deployment conditions. The SBFD-TDD scenario was thoroughly analyzed during Rel-18 and is backed by extensive simulations and data. As such, focusing Rel-19 efforts on SBFD-TDD is both efficient and practical. In contrast, introducing SBFD-SBFD at this stage would require new studies and filter design work, potentially delaying progress. Since initial SBFD deployments are expected to coexist primarily with legacy TDD systems, prioritizing SBFD-TDD offers immediate value without incurring unnecessary cost or complexity. Given the varying pace of SBFD adoption and the uncertain regulatory timelines across regions, delaying SBFD-SBFD requirements helps avoid premature over-specification. At this stage, emission mask requirements can be scoped to SBFD-TDD, with SBFD-SBFD considerations addressed in a future WI based on real-world deployment experience. We also support deferring the definition of emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario to a future WI.

Proposal 1: We propose focusing Rel-19 efforts on SBFD-TDD scenarios with at least 10% GS. SBFD-SBFD requirements should be addressed in a future WI based on deployment priorities.

### Simulation results

The simulation assumptions remain consistent with those presented in our earlier contributions [2]–[4].

#### SBFD Aggressor

We consider a coexistence scenario where UMa deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) also deploys a SBFD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the SBFD aggressor. The baseline noise figure model used in the analysis is based on the default parameters specified in 3GPP TR 38.858 Annex E [5].

Figure 1: FR1, UMa- UMa, in-band blocking, SBFD Aggressor

Table 1: FR1 SBFD UMa -- UMa conducted in-band blocking simulation results

|  | 100% grid-shift | 10% grid-shift |
| --- | --- | --- |
|  | 99% CDF | 99% CDF |
| Conducted in-band blocking (dBm) | -30.47 | -3.82 |

The in-band blocking requirement is defined for the 99%-tile of the CDF of the received UL wideband interference power. From Figure 1, it can be observed that the in-band blocking requirement is higher as the grid shift decreases. It can also be observed that for the 100% grid shift case, the 99 percentile of the in-band blocking power is 13.54 dBm, and for the 10% grid shift case, it is approximately 40.18 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS, respectively.

Observation 1: For UMa deployments with SBFD as the aggressor network and a 100% grid shift, the 99th percentile of in-band blocking power is 13.54 dBm. With a 10% grid shift, the in-band blocking power is approximately 40.18 dB above the current threshold of -43 dBm.

Figure 2: FR1, UMa—different power levels, SBFD Aggressor

In Figure 2, we have plotted the distributions of received power levels, considering 10% grid shift for this simulation scenario. We have captured adjacent channel gNB DL power level, co-channel gNB DL power level, co-channel UE-UL power level, and wanted signal level. The results show that for 10% grid shift, adjacent channel interference power is dominating the total interference. This study is also available in our previous contribution [3].

Observation 2: For a 10% grid shift, adjacent channel CLI power dominates the total in-band blocking power. gNB-to-gNB CLI handling schemes or deployment restrictions are required to mitigate this impact.

Proposal 2:  For the 10% GS requirement, interference and in-band blocking should be evaluated after RAN1 studies CLI handling for adjacent channels or relevant deployment restrictions.

#### TDD Aggressor

We consider a coexistence scenario where UMa deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator. The default noise figure model defined in the 38.858 Annex E [5] is considered as baseline.

Figure 3: FR1, UMa-- UMa, in-band blocking, TDD Aggressor [10% and 100% GS]

Figure 4: FR1, UMa-- UMa, in-band blocking, TDD Aggressor [20%  GS]

Table 2: FR1 SBFD UMa -- UMa conducted in-band blocking simulation results

|  | 100% grid-shift | 20% grid-shift | 10% grid-shift |
| --- | --- | --- | --- |
|  | 99% CDF | 99% CDF | 99% CDF |
| Conducted in-band blocking (dBm) | -28.05 | -14.10 | -0.78 |

The in-band blocking requirement is defined based on the 99th percentile of the CDF of the received UL wideband interference power. From Figures 3 and 4, it can be observed that the in-band blocking power increases as the grid shift decreases. Specifically, for the 100% grid shift case, the 99th percentile in-band blocking power is 14.95 dB higher than the specified requirement of –43 dBm. For the 20% grid shift case, this deviation increases to 28.90 dB, and for the 10% grid shift case, it reaches approximately 42.22 dB above the specified in-band blocking limit for a Wide Area Base Station.

Observation 3: The 99th percentile in-band blocking power exceeds the –43 dBm requirement by 14.95 dB, 28.90 dB, and 42.22 dB for grid shifts of 100%, 20%, and 10%, respectively—indicating increased interference as grid shift decreases.

#### Summary of Results

Table 7: FR1 UMa- UMa conducted in-band blocking simulation results

| Scenario | Aggressor | Grid Shift | 99% CDF |
| --- | --- | --- | --- |
| UMa | SBFD | 10% | -3.82 |
|  |  | 100% | -30.46 |
|  | TDD | 10% | -0.78 |
|  |  | 20% | -14.10 |
|  |  | 100% | -28.05 |

Observation 4: In the FR1 Urban Macro scenario with WA base stations and 10% GS, the current -43 dBm in-band blocking limit is inadequate. Higher blocking levels are consistently observed with both TDD and SBFD as aggressors. These results exclude gNB-to-gNB CLI mitigation, deployment constraints, and additional analog filtering. Therefore, relaxing the in-band blocking requirement is necessary to preserve receiver performance in high-interference environments.

Proposal 3:  In FR1 UMa with WA base stations and 10–20% GS, the current -43 dBm in-band blocking requirement is insufficient due to significantly elevated blocking levels. In the absence of gNB-to-gNB CLI mitigation or analog filtering, a higher in-band blocking threshold is necessary to ensure reliable system performance.

## In-band blocking for FR1 MR BS

- FFS on this issue - Option 1: The IBB requirements of FR1 micro scenario can reuse existing requirements - Option 2: Introduce a new in-band blocking requirement for FR1 MR SBFD BS of -4 dBm - Option 3: propose to define the in-band blocking requirement as -38dBm under the assumption of without any analog filtering design after the LNA of receiver chain which is close to 100% grid shift in the practical deployment - Option 4: Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs

Building upon the simulation results presented in our previous paper [4], we make the following observations regarding the FR1 MR BS.

Observation 5: In the FR1 MR BS scenario with a medium-area BS and a 10% GS, the current in-band blocking requirement of -38 dBm needs to be increased.

Proposal 4: For the FR1 MR BS scenario with a 10% GS, the current in-band blocking requirement of -38 dBm should be increased to account for the elevated levels of in-band blocking power. Additionally, we agree with Option 2 to introduce a new in-band blocking requirement of -4 dBm for FR1 MR SBFD BSs.

# Conclusion

In this contribution, we discuss about in-band blocking with the following proposals and observations.

Observation 1: For UMa deployments with SBFD as the aggressor network and a 100% grid shift, the 99th percentile of in-band blocking power is 13.54 dBm. With a 10% grid shift, the in-band blocking power is approximately 40.18 dB above the current threshold of -43 dBm.

Observation 2: For a 10% grid shift, adjacent channel CLI power dominates the total in-band blocking power. gNB-to-gNB CLI handling schemes or deployment restrictions are required to mitigate this impact.

Observation 3: The 99th percentile in-band blocking power exceeds the –43 dBm requirement by 14.95 dB, 28.90 dB, and 42.22 dB for grid shifts of 100%, 20%, and 10%, respectively—indicating increased interference as grid shift decreases.

Observation 4: In the FR1 Urban Macro scenario with WA base stations and 10% GS, the current -43 dBm in-band blocking limit is inadequate. Higher blocking levels are consistently observed with both TDD and SBFD as aggressors. These results exclude gNB-to-gNB CLI mitigation, deployment constraints, and additional analog filtering. Therefore, relaxing the in-band blocking requirement is necessary to preserve receiver performance in high-interference environments.

Observation 5: In the FR1 MR BS scenario with a medium-area BS and a 10% GS, the current in-band blocking requirement of -38 dBm needs to be increased.

Proposal 1: We propose focusing Rel-19 efforts on SBFD-TDD scenarios with at least 10% GS. SBFD-SBFD requirements should be addressed in a future WI based on deployment priorities.

Proposal 2:  For the 10% GS requirement, interference and in-band blocking should be evaluated after RAN1 studies CLI handling for adjacent channels or relevant deployment restrictions.

Proposal 3:  In FR1 Urban Macro scenarios with WA base stations and 10–20% GS, the current -43 dBm in-band blocking requirement is insufficient due to significantly elevated blocking levels. In the absence of gNB-to-gNB CLI mitigation or analog filtering, a higher in-band blocking threshold is necessary to ensure reliable system performance.

Proposal 4: For the FR1 MR BS scenario with a 10% GS, the current in-band blocking requirement of -38 dBm should be increased to account for the elevated levels of in-band blocking power. Additionally, we agree with Option 2 to introduce a new in-band blocking requirement of -4 dBm for FR1 MR SBFD BSs.

# Reference

- R4-2504752 Way Forward for [114bis][307] NR_duplex_evo_BSRF

- R4-2503299, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.

- R4-2415046, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.

- R4-2417789, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.

- TR 38.858, “Study on evolution of NR duplex operation,” 3GPP.