3GPP TSG-RAN WG4 Meeting #115        				            R4-2505474

Malta, MT, 19th - 23rd May 2025

Title:	RRM Core requirements on CSI-RS based L1 measurement for LTM

Source:	China Telecom

Agenda item:			7.29.3.2

Document for:	Discussion

- Introduction

In the RAN4 #114bis meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on CSI-RS based L1 measurement for LTM.

- Discussion

Issue 4-4: CSI-RS based intra-frequency/inter-frequency measurement definition

Candidate options: - Option 1: Apple, CMCC A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: MediaTek, ZTE, Ericsson, Nokia, Qualcomm A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP. - Option 3: Huawei If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference. If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred. - Option 4: vivo - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP. at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

In the previous meeting, we have agreed that not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. In which case, we think both option 1 and option 2 are ok. And we prefer option 2.

Proposal 1:

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

-	the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

•	It is applied for SCS = 60KHz

-	the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

-	at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1:

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

-	the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

•	It is applied for SCS = 60KHz

-	the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

-	at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

- References

[1] R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.