# 3GPP TSG-RAN WG4 Meeting #115        							        	                      R4-2505476

# Malta, MT, 19th - 23rd May 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.133 on NR mobility enhancements Phase4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-19 NR_Mob_Ph4, the mechanism of conditional LTM has been supported, the corresponding RRM requirement needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the RRM requirement on conditional LTM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corresponding RRM requirement would still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

AoA	Angle of Arrival

AoD	Angle of Departure

ATG	Air to Ground

AWGN	Additive White Gaussian Nouse

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BLER	Block Error Rate

BM-RS	Beam Management Reference Signal

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CBW	Channel Bandwidth

CC	Component Carrier

CCA	Clear Channel Assessment

CCE	Control Channel Element

CG-SDT	Configured Grant Small Data Transmission

CHO	Conditional Handover

CLI	Cross Link Interference

CLTM                 Conditional L1/L2 triggered mobility

CMR	Channel Measurement Resource

CN	Core Network

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

CSI_RP	Received (linear) average power of the resource elements that carry NR CSI-RS signals and channels, measured at the UE antenna connector

DAPS	Dual Active Protocol Stack

DBT	Discovery Burst Transmission

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DPC	Delta Power Class

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

eDRX	Extended DRX

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EMR	Early measurement reporting

EMW	Effective measurement window

EMWRP	Effective measurement window repetition period

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FH	Frequency Hopping

FR	Frequency Range

GEO	Geostationary Earth Orbit

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

HARQ	Hybrid Automatic Repeat Request

HO	Handover

HST	High Speed Train

GAP	Refers to any of Measurement Gap, activated Pre-MG and NCSG

IMR	Interference Measurement Resource

kHz	Kilo Hertz

L1-RSRP	Layer 1 RSRP

L1 SL-RSRP	Layer 1 Sidelink RSRP which corresponds to PSCCH-RSRP and/or PSSCH-RSRP

LEO	Low Earth Orbit

LMF	Location Management Function

LPP	LTE Positioning Protocol

LTM	L1/L2 triggered mobility

MAC	Medium Access Control

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MHz	Mega Hertz

MIB	Master Information Block

ML	Measurement Length

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MUSIM	Multi-Universal Subscriber Identity Module

NCSG	Network Controlled Small Gap

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NGSO	Non-Geosynchronous Orbit

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

PCell	Primary Cell

PCI	Physical Cell Identity

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

Pre-MG	Pre-configured Measurement Gap

ProSe	Proximity-based Service

PRB	Physical Resource Block

PRP	PRS Received Power

PRS	Positioning Reference Signal

PRS-RSRP	Positioning Reference Signal based Reference Signal Received Power

PPW	PRS Processing Window

PPWL	PRS Processing Window Length

PPWRP	PRS Processing Window Repetition Period

PSBCH	Physical Sidelink Broadcast Channel

PSBCH-RSRP	Physical Sidelink Broadcast Channel DMRS based Reference Signal Received Power

PSCCH	Physical Sidelink Control Channel

PSCCH-RSRP	Physical Sidelink Control Channel DMRS based Reference Signal Received Power

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared Channel

PSSCH-RSRP	Physical Sidelink Shared Channel DMRS based Reference Signal Received Power

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAN	Radio Access Network

RAT	Radio Access Technology

RF	Radio Frequency

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMC	Reference Measurement Channel

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRH	Remote Radio Head

RRM	Radio Resource Management

RRT	RF Retuning Time

RS	Reference Signal

RSCP	Reference Signal Carrier Phase

RSCPD	Reference Signal Carrier Phase Difference

RSSI	Received Signal Strength Indicator

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTD	Receive Timing Difference

RTOA	Relative Time Of Arrival

RTT	Round Trip Time

S-SSB	Sidelink Synchronization Signal Block

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector or radiated interface boundary.

SA	Standalone operation mode

SAB	Satellite access band

SAN	Satellite Access Node

SCC	Secondary Component Carrier

SCCH	Sidelink Control Channel

SCell	Secondary Cell

SCG	Secondary Cell Group

SCH	Synchronization Channel

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SDT	Small Data Transmission

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SL	Sidelink

SL AoA	Sidelink AoA

SL PRS-RSRP	Sidelink PRS-based RSRP

SL PRS-RSRPP	Sidelink PRS-based RSRPP

SL RSTD	Sidelink RSTD

SL RTOA	Sidelink RTOA

SL Rx-Tx	Sidelink Receive-Transmit time difference

SL-PRP	SL-PRS Received Power

SL-PRS	Sidelink PRS

SL-RSSI	Sidelink Received Signal Strength Indicator

SLPP	Sidelink Positioning Protocol

SLSS	Sidelink Synchronization Signal

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDCP	Time Domain Channel Properties

TDD	Time Division Duplex

TDOA	Time Difference Of Arrival

TE	Test Equipment

TN	Terrestrial Network

TRP	Transmission-Reception Point

TRS	Tracking Reference Signal

TTI	Transmission Time Interval

U2N	UE-to-Network

U2U	UE-to-UE

UE	User Equipment

UL	Uplink

V2X	Vehicle-to-Everything service

VIL	Visible Interruption Length

VIRP	Visible Interruption Repetition Period

VSAT	Very Small Aperture Terminal

End of Change 1

Start of Change 2

## 6.3.X	Conditional L1/L2-Triggered Mobility

#### 6.3.X.1	Introduction

The requirements in this clause are applicable to both intra-frequency and inter-frequency CLTM cell switch and subsequent CLTM cell switch.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighbouring CLTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to an CLTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this clause are not applicable to NR-DC scenarios.

#### 6.3.X.2	CLTM Cell Switch delay

When the UE receives a RRC message implying conditional LTM the UE shall start to transmit the the new uplink transmission within DCLTM seconds from the end of the last TTI containing the RRC command for conditional LTM.

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + TCLTM-interrupt

Where:

- TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is the time for the UE to measure and realize the condition for CLTM is fulfilled. Tmeasure equals to the measurements time stated in clause 6.3.X.2.1.

- TCLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the CLTM target cell indicated in the RRC command stated in clause 6.3.X.2.2.

- TCLTM-interrupt is the interruption time stated in clause 6.3.X.2.3.

##### 6.3.X.2.1	Measurement time

Tmeasure is the time for the UE to measure and realize the condition for CLTM is fulfilled.

1> if SSB based L1-RSRP measurement is used in the event,

2> For intra-frequency, Tmeasure is TL1-RSRP_Measurement_Period_SSB_intra stated in clause 9.14 plus TSSB_time_index_intra stated in clause 9.2.5.1 if needed.

2> For inter-frequency, Tmeasure is TL1-RSRP_Measurement_Period_SSB_Inter stated in clause 9.15 plus TSSB_time_index_inter stated in clause 9.3.4 if needed.

1> else if SSB based L3-RSRP measurement is used in the event,

2> Tmeasure in CHO stated in clause 6.1.4.2.2 is applicable.

1> FFS: else if CSI-RS based L1-RSRP measurement is used in the event,

2> Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

##### 6.3.X.2.2	Conditional LTM RRC Processing time

TCLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the CLTM target cell indicated in the RRC command command.

1> if the UE supports ltm-FastProcessingConfig-r18 capability, and

2> if the number of LTM and CLTM candidate cell configurations does not exceed maxNumberConfigs-r18 and

2> if the number of the configured serving cells and the cells in the LTM and CLTM configuration does not exceed maxNumberStoredConfigCells-r18

3> TCLTM-RRC-processing = 0

2> else:

3> if UE has received CLTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before cell switch, and/or

3> if UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before cell switch, where NT,2 is defined in section 8.1 of TS 38.213 [3], and

3> if the total number of LTM and CLTM candidate cells for which TCI state(s) were activated or PDCCH order was received before the cell switch does not exceed maxNumberConfigs-r18, and

3> if the total number of serving cells and the cells in the LTM and CLTM configuration for which TCI state(s) were activated and/or PDCCH order was received before the cell switch does not exceed maxNumberStoredConfigCells-r18.

4> TCLTM-RRC-processing = 0

1> else:

2> TCLTM-RRC-processing = 10 ms.

##### 6.3.X.2.3	Interruption time

The interruption time TCLTM-interrupt is the time between when the UE starts to execute the conditional LTM to the target cell and the time the UE starts to transmit the the new uplink transmission.

TCLTM-interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

Where:

-	TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change..

1>  i	f the UE supports ltm-FastUE-Processing-r18 capability,

2> the value of TLTM-processing equals to

3> fr1-r18 for FR1 to FR1 LTM cell switch.

3> fr2-r18 for FR2 to FR2 LTM cell switch.

3> fr1-AndFR2-r18 for FR1 to FR2 and FR2 to FR1 LTM cell switch.

1> else:

2>  the value of TLTM-processing equals to

3> 20 ms for FR1 to FR1 and FR2 to FR2 LTM cell switch.

3> 40 ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

-	 [Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell stated in clause 6.3.1.3.

-	TRS-proc is the time for SSB processing stated in clause 6.3.1.3.

-	The values and conditions of Tfirst-RS and TRS-proc are same].

-	TLTM-IU is the interruption uncertainty during LTM cell switch stated in clause 6.3.1.3.

#### 6.3.X.3	Subsequent Conditional L1/L2-Triggered Mobility delay

When the UE sends a RRC message implying the previous CLTM has completed the UE shall start to transmit the the new uplink transmission within DSubsequent-CLTM seconds from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch.

DSubsequent-CLTM = TEvent_DU + Tmeasure + TCLTM-RRC-processing + TCLTM-interrupt

Where:

-	Tmeasure, TCLTM-RRC-processing and TCLTM-interrupt are same as the those defined in clause 6.3.X.2.

## -	TEvent_DU is the delay uncertainty which is the time from when the UE successfully sends a RRCReconfigurationComplete message until a condition exists at the measurement reference point which will trigger the subsequent conditional LTM

End of Change 2