3GPP RAN4 Meeting #115		R4-2505477

St Julian, Malta, 19th – 23rd May, 2025

Agenda item:	7.26.5

Source:	Ericsson

Title:	Discussion on OTA testing for A-IoT device 1

WI/SI:	Ambient_IoT_Solutions-Core

Release:	Rel-19

Document for:	Approval

# Introduction

During RAN4 #114bis meeting the following agreements were made in WF [1] concerning A-IoT device 1 OTA testing.

This contribution provides our views related to these issues.

# Discussion

## Testing antenna and CW antenna relationship

In a real deployment scenario, both the reader (R) and the CW node can have any arbitrary directions relative to the device, i.e. ∅R∈[-180°, 180°], θR∈[-90°, 90°], ∅CW∈[-180°, 180°], θCW∈[-90°, 90°], where ∅ is the azimuth angle and θ is the elevation angle, refer to Figure 1. If we consider that the reader and the CW node can only be in front side of the device, this leads to ∅R∈[-90°, 90°], θR∈[-90°, 90°], ∅CW∈[-90°, 90°], θCW∈[-90°, 90°]. Hence, in both cases, i.e. if reader and CW node have any arbitrary position or reader and CW node can only be in font side of the device, the relative position of the reader antenna and the CWT node can have any value ∆∅=∅R-∅CW∈[-180°, 180°] and ∆θ=θR-θCW∈[-180°, 180°].

Figure 1. Representation of azimuth and elevation angles

Observation 1: Given the agreement in RAN 4 that both CW antenna and testing antenna have equal distances from device, and assuming both antennas are mounted at the same height, leads to ∆∅=∅R-∅CW∈[-180°, 180°] and ∆θ=θR-θCW=0.

Observation 2: If the device has a symmetric radiation pattern around its axis, the same performances can be obtained in the two sides of the x-axis, i.e. ∅∈[-90°, 0°] and ∅∈[0°, 90°] .

Observation 3: To limit the number of tested combinations, RAN 4 can decide on a reasonable granularity for ∆∅, ex. 30°.

Proposal 1: Device should declare whether it has a symmetric radiation pattern.

Proposal 2: If the device has a symmetric radiation pattern, ∆∅ =[-90°, -60°,-30°,0] or ∆∅=[0, 30°, 60°, 90°], ∆θ=0.

Proposal 3: If the device does not have a symmetric radiation pattern, ∆∅ =[-90°, -60°,-30°,0, 30°, 60°, 90°], ∆θ=0.

## Isolation between CW node and measurement antenna

The measurement SINR is given by SINR=SbsCWleakage+N where Sbsis the received (at the measurement antenna) backscattered signal power, CWleakage is the CW leakage signal power (at the measurement antenna) and N is the noise power. Since the CW signal is a very narrow single-tone carrier, we can assume that the backscattered signal bandwidth is very small and hence we are in an interference limited scenario. Thus, the measurement SNR reduces to SINR≈SbsCWleakage, or in dB scale SINR dB=Sbs dB-CWleakage [dB]. Hence, to have a measurement SINR of x dB, the backscattered signal should be x dB higher than the residual (after possible analogue interference cancellation) CW leakage signal. In most RF systems, an SNR exceeding 10dB is necessary for effective signal detection (with a Bit Error Rate (BER) lower than 10-3).

Observation 4: An SNR exceeding 10 dB is necessary for effective signal detection in most RF systems.

Proposal 4: The difference between backscattered signal and “residual” CW leakage should be higher than [12]dB.

## Side conditions for testing for reference sensitivity

It is very important when testing for R2D sensitivity, to make sure that the device did not respond because the received R2D signal is below the device R2D sensitivity and not because the received CW signal is below the device CW sensitivity.

Observation 5: The device should have a CW signal above its CW sensitivity when testing for R2D sensitivity.

Proposal 5: The device should declare its CW sensitivity, i.e. the CW threshold at which the device is capable to backscatter.

Proposal 6: When testing for R2D sensitivity, the received CW signal at the device antenna should be at least 3dB higher than its CW sensitivity.

## Energy conditions

It is crucial to test A-IoT device under a wireless energy source instead of an external battery or cable because it accurately reflects how passive A-IoT devices operate in the real world. Indeed, passive A-IoT device do not have large batteries. They rely entirely on harvesting energy to power up their circuits and backscatter. Using an external power source (battery or cable), the device is getting unlimited, stable power, which does not reflect real performance. Furthermore, the device backscattering behaviour (the strength and reliability of the backscattered signal) depends on harvested power. When powering the device externally, it might respond strongly even when it normally would not.

Observation 6: The device backscattering behaviour (strength and reliability) depends on harvested power.

Proposal 7: The device should declare:

- If it can harvest energy in parallel of performing R2D reception and/or D2R transmission.

- If not, charging time and discharging time for both R2D reception and D2R transmission

Proposal 8: Device testing should consider the device unavailability time due to charging time.

# Conclusions

This contribution provides our views related to OTA test methodology for A-IoT device 1. The contribution makes the following proposals.

Observation 1: Given the agreement in RAN 4 that both CW antenna and testing antenna have equal distances from device, and assuming both antennas are mounted at the same height, leads to ∆∅=∅R-∅CW∈[-180°, 180°] and ∆θ=θR-θCW=0.

Observation 2: If the device has a symmetric radiation pattern around its axis, the same performances can be obtained in the two sides of the axis, i.e. ∅∈[-90°, 0°] and ∅∈[0°, 90°] .

Observation 3: To limit the number of tested combinations, RAN 4 can decide on a reasonable granularity for ∆∅, ex. 30°.

Proposal 1: Device should declare whether it has a symmetric radiation pattern.

Proposal 2: If the device has a symmetric radiation pattern, ∆∅ =[-90°, -60°,-30°,0] or ∆∅=[0, 30°, 60°, 90°], ∆θ=0.

Proposal 3: If the device does not have a symmetric radiation pattern, ∆∅ =[-90°, -60°,-30°,0, 30°, 60°, 90°], ∆θ=0.

Observation 4: An SNR exceeding 10 dB is necessary for effective signal detection in most RF systems.

Proposal 4: The difference between backscattered signal and “residual” CW leakage should be higher than [12]dB.

Observation 5: The device should have a CW signal above its CW sensitivity when testing for R2D sensitivity.

Proposal 5: The device should declare its CW sensitivity, i.e. the CW threshold at which the device is capable to backscatter.

Proposal 6: When testing for R2D sensitivity, the received CW signal at the device antenna should be at least 3dB higher than its CW sensitivity.

Observation 6: The device backscattering behaviour (strength and reliability) depends on harvested power.

Proposal 7: The device should declare:

- If it can harvest energy in parallel of performing R2D reception and/or D2R transmission.

- If not, charging time and discharging time for both R2D reception and D2R transmission

Proposal 8: Device testing should consider the device unavailability time due to charging time.

# References

R4-2505230, “WF on A-IOT device requirements” CMCC, 3GPP RAN4 #114bis.