3GPP TSG-RAN WG4 Meeting #115	R4- 2505479

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	6.13.1

Source:	MediaTek Inc.

Title:	Discussion on NR-NTN n252 UE RF REFSENS

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration for NR-NTN n252 UE RF REFSENS is provided for discussion.

- Discussion

2.1		Spectrum WI scope for an NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario - Support channel bandwidths and SCS as presented in Table 4.1-1 below; - Mandatory support of channel raster points at step of 10 kHz; - Support UE Power class 3 (+23dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. Table 4.1-1: Channel bandwidth and SCS system parameters. SCS (kHz); Channel bandwidth (MHz). 15; 5; 10; 15; 20. 30; 10; 15; 20. 60; 10; 15; 20. The specification work of this WI shall leverage the studies and requirements for NR NTN n256, n255, n254, where applicable. All UE requirements specified as part of this WI shall be Release-independent from Rel-17.

2.2		Previous Meeting Agreements

In RAN4#114bis, the approved WF [2] provided the guidance for following works. The partial contents are listed below for reference.

Issue 1-1-3: UE REFSENS - Consider the following table with the REFSENS values for further discussions Operating band / SCS / Channel bandwidth. Operating Band; SCS kHz; 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm). 15; -99.5; -96.3; -94.5; -93.3. n252; 30; -96.6; -94.6; -93.5. 60; -97.0; -94.9; -93.7.

2.3		UE RF requirements

2.3.1	NR-NTN UE RF REFSENS for n252

In NR-NTN TS 38.101-5, UE reference sensitivity is shown below for reference.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |

In RAN4#114bis, the approved WF [3] provided n256/n255/n254 REFSENS for 3MHz CBW. The REFSENS requirements are shown below for reference.

Issue 1-1-3: UE n256/n255/n254 REFSENS for 3MHz CBW - Agreement - For band n254, 2.2 dB improvement compared with 5 MHz CBW; - For band n256, 2.2 dB improvement compared with 5 MHz CBW; - For band n255, 1.7 dB improvement compared with 5 MHz CBW;

| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) |
| --- | --- | --- | --- |
| N256 | 15 | -101.7 | -99.5 |
| N255 | 15 | -101.7 | -100 |
| N254 | 15 | -101.7 | -99.5 |

Typically, regarding REFSENS for 3MHz, -101.7dBm/3MHz is reasonable due to higher DL noise.

It is observed that n252 DL frequency range (i.e., 2180-2200 MHz) is within n256 DL frequency range (i.e., 2170-2200 MHz). Thus, it is expected that band n252 would have similar RX REFSENS performance as band n256.

Observation 1: It is expected that band n252 would have similar RX REFSENS performance as band n256.

Observation 2: It seems NR-NTN bands n256, n255, and n254 apply the same REFSENS of -101.7dBm for 3MHz CBW.

On the other hand, there is still comment from satellite company on whether to consider RX REFSENS of -100dBm/5MHz for band n252.

Observation 3: There is CR [4] about updating band n256 REFSENS as -100dBm/5MHz.

Proposal 1: Consider whether bands n252 and n256 could apply the same REFSNES for all CBW.

3 Conclusion

In this paper, our views on NR-NTN band n252 UE RF REFSENS are provided.

Observation 1: It is expected that band n252 would have similar RX REFSENS performance as band n256.

Observation 2: It seems NR-NTN bands n256, n255, and n254 apply the same REFSENS of -101.7dBm for 3MHz CBW.

Observation 3: There is CR [4] about updating band n256 REFSENS as -100dBm/5MHz.

Proposal 1: Further discuss whether bands n252 and n256 could apply the same REFSNES requirements for all CBW.

4 Reference

- RP-241661, “New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- R4-2505235, “WF on NR-NTN and IoT-NTN bands”, Apple

- R4-2504713, “WF for [114bis][310] NR_IoT_NTN_less_than_5MHz_UERF”, Xiaomi

- R4-2505480, “(NR_NTN_solutions-Core) CR to TS 38.101-5 n256 REFSENS (Rel-17)”, MediaTek Inc.