3GPP TSG-RAN WG4 Meeting #115                                                                     R4-2505495

St Julian’s, Malta, 19-23 May, 2025

Agenda item:	7.16.3

Source:	Orange, Fraunhofer

Title:	Performance comparison of high and low-resolution codebook for 32 antenna ports with respect to different subarray sizes

Release:	Rel-19

Related WI:	FS_NR_demod_SCM

Document for:	Discussion

# Introduction

Last meeting [1] the following antenna array configurations have been selected for alignment with Antenna Array Virtualization (AAV) option 3:

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

The following configurations with AAV option 1, i.e., vertical subarray of 8 antenna elements per antenna port, remain for comparison

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

  - For interested companies: (M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eType-II, for 32 TX, for comparison only

Cluster Delay Line (CDL) Antenna Array Virtualization AAV option 1 has been discussed for high resolution codebook (eTypeII) for 32 CSI-RS Antenna Ports (APs). Since vertical subarray of size 8 was agreed previously the case (M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eType-II, for 32 TX  has been added for comparison during RAN1#114 meeting.  However, the effect of the size of the subarray was unknown during the discussions. The antenna configuration (2,8,2,1,1) with AAV option 3 was also suggested for 32Tx but was not captured as such into the way forward [2].

A first goal of this contribution is to provide the Single User with PMI reporting (SU PMI) simulation results for alignment and comparison. A second goal is to investigate the effect of the subarray size on the performance of Type-I and eType-II codebooks by comparing the AAV options (16,8,2,8,1) and (2,8,2,1,1). As a benchmark we consider also the 8 CSI-RS Antenna Ports configuration (8,4,2,8,1).

The simulation parameters are given in Annex A. We adopt the CDL tables based on Table 7.7.1-3 in TR 38.901 with truncation as given in Annex A. We consider perfect channel estimation, 10Hz-3km/h and 100Hz-30km/h speed. Our simulation results rely on the MIESM PHY abstraction as described in [3].

# Simulation results

## Doppler 3km/h or 10Hz

### Simulations for alignment

#### Antenna configuration (1,2,2,1,1)

Figure 1: 4Tx AAV(1,1)

#### Antenna configuration (1,4,2,1,1)

Figure 2: 8Tx AAV(1,1)

### Simulations for comparison

#### Antenna configuration (8,4,2,8,1)

Figure 3: 8Tx AAV(8,1)

#### Antenna configuration (2,8,2,1,1)

Figure 4: 32 Tx AAV(1,1)

Figure 5: 32 TX AAV(1,1) with d=4λ between antenna elements in the vertical dimension

#### Antenna configuration (16,8,2,8,1)

Figure 6: 32Tx AAV(8,1)

## Doppler 30km/h or 100Hz

### Simulations for alignment

Antenna configuration (1,2,2,1,1)

Figure 7: 4 Tx AAV(1,1)

Figure 8: 8 Tx AAV(1,1)

Antenna configuration (1,4,2,1,1)

### Simulations for comparison

Antenna configuration (8,4,2,8,1)

Figure 9: 8 Tx AAV(8,1)

Antenna configuration (2,8,2,1,1)

Figure 11: 32 Tx AAV(1,1)

Figure 10: 32 Tx AAV(1,1) with d=4λ between antenna elements in the vertical dimension

Antenna configuration (16,8,2,8,1)

Figure 12: 32 Tx AAV(8,1)

## Observations

The following observations can be made:

Observation 1: eType-II and Type-I give similar performance for 4Tx and 8Tx

Observation 2: For 32Tx the eType-II outperforms significantly Type-I for 10Hz Doppler, it is even more the case for the configuration (2,8,2,1,1) with an inter antenna element spacing of λ2 in the vertical dimension

Observation 3: (2,8,2,1,1)  with an inter antenna element spacing of  4λ in the vertical dimension gives similar performance as  (16,8,2,8,1)

# Effect of subarray size

We consider that the antenna panel of the BS is in the plan (z,y) of the Local Coordinate System (LCS)  (x,y,z). The configuration (16,8,2,8,1) has a separation between antenna elements of d=λ2 which yields a separation of d=4λ between subarray antenna ports whose antenna radiation pattern is given by 8 antenna elements separated by λ2 steered to direction x (θtilt=90 degrees) with respect to the LCS (see Fig. 6). It is well known that large spacing between antenna port (d>λ2) produces grating lobes. Note that the mechanical down-tilt of 10 degree as no effect on the grating lobes. In the case of subarray, as soon as the desired direction in the elevation domain deviates from boresight (mechanical down-tilt), the required phase shift for each antenna element is quantized at the subarray level which implies secondary lobes due to grating lobes weighted by the subarray antenna diagram. For, example, for a steering of -5 degree away from x, it appears a grating lobe at +9.4 degree from the desired direction (the grating lobes elevation θgl  is given by  cosθgl-cosθ0 =nλd  ∀n∈N\{0} , as a result, cosθgl=cos⁡(90-5)±0.25 ). These secondary lobes are beneficial for Type-I codebook since they generate more channel diversity (the secondary lobes illuminate clusters in the elevation domain) for both (16,8,2,8,1) and (2,8,2,1,1) with antenna element spacing of d=4λ in the vertical dimension.

Figure 13: (16,8,2,8,1) equivalent to (2,8,2,1,1) with d=4λ between antenna elements in the vertical dimension

Figure 14: Comparison between 32 Tx AAV1Y & AAV3 with d=4λ between antenna elements in the vertical dimension

It is confirmed by Monte Carlo Simulations where the configuration (16,8,2,8,1) yields similar performance than (2,8,2,1,1) with antenna elements separated by 4λ in the vertical dimension. Note that the absolute (CSI-RS) beamforming gain is averaged out by the RAN4 SNR definition which can explain that we have similar performance.

Figure 15: Comparison between 32 Tx AAV1Y & AAV3

| Antenna Configuration | SNR at 50% Efficiency for Rel.16 eType-II | SNR at 50% Efficiency for Rel.15 Type-I PMI follow | SNR at 50% Efficiency for Rel.15 Type-I PMI random |
| --- | --- | --- | --- |
| (16,8,2,8,1) | 0.05 dB | 4.37 dB | 11.9 dB |
| (2,8,2,1,1) with d=4λ | -0.35 dB | 4.38 dB | 11.6 dB |
| (2,8,2,1,1) with d=λ/2 | -0.25 dB | 11.5 dB | 19.5 dB |

Table 1: required SNR at 50% normalized efficiency for the 2 AAV configurations with 10Hz

Observation 4: For 32 Tx (16,8,2,8,1) the SNR gap between eType-II and Type-I PMI follow is 4.32 dB. For 32Tx (2,8,2,1,1) the gap rises to 4.73 dB with d =4λ and 11.75 dB with d=λ/2, d distance between antenna elements in the vertical dimension.

# Conclusion

Observation 1: eType-II and Type-I give similar performance for 4Tx and 8Tx

Observation 2: For 32Tx, the eType-II significantly outperforms Type-I for a 10 Hz Doppler; this is even more the case for the configuration (2,8,2,1,1) with an inter antenna element spacing of λ/2 in the vertical dimension.

Observation 3: (2,8,2,1,1)  with inter antenna element spacing of  4λ in the vertical dimension gives similar performance as  (16,8,2,8,1)

Observation 4: For 32 Tx (16,8,2,8,1) the SNR gap between eType-II and Type-I PMI follow is 4.32 dB. For 32Tx (2,8,2,1,1) the gap rises to 4.73 dB with d =4λ and 11.75 dB with d=λ/2, d distance between antenna elements in the vertical dimension.

Proposal: To discriminate between eType-II and Type-I select configuration (2,8,2,1,1), with an inter antenna element spacing of λ/2 in the vertical dimension, where the performance gap is the largest.

# References

- R4-2504756, “Way Forward for [114bis][320] NR_SCM,” Nokia, 3GPP RAN4#114b, Apr., 2025.

- R4-2502277, “Way Forward for [114][322] NR_SCM,” Nokia, 3GPP RAN4#114, Feb., 2025.

- X. Fu, D. Le Ruyet, R. Visoz, V. Ramireddy,  M. Grossman, M. Landmann, W. Quiroga , "A Tutorial on Downlink Precoder Selection Strategies for 3GPP MIMO Codebooks," in IEEE Access, vol. 11, pp. 138897-138922, 2023 (URL).

# Annex A: Simulation parameters

| Parameter |  |  | Unit | 32TX (for information) |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD DL-UL configurations |  |  |  | FR1.30-1 as specified in Annex A |
| Antenna configuration |  |  |  | 32 x 4 (N1,N2) = (8,2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 32 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 17,(5, 12) |
|  | CSI-RS interval and offset |  | slot | 10, 0 |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured for eType II FFS Subband or Wideband for Type I |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| Codebook configuration | Codebook Type |  |  | - typeII-r16 - typeI-SP |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (8,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,4) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 7 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Measurement channel |  |  |  | R.PDSCH.2-8.3 TDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Note: Use DM-RS based FOE and compensation. |  |  |  |  |

| BS antenna configuration | (M,N,P,Ms,Ns) = (1,2,2,1,1) - 4Tx |
| --- | --- |
|  | (M,N,P,Ms,Ns) = (1,4,2,1,1) – 8Tx |
|  | (M,N,P,Ms,Ns) = (8,4,2,8,1) - 8Tx |
|  | M,N,P,Ms,Ns=(16,8,2,8,1) -32Tx |
|  | M,N,P,Ms,Ns=(2,8,2,1,1) -32Tx |
| BS antenna spacing | dH,dV=0.5,0.5 λ dH,dV=0.5,4 λ for AAV3 32Tx |
| UE antenna configuration | M,N,P,Ms,Ns=1,2,2,1,1-4Rx |
| UE antenna spacing | dH,dV=0.5, - λ |
| BS antenna polarization | (+45,-45°) |
| UE antenna polarization | (0,90°) |
| BS antenna radiation pattern | Defined in TR38.901 Table 7.3-1 |
| UE antenna radiation pattern | Omnidirectional |
| BS height | 25m |
| UE height | 1.5m |
| BS antenna orientation | α=0°, β=10°,γ=0° |
| UE antenna orientation | α=180°, β=0°,γ=0° |
| Distance BS-UE (D2D) | 100m |
| UE speed | 3km/h and 30km/h |
| UE travel direction | ϕv,θv=(65°,90°) |
| MCS/RI/PMI delay | 7ms |
| FR/carrier frequency | FR1/3.5 GHz |
| Subcarrier Spacing | μ=30 kHz |
| Number of Sub-bands (SB) | N3=14 |
| Total number of PRBs (40MHz, SCS=30kHz) | TBW=106 |
| Rank | rank 4 |
| Modulation and coding scheme (MCS) | WF compliant |
|  | MCS 7 from Table 2 for PDSCH (corresponding to MCS 13 from Table 1) |
| Retransmission type | Chase |
| Max. number of retransmissions | NRTmax=4 |
| Channel model | CDL tables based on Table 7.7.1-3 in TR 38.901 with truncation (see tables below) |
| Detector at the receiver | LMMSE |
| eType-II configuration | paramCombination-r16=6 |
| Channel Estimation | We use perfect channel estimation in the simulations. |

AAV(1,1) based on [Post-114bis] discussions

| CLID | Delay, nsec | Power, dB | AoD, deg | AoA, deg | ZoD, deg | ZoA, deg |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -97 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.5 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.5 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121 |
| 12 | 2906 | -16.9 | 55 | -1.6 | 91.8 | 61.6 |

AAV(8,1) based on [Post-114bis] discussions

| CLID | Delay, nsec | Power, dB | AoD, deg | AoA, deg | ZoD, deg | ZoA, deg |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -97 | 96.8 | 98.1 |
| 2 | 122 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 126 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 370 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 384 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 462 | -10.1 | -50.5 | 63.4 | 94.5 | 56.6 |
| 7 | 478 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 714 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 762 | -9.8 | -50.4 | -63.5 | 105.2 | 50.1 |
| 10 | 1264 | -11.7 | -60.2 | 79 | 91.7 | 51.7 |
| 11 | 2478 | -16.9 | 62.2 | -22.6 | 94 | 53.9 |
| 12 | 2678 | -16.9 | 55 | -1.6 | 91.8 | 61.6 |