3GPP TSG-RAN WG4 Meeting #115	R4-2505496

Malta, 19-23 May, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on NR FR1 7MHz CBW for UE Demod

Agenda item:	 7.8.6

Document for:	Agreement

- Introduction

According to the WID in [1] we present in this document our view on the introduction of a native 7MHz Channel Bandwidth in Rel.19.

# Views on NR FR1 7MHz CBW for UE Demod

The current status of UE performance requirements in 38.101-4, for both Demodulation and CSI reporting requirements, shows that only a subset of the natively supported Channel Bandwidths is used for the purpose of performance requirements. In particular, one or two different CBWs are associated with each SCS and used for the definition of the majority of the requirements.

Observation 1: UE Demodulation and CSI reporting requirements in the existing RAN4 spec are defined for a subset of native Channel Bandwidths, not covering all of the native possible CBW/SCS options and combinations.

There could be multiple reasons for previous choices not to cover all native CBW options, but we can further point out that, with the exception of particular cases (i.e. RAN1 features with PHY changes due to the reduction of RB allocation), there is no impact expected from a reduced CBW allocation to the baseband algorithmic processing.

Observation 2: In the absence of changes to the PHY layer (e.g. RB puncturing for CORESET 0 in 3MHz), baseband processing and algorithm implementation is not expected to be impacted by changes of the CBW, and the coverage of the existing UE Demodulation and CSI reporting coverage would not be improved in any significant measure by adding specific requirements for 7MHz CBW.

In conclusion, we believe that there would be no additional coverage provided by new Demodulation and CSI reporting requirements for a dedicated new CBW, and RAN4 should consider existing requirements already sufficient to cover the existing PHY features.

Proposal 1: RAN4 should not introduce UE Demodulation and CSI reporting requirements specific to the new native Channel BW of 7MHz, as they would not meaningfully expand the test coverage compared to the set of existing requirements.

- Conclusions

Observation 1: UE Demodulation and CSI reporting requirements in the existing RAN4 spec are defined for a subset of native Channel Bandwidths, not covering all of the native possible CBW/SCS options and combinations.

Observation 2: In the absence of changes to the PHY layer (e.g. RB puncturing for CORESET 0 in 3MHz), baseband processing and algorithm implementation is not expected to be impacted by changes of the CBW, and the coverage of the existing UE Demodulation and CSI reporting coverage would not be improved in any significant measure by adding specific requirements for 7MHz CBW.

Proposal 1: RAN4 should not introduce UE Demodulation and CSI reporting requirements specific to the new native Channel BW of 7MHz, as they would not meaningfully expand the test coverage compared to the set of existing requirements.

- References

[1] RP-250373, “Revised WID on NR FR1 7 MHz Channel Bandwidth”