3GPP TSG-RAN WG4 Meeting #115	R4-2505498

Valletta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on 6 Layer MIMO performance requirements

Agenda item:	7.1.4

Document for:	Discussion

Introduction

The ongoing work item on UE RF enhancements in Rel-19 [3] includes the following objective (highlighting added for emphasis):

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

During RAN4#114, the ongoing discussion led to an agreement that 6 Layer MIMO was feasible for FWA Devices, but not for handheld.

During RAN4#114-bis several issues were agreed [2], but many were left open; in this document Nokia provides views on some of those open issues.

In this document we present views to help define performance requirements for 6Rx devices

Discussion

Test applicability rule for 6Rx test case

Regarding the test applicability rules for 6Rx, Nokia believes that an applicability rule needs to only be applied to the 6 Layer case if absolutely required.

RAN4 shall introduce applicability rule only for 6 Layer 6Rx cases.

Test cases for 4 MIMO layers for FWA and Handheld UE

For test cases, Nokia believes that a limited set of test cases would be most useful and this should be the most challenging test case for the device, in this case one test case, corresponding to the Handheld UE case.

RAN4 shall use a single test case for 4 layers, corresponding to that of a handheld UE.

Channel for 6 Layers

For 6 layer cases, the channel will be static and benign, so a TDLD channel can be used

RAN4 shall use TDLD 30-5 ULA Low for 6 Layer

MCS for 6 Layers

For 6Layer cases, MCS13 (Table 2) should be used to provide an initial perspective; however different MCS may work better for requirements derivation

RAN4 shall MCS 13 (Table 2) initially to explore requirements derivation for 6 Layers

Precoding for 6 Layers

For precoding options, as a spatial channel model is not being used, and if a low correlation matrix is used the precoder has no impact; thus any precoder can be utilized and will achieve the same performance.

Therefore RAN4 can use any precoder, and that identified in WF from RAN4#114-bis [2] should suffice.

RAN4 to use the precoders identified in ‘option 2’ in Issue 3-3 in the WF from RAN4#114-bis

- For 1T6R (Random precoding):

H=111111

- For 2T6R (Random precoding):

H=111j-jj111-jj-j

- For 4T6R (Random precoding):

H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

- For 8T6R (Fixed precoding with i1,1=i1,2=i2=0):

H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Channel for 4 Layers

For 4 layer cases, the channel will be representative of mobile user with a handheld UE, so TDLC 300-100 channel should be used

RAN4 shall use TDLC 300-100 for 4 Layer

MIMO Correlation Matrix for 4 Layers

For 4 layer cases, ULA Medium B can be used, as this provides some indication for the correlation that might be achieved between antenna elements (noting that SCM should be used if it was available).

RAN4 shall use ULA Medium B for 4 Layer

MCS for 4 Layers

For 4 Layer cases, MCS13 (Table 2) should be used to provide an initial perspective; however different MCS may work better for requirements derivation

RAN4 shall MCS 13 (Table 2) initially to explore requirements derivation for 4 Layers

Conclusion

The paper discusses the topic of 6 Layer MIMO evaluation in RAN4 for performance requirements derivation, and presents some of the Nokia views on this topic for discussion during RAN4#115.

The following Observations and Proposals were made:

Proposal 1: RAN4 shall introduce applicability rule only for 6 Layer 6Rx cases.

Proposal 2: RAN4 shall use a single test case for 4 layers, corresponding to that of a handheld UE.

Proposal 3: RAN4 shall use TDLD 30-5 ULA Low for 6 Layer

Proposal 4: RAN4 shall MCS 13 (Table 2) initially to explore requirements derivation for 6 Layers

Proposal 5: RAN4 to use the precoders identified in ‘option 2’ in Issue 3-3 in the WF from RAN4#114-bis

Proposal 6: RAN4 shall use TDLC 300-100 for 4 Layer

Proposal 7: RAN4 shall use ULA Medium B for 4 Layer

Proposal 8: RAN4 shall MCS 13 (Table 2) initially to explore requirements derivation for 4 Layers

- References

- R4-2417095, “WF on requirements for 6Rx”, AT&T, RAN4#112-bis, Hefei, October  2024

- R4-2504750, “WF for 6Rx UE performance requirements”, Huawei, RAN4#114-bis, Wuhan, April 2025

- RP-241656,  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024