3GPP TSG-RAN WG4 Meeting #115	R4-2505503

Valletta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on IoT NTN SAN demodulation performance

Agenda item:	7.32.6

Document for:	Discussion

Introduction

In this document Nokia presents some perspectives on the ongoing Rel-19 work item on NR IoT NTN for less than 5 MHz.

Some views are presented based on open issues from the WF from RAN4#114-bis [1].

Discussion

PUSCH

For NR IoT NTN the performance requirements for less than 5 MHz will have negligible difference, similar to that when moving from 5 MHz to 3 MHz in TN. Therefore there is no need to define PUSCH requirements for less than 5 MHz and indeed the same requirement can be reused.

Furthermore, an applicability rule should be applied such that a SAN which passes the 5 MHz requirement is suitable for 3 MHz deployments

It is expected that it will be unlikely that a SAN will be deployed that will only utilize a 3 MHz BW channel.

RAN4 shall not introduce new PUSCH performance requirements for less than 5MHz IoT NTN

RAN4 should consider the introduction of an applicability rule for PUSCH

PUCCH

For PUCCH the reduced frequency hopping bandwidth that is enforced with a BW of 3 MHz means that some formats are negatively impacted in terms of performance. This was shown in the TN work item in Rel-18 to be PUCCH Format 2; thus requirements may wish to be defined to this in 3 MHz channels.

RAN4 shall define PUCCH format 2 cases for SAN demodulation

PRACH

For NR IoT NTN the performance requirements for less than 5 MHz will have negligible difference, similar to that when moving from 5 MHz to 3 MHz in TN. Therefore there is no need to define PRACH requirements for less than 5 MHz and indeed the same requirement can be reused.

RAN4 shall not introduce new PUSCH performance requirements for less than 5MHz IoT NTN

RAN4 should consider the introduction of an applicability rule for PUSCH

Conclusion

The paper discusses the topic of 6 Layer MIMO evaluation in RAN4 for performance requirements derivation, and presents some of the Nokia views on this topic for discussion during RAN4#115.

The following Observations and Proposals were made:

Proposal 1: RAN4 shall not introduce new PUSCH performance requirements for less than 5MHz IoT NTN

Proposal 2: RAN4 should consider the introduction of an applicability rule for PUSCH

Proposal 3: RAN4 shall define PUCCH format 2 cases for SAN demodulation

Proposal 4: RAN4 shall not introduce new PUSCH performance requirements for less than 5MHz IoT NTN

Proposal 5: RAN4 should consider the introduction of an applicability rule for PUSCH

- References

- R4-2504753, “WF for NR IoT NTN less than 5MHz demod”, Ericsson, RAN4#114-bis, Wuhan, May 2025