3GPP TSG-RAN WG4 Meeting #115		R4-2505506

St Julian, Malta 19th – 23rd May, 2025

Agenda Item:	7.18.3.2

Source: 		Qualcomm Incorporated

Title: 			Enhancement on FR2 CSSF

Document for:	Discussion

# 1.	Introduction

In this paper, we present our view on the enhancement of FR2 CSSF.

# 2.  Past Agreements

Following agreements happened during the last couple of meetings [1].

Issue 1-1: Applicability requirement of L3 measurement delay reduction by optimizing CSSF Agreement: - the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios. - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only Details of whether/how the CSSF factor is modified can be left to CR discussion Agreement in online session: - Solution 3 includes NR-DC of FR1 and FR1 + FR2 Agreement in online session: - This requirement applies regardless of whether measurement gap is configured or not. Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG. Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap. Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing Agreement: - RAN4 to introduce new individual capability(s) for CSSF reduction in R19. - delay the capability detail discussion to the end of the core part. ### Issue 2-7: Other WID scope discussion Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable

# 3. 	Discussion

## Restriction regarding configuration of 3 searchers and per FR gap

RAN4 kept the following issue open during the last meeting.

- FFS: Other restriction (triggered by Huawei): - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple) - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple) - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT) - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

UE typically reserves one searcher for measurement in gaps. If UE supports three searchers and per FR gap and if all SMTC occasions of outside gap MO in one FR overlap with the per-FR MG in another FR, UE will have to effectively utilize four searchers to meet measurement requirements in two FRs. This would be quite stringent for the UE and spec should allow UE to avoid this complexity.

Option 1 of the FFS is very restrictive for the network because it calls this configuration invalid. Option 2 will lead to poor performance since UE does not have to meet any requirement with this network configuration. Option 3 is more reasonable. It allows network flexibility, does not increase UE’s burden and ensures that 3 searcher capable UE’s performance does not degrade below legacy requirement with this network configuration.

Observation 1: If UE supports 3 searchers and per FR gap and if all SMTC occasions of outside gap MO in one FR overlap with the per-FR MG in another FR, UE will have to effectively utilize four searchers to meet measurement requirements in two FRs. This would be quite stringent for the UE.

Observation 2: Option 1 of the FFS is very restrictive for the network because it calls this configuration invalid. Option 2 will lead to poor performance since UE does not have to meet any requirement with this network configuration.

Observation 3: Option 3 allows network flexibility, does not increase UE’s burden and ensures that three searcher capable UE’s performance does not degrade below legacy requirement with this network configuration.

Proposal 1: RAN4 supports option 3 of above FFS, i.e., the following:

- UE can support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR.

# 4.	Conclusion

Observation 1: If UE supports 3 searchers and per FR gap and if all SMTC occasions of outside gap MO in one FR overlap with the per-FR MG in another FR, UE will have to effectively utilize four searchers to meet measurement requirements in two FRs. This would be quite stringent for the UE.

Observation 2: Option 1 of the FFS is very restrictive for the network because it calls this configuration invalid. Option 2 will lead to poor performance since UE does not have to meet any requirement with this network configuration.

Observation 3: Option 3 allows network flexibility, does not increase UE’s burden and ensures that 3 searcher capable UE’s performance does not degrade below legacy requirement with this network configuration.

Proposal 1: RAN4 supports option 3 of above FFS, i.e., the following:

- UE can support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR.

# 5.  Reference

[1] R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN3 #113.

[2] 3GPP TS 38.331 V 18.2.0

[3] R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, 3GPP TSG-RAN WG4 #114.