3GPP TSG-RAN4 Meeting #115		R4-2505509

# St Julian, Malta 19th – 23rd May, 2025

Source: 	Qualcomm Incorporated

Title: 	Generalization Aspects of AI-ML

Agenda item:	7.21.2

Document for:	Discussion

# Introduction

RAN4 made the following agreements during the last meeting [4]

Agreement: - Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner - FFS on which use cases the requirement will be defined for - FFS on the details for each use case in WID Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality Agreement: - FFS on whether the deactivation requirements will be introduced - The starting point is when the UE receives the signalling to commence deactivation - FFS when the endpoint for deactivation is - FFS whether fallback delay is the same as deactivation delay or has some other components Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting Agreements - There shall be consistency between applicable conditions signalling and testing - No test will be defined that implies a change of network condition or associated ID during the test. Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario.

# Discussion

## Performance Monitoring

RAN4 made the following agreement during the last meeting regarding performance monitoring reporting

Agreement: - Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner - FFS on which use cases the requirement will be defined for - FFS on the details for each use case in WID

RAN1 made the following agreement performance monitoring during the last meeting:

Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model: - Support the size of a set for monitoring is the same as the size of Set A, - The n-th resource in the set for monitoring is linked to the n-th resource in Set A. - Support the size of a set for monitoring is smaller than the size of Set A

RAN4 made the following agreement regarding inference metric during the last meeting

RAN4 114bis Agreement - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

RAN1 has already defined a metric for AI-ML performance monitoring. This metric is similar to RAN4’s defined metric for inference. Hence, RAN4 can mostly reuse RAN1’s defined metric.

Observation 1: RAN1 has already selected a metric for UE sided AI-ML BM performance monitoring. This is similar to the inference metric that RAN4 agreed recently. Hence, RAN4 can mostly reuse RAN1 metric.

Observation 2: RAN1 metric allows the top-K predicted beams to contain one of the top M strongest beams where the value of M is configurable between 1 or 2. RAN4 can focus on M = 1 to reduce the number of tests.

Proposal 1: For UE sided AI-ML BM type 1 option 2 performance monitoring, RAN4 uses the following RAN1 defined metric:

  - At least the top beam of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling)

    - Value of K is FFS

    - Where Top beam is the best beam based on L1-RSRP measurements of the resource set(s) for monitoring

RAN1 has allowed the size of performance monitoring set to be equal or smaller to that of setA. If gNB knows that a particular UE is not expected to “see” a subset of setA beams, gNB does not need to include that subset of setA in performance monitoring set. This reduces the complexity of performance monitoring process since it allows UE to measure a subset of setA resources.

It is not clear how a test equipment vendor would determine the subset of setA resources that a UE cannot “see” during a RAN4 test. Hence, RAN4 can start focusing on the performance monitoring scenario where the size of performance monitoring set is same as that of setA.

Figure 1 shows a scenario where performance monitoring set is equal to setA.

Observation 3: RAN1 has allowed performance monitoring set to be of same size or reduced size compared to setA.

Observation 4: If the size of performance monitoring set is smaller than that of setA, UE’s top K predicted beams for setA may not contain one of the top M beams of resource set for monitoring even if UE’s prediction is fully accurate.

Proposal 2: For defining the metric of Type 1 Option 2 AI-ML BM performance monitoring, RAN4 assumes that the size of a set for monitoring is the same as the size of Set A and the n-th resource in the set for monitoring is linked to the n-th resource in Set A.

- FFS: Defining metric for the scenario where performance monitoring set is smaller than setA.

Figure 1 Performance monitoring set equal to Set A for performance monitoring instances (ref: [3])

RAN1 is considering both periodic and event triggered AI-ML BM type 1 option 2 performance monitoring report. RAN1 has not yet determined the exact details of these performance monitoring reports. RAN4 should wait for RAN1 to define the reporting of AI-ML BM type 1 option 2 performance monitoring before defining RAN4 core and performance requirements.

Observation 5: RAN1 is considering both periodic and event-triggered AI-ML BM type 1 option 2 performance monitoring report.

Proposal 3: RAN4 waits for RAN1 to define the reporting of AI-ML BM type 1 option 2 performance monitoring.

## Concurrency

RAN1 is introducing different feature groups for Rel-19 AI-ML

RAN4 has started discussing inference delay of AI-ML beam management. Similar discussion for inference delay of AI-ML CSI prediction has started in RAN1 and that inference delay is expected to influence RAN4 CSI prediction tests.

Theoretically, network may configure UE to perform multiple different AI-ML tasks (e.g., CSI PMI prediction and beam prediction) simultaneously. Inference delay of each individual AI-ML task will depend on the concurrent processing of multiple AI-ML BM tasks since UE may have to pre-empt one AI-ML task to complete the other. If concurrent processing is introduced in RAN4 tests, TE needs to check if UE is pre-empting AI-ML tasks properly and adjusting inference delays accordingly or not.

To avoid complexity, RAN4 Rel-19 tests should not assume concurrent processing of multiple spec defined features, i.e., UE should not have to activate more than one Rel-19 AI-ML feature group during a Rel-19 test.

Observation 6: Inference delay of each individual AI-ML task will depend on the concurrent processing of multiple AI-ML BM tasks since UE may have to pre-empt one AI-ML task to complete the other. If concurrent processing is introduced in RAN4 tests, TE needs to check if UE is pre-empting AI-ML tasks properly and adjusting inference delays accordingly or not.

Proposal 4: RAN4 Rel-19 tests don’t assume concurrent processing of multiple spec defined AI-ML features, i.e., UE does not have to activate more than one Rel-19 AI-ML feature group during a Rel-19 test.

# Conclusions

Observation 1: RAN1 has already selected a metric for UE sided AI-ML BM performance monitoring. This is similar to the inference metric that RAN4 agreed recently. Hence, RAN4 can mostly reuse RAN1 metric.

Observation 2: RAN1 metric allows the top-K predicted beams to contain one of the top M strongest beams where the value of M is configurable between 1 or 2. RAN4 can focus on M = 1 to reduce the number of tests.

Observation 3: RAN1 has allowed performance monitoring set to be of same size or reduced size compared to setA.

Observation 4: If the size of performance monitoring set is smaller than that of setA, UE’s top K predicted beams for setA may not contain one of the top M beams of resource set for monitoring even if UE’s prediction is fully accurate.

Observation 5: RAN1 is considering both periodic and event-triggered AI-ML BM type 1 option 2 performance monitoring report.

Observation 6: Inference delay of each individual AI-ML task will depend on the concurrent processing of multiple AI-ML BM tasks since UE may have to pre-empt one AI-ML task to complete the other. If concurrent processing is introduced in RAN4 tests, TE needs to check if UE is pre-empting AI-ML tasks properly and adjusting inference delays accordingly or not.

Proposal 1: For UE sided AI-ML BM type 1 option 2 performance monitoring, RAN4 uses the following RAN1 defined metric:

  - At least the top beam of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling)

    - Value of K is FFS

    - Where Top beam is the best beam based on L1-RSRP measurements of the resource set(s) for monitoring

Proposal 2: For defining the metric of Type 1 Option 2 AI-ML BM performance monitoring, RAN4 assumes that the size of a set for monitoring is the same as the size of Set A and the n-th resource in the set for monitoring is linked to the n-th resource in Set A.

- FFS: Defining metric for the scenario where performance monitoring set is smaller than setA.

Proposal 3: RAN4 waits for RAN1 to define the reporting of AI-ML BM type 1 option 2 performance monitoring.

Proposal 4: RAN4 Rel-19 tests don’t assume concurrent processing of multiple spec defined AI-ML features, i.e., UE does not have to activate more than one Rel-19 AI-ML feature group during a Rel-19 test.

# References

- RAN4 Chair’s Notes, 3GPP TSG RAN WG4 #114bis

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #120bis

- R1-2502829, Specification support for AI-ML based beam management, Qualcomm Incorporated, 3GPP RAN1 120bis.