3GPP TSG-RAN WG4 Meeting #115                                           R4-2505519

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS

Agenda Item:	7.28.3.3

Document for:	Approval

Introduction

Based on last meeting discussion for Rel-19 LP-WUS, the issues to be further studied on Rx requirements other than REFSNES, ACS and ASCS are captured in WF [1].

This contribution provides further analysis on other Rx requirements including IBB/OBB, intermodulation, spurious emissions and spurious response. Draft requirements for LP-WUR type 1 and type 2 UE are also attached in the Annex for further discussion. We made some update for the proposed requirements in the Annex based on WF in last meeting regarding the interferer level.

Discussion

IBB, OBB and Narrow Band Blocking

Regarding IBB and OBB, the main progress is that the interference level is the same as MR.

Issue 3-1-1: IBB, OBB requirements

Agreements:

  - Same interference level as MR. further discuss and decide next meeting on relaxed wanted signal.

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. Noted that spurious response is a measure to avoid failure for blocking test due to instinct character of some UE architectures design, e.g. IF architecture. In that sense, it is reasonable to keep the spurious response requirement for LP-WUR as well.

In the specification, the absolute interferer levels are adopted for both in-band blocking (IBB) and out-of-band blocking (OBB). IBB refers to interference that occurs within the second adjacent and next adjacent channels, while OBB occurs outside a frequency range 15 MHz below or above the UE receive band. The interference of IBB and OBB are higher than ACS. The interferers relevant to the wanted signal are illustrated in the figure below:

Figure 1: Interference of ACS, IBB and OBB

The main challenges for receiver design in presence of strong interferer are the Rx chain dynamic range as well as the filter suppression capability, which could be the channel filters to alleviate interfering by ACS or IBB, or could be the band filter/duplexer to suppress OBB.

Regarding the dynamic range, the main determining components are ADC and AGC. The more ADC bits, the larger dynamic range for the receiver. However, the ADC bits cannot be increased without limitation. The relationship between ADC bit depth and power consumption is typically inversely proportional. Higher bit-depth ADCs generally offer greater precision and resolution but tend to consume more power due to the increased complexity of circuitry required for finer quantization and digitalization. Conversely, lower bit-depth ADCs consume less power but sacrifice resolution and dynamic range.

In the context of 5G smartphones, ADCs with 12 bits or higher are commonly employed. These higher precision ADCs are favoured to accommodate the demanding requirements of 5G communication, such as higher data rates and broader spectrum ranges. While some 5G smartphones may opt for even higher bit-depth ADCs like 14 or 16 bits for enhanced performance, the general trend revolves around 12 bits.

However, there's a trade-off to consider. LP-WUS which aims to minimize power consumption in smartphones, often utilize lower bit-depth ADCs to reduce power consumption. While this approach yields power-saving benefits, it may compromise the dynamic range and overall performance of the ADC. During the SI evaluation, 4 bits, 6 bits and 8 bits are considered for LLS simulation. Apparently, if low power consumption is the main target for LP-WUR design, it may not be possible that a LP-WUR could adopt the same ADC as MR, which means the dynamic range for LP-WUR could be sacrificed to some extent.

Apart from ADC, the dynamic range of AGC is another main factor which could have impact on the possibly large interferer handled by LR. As we discussed previously, the AGC dynamic range could be much lower compared to that of MR for sake of power consumption design.

On the other hand, due to the limited suppression capability of the LP-WUR filters, it may not guarantee the same level of resistance against IBB and OBB interference as the main receiver (MR). Or if we still keep the same interference levels as MR, the wanted signal for blocking requirements should be relaxed more instead.

Observation 1: LP-WUR may have less dynamic range and capability to resist against strong blocking interference as MR could do owing to the tradeoff for the implementation of low power consumption for both of the LP-WUR types.

Proposal 1: For IBB, given the implementation constraints due to low power consumption of LR, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR. Consequently, 18dB and 12dB IBB REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively.

Similar to ACS, different cases are considered for IBB, typically Case 1 and Case 2 for NR operating bands. The power difference between Case 2 and Case 1 is 12 dB, reflecting filtering effects with further frequency offsets and the dynamic range of the receiver. However, due to RF impairments impact, keeping MR like interference levels for Case 1 and Case to for IBB would result more relaxed REFSENS for LP-WUR comparted to MR.

Table 1: In-band blocking parameters for LP-WUR type 1 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 18 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 2: In-band blocking parameters for LP-WUR type 2 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 12 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 3: In-band blocking for LP-WUR

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | -56 | -44 |
| Finterferer (offset) | MHz | – FIoffset, case 1 and + FIoffset, case 1 | – FIoffset, case 2 and + FIoffset, case 2 |

Regarding OBB, with suppression of the out-of-band interferer by band filter and the analog filter before ADC, the blocking could be suppressed to relatively low level even with less dynamic range of LR, unless band filter is not considered for better coverage with less insertion loss for LR design. According to our evaluation, at least 6dB relaxation should be considered for OBB, either to reduce the interferer level or to increase the wanted signal level. Keeping the same Pinterferer as MR, the REFSENS should be relaxed as REFSENS + 18dB and REFSENS + 12dB for type 1 and type 2 respectively.

In addition to the REFSENS degradation level and the interferer level, the allowed exceptions for spurious responses should also be taken into account for LP-WUR. Due to the constraints of low-cost implementation, the ability to restrict harmonics may be inferior compared to MR. Consequently, the number of exceptions may need to be increased to accommodate this limitation.

Proposal 2: For OBB, 18dB and 12dB REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively. Additionally, the number of exceptions for spurious response may need to be increased to account for limitations arising from low-cost implementation constraints.

As for Narrow Band Blocking, it was initially defined to address coexistence scenarios with GSM at a 200 kHz frequency offset, inherited from LTE for NR. We also apply this requirement to LP-WUS. Given the close offset to the desired signal, NBB can significantly degrade REFSENS, particularly if the phase noise of the interferer is poor. If keeping the same Puw (CW) level as MR, the relaxed sensitivity level as REFSENS + 34 dB and REFSENS + 22 dB for Type 1 and Type 2 UE are proposed respectively.

Table 4: Narrow Band Blocking or LP-WUR Type 1 UE

| Parameter | Unit | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + 34 |
| Puw (CW) | dBm | -55 |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 5: Narrow Band Blocking or LP-WUR Type 2 UE

| Parameter | Unit | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + 22 |
| Puw (CW) | dBm | -55 |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Proposal 3: For NBB, due to proximity of the NBB to LP-WUS signal and worse phase noise for LR, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR. Consequently, 34dB and 22dB NBB REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively.

Spurious response

The source of generating spurious response includes mirror frequency interference, intermediate frequency interference, high-order mixing products of mixers, and other Nyquist zone aliasing of ADCs that happen to fall within the receiving band. These interferences cannot be processed by the receiver, thus causing a decrease in sensitivity. The only way to address this is by selecting appropriate intermediate frequency points, ADC sampling frequencies, and mixers with high-order suppression to reduce the number of spurious response points.

Spurious response reflects situations where the UE receiver architecture generates response points that cannot be adequately processed by the receiver. Blocking requirements cannot pass the test on these scattered points, therefore, spurious response can serve as a remedial measure for blocking tests.

When encountering spurious response, the level of the interfering signal is uniformly relaxed to -44dBm, while the wanted signal remains the same as that for out-of-band blocking. Since spurious response is considered together with OBB, similar relaxation as OBB should be considered for spurious response.

Proposal 4: spurious response as a remedial measure for blocking tests needs to be considered for LP-WUR, and it should be relaxed for the LP-WUR types similar to OBB. Thus, 18dB and 12dB spurious response REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively.

Similar to that of IBB, if the interferer level is relaxed several dBs, the spurious response should be relaxed as well.

Table 9: Spurious response parameters for LP-WUR Type 1 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | PREFSENS + 18dB |

Table 10: Spurious response parameters for LP-WUR Type 2 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | PREFSENS + 12dB |

Table 11: Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

Intermodulation

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal

This requirement is designed to verify the linearity of the Rx chain. Similar to the analysis of blocking requirement, the implementation of LR, specifically the linearity of the receiver may not as good as that for MR, thus, if the interferer level of blocking is relaxed, similar relaxation should be considered for intermodulation as well.

Proposal 5: If the blocking interferer is relaxed to accommodate the low power design for LP-WUR types, intermodulation requirement could also be relaxed for LP-WUR in terms of REFSENS degradation. 24dB and 12dB intermodulation REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively.

Table 6: Wide band intermodulation parameters for LP-WUR Type 1 UE

| Rx parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS bandwidth, per CC | dBm | REFSENS + 24 dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -2BWChannel / +2BWChannel |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier with channel bandwidth ≥ 5 MHz. |  |  |

Table 7: Wide band intermodulation parameters for LP-WUR Type 2 UE

| Rx parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS bandwidth, per CC | dBm | REFSENS + 12 dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -2BWChannel / +2BWChannel |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier with channel bandwidth ≥ 5 MHz. |  |  |

Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. In current specification, the power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table below:

Table 8: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30 MHz f < 1 GHz | 100 kHz | -57 dBm |  |
| 1 GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 2 |
| 12.75 GHz – 26 GHz | 1 MHz | -47 dBm | 3 |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. NOTE 2: Applies for Band that the upper frequency edge of the DL Band more than 2.69 GHz. NOTE 3: Applies for Band that the upper frequency edge of the DL Band more than 5.2 GHz. |  |  |  |

For receiver, the emissions at the antenna connector usually come from the reverse LO leakage. Since the emissions could be considered as kind of regulatory requirements, the same levels for MR should also be defined for LR.

Proposal 6: The same level of receiver spurious emissions for MR shall be defined for LR as well regardless of the LP-WUR types.

Conclusion

This contribution provides further consideration on IBB/OBB/NBB, spurious response, intermodulation and spurious emissions requirements for LP-WUR Type 1 and Type 2 UE.

Observation 1: LP-WUR may have less dynamic range and capability to resist against strong blocking interference as MR could do owing to the tradeoff for the implementation of low power consumption for both of the LP-WUR types.

Proposal 1: For IBB, given the implementation constraints due to low power consumption of LR, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR. Consequently, 18dB and 12dB IBB REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively.

Proposal 2: For OBB, 18dB and 12dB REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively. Additionally, the number of exceptions for spurious response may need to be increased to account for limitations arising from low-cost implementation constraints.

Proposal 3: For NBB, due to proximity of the NBB to LP-WUS signal and worse phase noise for LR, keeping the similar interferer level as MR means REFSENS should be relaxed more compared to MR. Consequently, 34dB and 22dB NBB REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively.

Proposal 4: Spurious response as a remedial measure for blocking tests needs to be considered for LP-WUR, and it should be relaxed for the LP-WUR types similar to OBB. Thus, 18dB and 12dB spurious response REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively.

Proposal 5: If the blocking interferer is relaxed to accommodate the low power design for LP-WUR types, intermodulation requirement could also be relaxed for LP-WUR in terms of REFSENS degradation. Consequently, 24dB and 12dB intermodulation REFSENS degradation are proposed for LP-WUR Type 1 and Type 2 UE respectively.

Proposal 6: The same level of receiver spurious emissions for MR shall be defined for LR as well regardless of the LP-WUR types.

To facilitate the discussion, draft requirements for IBB/NBB/OBB, spurious response, intermodulation and spurious emissions for LP-WUR are presented in Annex for reference. The requirements are separated for two LP-WUR types and the definitions and discussion of LP-WUR types can be found in [2]. The suffix 'M' is tentatively used here for LP-WUR and may be revised if conflicts arise with the introduction of other features.

- References

[1] R4-2505234, WF on UE RF requirements for LP-WUS, vivo

[2] R4-2505518, Further consideration on UE RF REFSENS, ACS, ASCS requirements for Rel-19 LP-WUS, Huawei, HiSilicon

- Annex (draft requirements for IBB/NBB/OBB and others)

Editor’s note: the draft requirements are for information, which are subjected to further revisions according to the standard progress.

- 7.6M	Blocking characteristics for LP-WUR

- 7.6M.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

- 7.6M.2	In-band blocking for LP-WUR

The UE shall fulfil the minimum requirements specified in Table7.6M.2-1 and Table 7.6M.2-2 for LP-WUR type 1 UE and LP-WUR type 2 UE respectively. The test parameters for the interferer are chosen as in Table 7.6M.2-3 for verification of the requirements specified in Table 7.6M.2-1 and Table 7.6M.2-2. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x.

Table 7.6M.2-1: In-band blocking parameters for LP-WUR type 1 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 18 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.6M.2-2: In-band blocking parameters for LP-WUR type 2 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 12 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.6M.2-3: In-band blocking for LP-WUR

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | -56 | -44 |
| Finterferer (offset) | MHz | – FIoffset, case 1 and + FIoffset, case 1 | – FIoffset, case 2 and + FIoffset, case 2 |

- 7.6M.3	Out-of-band blocking for LP-WUR

The out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band. The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x with parameters for the wanted signal as specified in Table 7.6M.3-1 and Table 7.6M.3-2 for LP-WUR type 1 and type 2 UE respectively, and for the interferer as specified in Table 7.6M.3-3.

Table 7.6M.3-1: Out-of-band blocking parameters LP-WUR type 1 UE

| LP-WUS bandwidth (RBs) | Power in LP-WUS bandwidth [dBm] |
| --- | --- |
| 11 | REFSENS + 18 dB |

Table 7.6M.3-2: Out-of-band blocking parameters LP-WUR type 2 UE

| LP-WUS bandwidth (RBs) | Power in LP-WUS bandwidth [dBm] |
| --- | --- |
| 11 | REFSENS + 12 dB |

Table 7.6M.3-3: Out of-band blocking for LP-WUR

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | -44 | -30 | -15 |
| Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

[For interferer frequencies across ranges 1, 2 and 3 in Table 7.6M.3-3, a maximum of

max24,6⋅n⋅NRB/6minn⋅NRB/10,5

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of min(BWchannel/2,5) MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration of NR signal, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2 for SCS = 15, 30 kHz, respectively. For these exceptions, the requirements in clause 7.7M apply.]

- 7.6M.4	Narrow band blocking for LP-WUR

This requirement is measure of a receiver's ability to receive a LP-WUS signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x with parameters specified in Table 7.6M.4-1 and Table 7.6M.4-2 for LP-WUR type 1 and type 2 UE respectively.

Table 7.6M.4-1: Narrow Band Blocking for LP-WUR type 1 UE

| Parameter | Unit | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + 34 |
| Puw (CW) | dBm | -55 |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 7.6M.4-2: Narrow Band Blocking for LP-WUR type 2 UE

| Parameter | Unit | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + 22 |
| Puw (CW) | dBm | -55 |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

- 7.7M	Spurious response for LP-WUR

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in clause 7.6M.3 is not met.

The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x with parameters for the wanted signal as specified in Table 7.7M-1 and Table 7.7M-2 for LP-WUR type 1 and type 2 UE respectively, and for the interferer as specified in Table 7.7M-3.

Table 7.7M-1: Spurious response parameters for LP-WUR type 1 UE

| RX parameter | Units | LP-WUS channel bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS channel bandwidth | dBm | PREFSENS + 18dB |

Table 7.7M-2: Spurious response parameters for LP-WUR type 2 UE

| RX parameter | Units | LP-WUS channel bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS channel bandwidth | dBm | PREFSENS + 12dB |

Table 7.7M-3: Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

- 7.8M	Intermodulation characteristics for LP-WUR

- 7.8M.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal

- 7.8M.2	Wide band Intermodulation for LP-WUR

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively.

The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x with parameters specified in Table 7.8M.2-1 and Table 7.8M.2-2 for LP-WUR type 1 and type 2 UE respectively.

Table 7.8M.2-1: Wide band intermodulation parameters for LP-WUR type 1 UE

| Rx parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS bandwidth, per CC | dBm | REFSENS + 24 dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -2BWChannel / +2BWChannel |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier with channel bandwidth ≥ 5 MHz. NOTE 2: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 3: The Finterferer 1 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

Table 7.8M.2-2: Wide band intermodulation parameters for LP-WUR type 2 UE

| Rx parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Pw in LP-WUS bandwidth, per CC | dBm | REFSENS + 12 dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -2BWChannel / +2BWChannel |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier with channel bandwidth ≥ 5 MHz. NOTE 2: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 3: The Finterferer 1 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

- 7.9M	Spurious emissions for LP-WUR

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9M-1

Table 7.9M-1: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30 MHz f < 1 GHz | 100 kHz | -57 dBm |  |
| 1 GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 2 |
| 12.75 GHz – 26 GHz | 1 MHz | -47 dBm | 3 |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. NOTE 2: Applies for Band that the upper frequency edge of the DL Band more than 2.69 GHz. NOTE 3: Applies for Band that the upper frequency edge of the DL Band more than 5.2 GHz. |  |  |  |