3GPP TSG-RAN WG4 Meeting #115                                           R4-2505521

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on BS RF for Rel-19 LP-WUS

Agenda Item:	7.28.4

Document for:	Approval

Introduction

BS RF requirements have been further discussed for LP-WUS and some aspects to be further analysed are captured in WF [1].

This contribution provides further analysis for the issues identified in the WF.

Discussion

Manufacturer declaration

BS manufacturer declaration for LP-WUS has been further discussed in last meeting. The WF is listed as below:

Issue 1-1: How to specify the BS manufacturer declaration for LP-WUS

- Manufacturer shall declare the support of LP-WUS for each supported band, [frequency range] and channel bandwidth.

- If the BS supports LP-WUS power boosting, manufacturer shall declare the LP-WUS power boosting level it could support with a minimum of +[X]dB.

  - X can be different for different channel bandwidth.

  - FFS on the value of X and use 3 as starting point.

Given RAN4’s agreement on adopting NB-IoT-like power boosting levels, we can use per-RE EPRE boosting as a reference to determine the channel bandwidth (CBW) threshold beyond which the power boosting sensitivity becomes negligible. As defined, this refers to the case where power boosting variations across different CBWs become insignificant. The values presented in the tables below indicate that this threshold occurs at CBW≥30MHz, where the boosting levels show minimal variation.

Table 1: LP-WUS power boosting for different CBWs (fixed EPRE as reference)

| Reference EPRE ratio (dB) | LP-WUS power boosting (fixed EPRE boosting as reference) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CBW 10M (52RB) | CBW 20M (106RB) | CBW 30M (160RB) | CBW 40M (216RB) | CBW 50M (270RB) |
| 3 | 1.3 | 2.1 | 2.4 | 2.5 | 2.6 |
| 4 | 1.6 | 2.7 | 3.1 | 3.3 | 3.4 |
| 5 | 1.9 | 3.2 | 3.7 | 4.0 | 4.2 |

Table 2: LP-WUS power boosting for different CBWs (fixed LP-WUS boosting)

| LP-WUS Boosting (dB) | EPRE boosting (fixed LP-WUS power boosting as reference) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CBW 10M (52RB) | CBW 20M (106RB) | CBW 30M (160RB) | CBW 40M (216RB) | CBW 50M (270RB) |
| 2 | 5.4 | 2.9 | 2.5 | 2.3 | 2.3 |
| 3 | 14.1 | 4.6 | 3.9 | 3.6 | 3.5 |

Observation 1: The LP-WUS power boosting level exhibits reduced sensitivity to channel bandwidth variations when CBW ≥30MHz.

Proposal 1: It is proposed to adopt minimum 3dB power boosting for CBW≥30MHz. For CBW less than 30MHz, additional values could be declared separately.

Based on the approved Way Forward, and similar to the NB-IoT approach, the following power boosting declarations could be considered for LP-WUS:

Table 3: Declarations for LP-WUS

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.x | LP-WUS operation | Manufacturer shall declare the support of LP-WUS operation in NR in-band for each supported band, frequency range and channel bandwidth. | x |  |
| D.y | LP-WUS power dynamic range | If the BS supports in-band LP-WUS operation with NR, the manufacturer shall declare the supported maximum power dynamic range with a minimum of +3 dB power boosting for channel bandwidths >= [30MHz], while additional boosting levels can be declared separately for narrower bandwidths (<30 MHz). | x |  |

While defining a minimum power boost level is not required as a core requirement, at least one boost level should be included in the declaration.

Proposal 2: It is proposed to adopt power boost declarations similar to those specified in Table 3. A single minimum value should be declared for certain larger CBWs, which is not sensitive to CBW according to the boosting definition. Different boosting levels could still be declared for smaller CBWs.

Other requirements other than dynamic range

Besides the dynamic range requirement, the main discussion in RAN4 is focused on LP-WUS EVM. In last meeting, we have the following WF:

Issue 1-3: Whether to define EVM requirements for LP-WUS

- Option 1: No need to define EVM requirements for LP-WUS

- Option 2: EVM requirements for LP-WUS is needed

Several companies have conducted simulations on the PAPR for the combined signal of LP-WUS and NR. The results indicate that LP-WUS has minimal impact on the PAPR of the composite signal. Consequently, these companies propose omitting the EVM requirement specification for LP-WUS. Alternatively, the influence of LP-WUS on NR signal quality could be assessed as an evaluation metric.

Table 1 presents comparative simulation results analyzing both PAPR and EVM characteristics for a pure NR signal versus an NR+LP-WUS composite signal, using 256QAM modulation for the NR component. The evaluation includes both average EVM and per-symbol peak EVM measurements across multiple channel bandwidths. While the PAPR values remain comparable between the two signal configurations, the results demonstrate measurable EVM degradation in specific 256QAM cases when LP-WUS is introduced. This suggests a non-negligible impact on NR signal integrity at higher modulation orders. Notably, our assessment reveals this effect to be significantly less pronounced for 64QAM modulation scenarios.

Table 4: PAPR & EVM comparison between pure NR CP-OFDM vs NR+LP-WUS

| Channel Bandwidth | Signal type | PAPR 0.01% | EVMavg (%) | EVMmax (%) |
| --- | --- | --- | --- | --- |
| 10MHz | NR only | 9.725 | 3.14 | 4.51 |
|  | NR+LP-WUS | 9.564 | 3.23 | 4.45 |
| 20MHz | NR only | 9.767 | 2.91 | 4.67 |
|  | NR+LP-WUS | 9.788 | 3.01 | 4.95 |
| 50MHz | NR only | 9.754 | 2.82 | 3.40 |
|  | NR+LP-WUS | 9.972 | 3.22 | 4.52 |

Observation 2: LP-WUS could introduce noticeable EVM degradation in certain cases for 256QAM.

Proposal 3: No need to define EVM requirements for LP-WUS.

Proposal 4: It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance.

During discussion in last meeting, companies agree to make a decision in this meeting for LP-WUS EVM. If there could be a conclusion that no EVM requirement needs to be defined for LP-WUS, the other issue is whether to verify the performance impact to NR due to LP-WUS. Several options were discussed in last meeting:

- Option 1: There is no need to do the EVM verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

- Option 2: Without specific EVM requirement defined for LP-WUS, just use a higher-order EVM test for NR with embedded LP-WUS as the verification metric.

- Option 3: The OOK4 signal quality test can be dependent on the 64QAM EVM test.

The WF in last meeting for issue “ Whether/how to verify EVM requirements for LP-WUS” is:

- Consider together with EVM requirement for LP-WUS

  - If no EVM requirement is defined, no verification is needed

  - If EVM requirement is defined, FFS on whether verification is needed.

Based on the above analysis, limiting the NR signal to 64QAM modulation makes the EVM degradation caused by LP-WUS negligible. We therefore recommend waiving specific NR performance verification requirements when LP-WUS is present. Following the same approach used for NB-IoT in-band operation, system performance guarantees can be adequately addressed through base station implementation.

Proposal 5: No alternative testing methods shall be required to indirectly verify LP-WUS signal quality when operating with NR signals at 64QAM maximum modulation order.

Conclusion

This contribution provides further consideration for BS RF of LP-WUS. With above analysis, we have the following observations and proposals:

Observation 1: The LP-WUS power boosting level exhibits reduced sensitivity to channel bandwidth variations when CBW ≥30MHz.

Proposal 1: It is proposed to adopt minimum 3dB power boosting for CBW≥30MHz. For CBW less than 30MHz, additional values could be declared separately.

Proposal 2: It is proposed to adopt power boost declarations similar to those specified in Table 3. A single minimum value should be declared for certain larger CBWs, which is not sensitive to CBW according to the boosting definition. Different boosting levels could still be declared for smaller CBWs.

Table 3: Declarations for LP-WUS

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.x | LP-WUS operation | Manufacturer shall declare the support of LP-WUS operation in NR in-band for each supported band, frequency range and channel bandwidth. | x |  |
| D.y | LP-WUS power dynamic range | If the BS supports in-band LP-WUS operation with NR, the manufacturer shall declare the supported maximum power dynamic range with a minimum of +3 dB power boosting for channel bandwidths >= [30MHz], while additional boosting levels can be declared separately for narrower bandwidths (<30 MHz). | x |  |

Observation 2: LP-WUS could introduce noticeable EVM degradation in certain cases for 256QAM.

Proposal 3: No need to define EVM requirements for LP-WUS.

Proposal 4: It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance.

Proposal 5: No alternative testing methods shall be required to indirectly verify LP-WUS signal quality when operating with NR signals at 64QAM maximum modulation order.

- References

[1] R4-2504744, WF on NR LP-WUS BS RF, Huawei, HiSilicon