3GPP TSG-RAN WG4 Meeting # 115	R4-2505528

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on UL CA_n28A-n71A

Agenda Item:	6.3.3

Document for:	Approval

Introduction

In the RAN4#114bis meeting, one TP [1] was provided to introduce UL CA_n28A-n71A configuration. As RAN4 didn’t reach consensus on whether to introduce UL CA_n28A-n71A configuration and a lot of issues were raised to be further addressed. Finally, one WF [2] was approved to discuss this topic in this meeting.

In this paper, we’d like to share our views on the potential RF architectures and study the impacts of RF requirements.

Discussion

Based on the WF [2], it’s encouraged for companies to investigate the different RF architectures in order to enable the UL CA_n28-n71. We proposed to consider Three-antenna RF architecture (NOTE: it isn’t precluded to implement Full band duplexer for band n28.) and two-antenna RF architecture as shown below.

Figure 1 Three-antenna RF architecture for UL CA_n28A-n71A

figure 2 Two-antenna RF architecture for UL CA_n28A-n71A

It’s identified [1] that PA Reverse IMD will dominate the MSD value and may have some impacts on the spurious emission requirements.

Thus, we investigate the reverse IMD3 issue for the two potential RF architectures as shown below

figure 3 The budget analysis for RIMD based on the three-antenna RF architecture

figure 4 The budget analysis for RIMD based on the two-antenna RF architecture

Based on the investigations above, no matter whether RAN4 assume three-antenna RF architecture or two-antenna RF architecture, the RIMD interference level at antenna connector is about -50 ~ -53dBm/UL BW.

The attenuation between UL band n71 and n28 is assumed as 3dB at least, since the pass band of Tx filter is specified based on the 3dB cut-off point. In addition, the frequency of RIMD is located in the DL band of n71 or n28, so the Tx filter will suppress the interference of RIMD. The attenuation of Tx filter at DL frequency band is assumed as 42~45dB.

Thus, if the RIMD interference level at antenna connector is about -50 ~ -53dBm/UL BW, the RIMD interference level will not affect the UE compliance of Tx RF requirements.

Observation 1: no matter whether RAN4 assume three-antenna RF architecture or two-antenna RF architecture, the RIMD interference level at antenna connector is about -50 ~ -53dBm/UL BW based on the following reasons.

1) Both band n71 and n28 Tx chain can’t be combined together under one antenna

2) Tx filter will suppress the interference of RIMD.

Observation 2: if the RIMD interference level at antenna connector is about -50 ~ -53dBm/UL BW, the RIMD interference level will not affect the UE compliance of Tx RF requirements.

Three antenna RF architecture is assumed for CA_n28-n71.

Figure 5.x.2.3-1 Three-antenna RF architecture for CA_n28-n71

Since the Tx filter in one band can not provide the sufficient rejection of another Tx band, the PA reverse mixing interference dominate the MSD requirements. For example, the following table represents the different interference component for DL n71 IMD3 MSD when both band n28 and n71 transmit signals.

| No. | Component | IMD3(dBm/5MHz) |  |
| --- | --- | --- | --- |
|  |  | Main_@Antenna | Div_@Antenna |
| 1) | n28 PA Forward mixing interference | -143.6 |  |
| 2) | n71 PA Forward mixing interference | -79.55 |  |
| 3) | n28 PA Reserve mixing interference | -74.6 |  |
| 4) | n71 PA Reserve mixing interference | -47.6 |  |
|  | n71 Duplexer | -93.5 |  |
| 5) | n71 LNA via filters | -57.5 | -88.5 |
|  | n71 LNA via PCB coupling | -92 | -92 |
|  | Diplexer (L-H) | -121.5 | -142 |
|  | Antenna switch | -84 | -108.5 |
|  | From main path |  | -57.12 |
|  | Total | -47.12 | -57.12 |

The MSD value due to IMD3 can be specified below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n71 | n28 | 728 | 5 | 25 | 773 | 38 | FDD | IMD34 |
|  | n71 | 683 | 5 | 25 | 637 | N/A | FDD | N/A |
|  | n28 | 729 | 5 | 25 | 774 | N/A | FDD | N/A |
|  | n71 | 683 | 5 | 25 | 637 | 38 | FDD | IMD34 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Proposal 1: It’s proposed to specify the MSD due to IMD3 below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n71 | n28 | 728 | 5 | 25 | 773 | 38 | FDD | IMD34 |
|  | n71 | 683 | 5 | 25 | 637 | N/A | FDD | N/A |
|  | n28 | 729 | 5 | 25 | 774 | N/A | FDD | N/A |
|  | n71 | 683 | 5 | 25 | 637 | 38 | FDD | IMD34 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

- Summary

Observation 1: no matter whether RAN4 assume three-antenna RF architecture or two-antenna RF architecture, the RIMD interference level at antenna connector is about -50 ~ -53dBm/UL BW based on the following reasons.

1) Both band n71 and n28 Tx chain can’t be combined together under one antenna

2) Tx filter will suppress the interference of RIMD.

Observation 2: if the RIMD interference level at antenna connector is about -50 ~ -53dBm/UL BW, the RIMD interference level will not affect the UE compliance of Tx RF requirements.

Proposal 1: It’s proposed to specify the MSD due to IMD3 below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n71 | n28 | 728 | 5 | 25 | 773 | 38 | FDD | IMD34 |
|  | n71 | 683 | 5 | 25 | 637 | N/A | FDD | N/A |
|  | n28 | 729 | 5 | 25 | 774 | N/A | FDD | N/A |
|  | n71 | 683 | 5 | 25 | 637 | 38 | FDD | IMD34 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

- References

[1] R4-2503687	TP for TR 38.719-02-01 on introduction of CA_n28A-n71A, Huawei, HiSilicon

[2] R4-2505216	WF for uplink CA n28-n71, Huawei