# 3GPP TSG-RAN WG4 Meeting # 115	R4-2505531

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the simultaneous operation between GNSS and NR NTN

Agenda Item:	7.31.3.1

Document for:	Approval

Introduction

In the RAN4#114bis meeting, we received the replied LS [1] from RAN2.

RAN2 would like to thank RAN4 for the original LS in R4-2419898. RAN2 has considered the simultaneous operation between GNSS and UL transmission in NR NTN and concluded not to work on the enhancements to the existing IDC solutions in Release 18. RAN2 might consider the extension to IDC TDM solution or autonomous denial solution in Release 19, and would like to receive more information on requirements, e.g. on the periodicity and duration of IDC free time to help the UE read the GNSS in NTN.

In the last meeting, different companies provided different proposals for IDC free duration and periodicity [2].

- Proposals on IDC free duration - Option 1: values for GNSS-PositionFixDuration in TS 36.331: 1, 2, 3, 4, 5, 6, 7, 13, 19, 25, 31 seconds. (Vivo) - Option 1a: Leave out some large values as cold start for GNSS is not in scope (Huawei, Sony) - Option 2: 1s to 10s, with max 50% duty Tx duty cycle (Qualcomm) - Option 3: Extend IDC-TDM activeDuration-r18 to include 2000ms (Nokia) - Proposals on IDC periodicity - Option 1: few seconds to few tens of seconds (Qualcomm) - Option 2: Extend IDC-TMD cycleLength-r18 to 40960 ms if no RRM-impact (Nokia) - Other proposals - No IDC enhancement is needed (Ericsson) Moderator: RAN4 specifically asked for IDC improvements in the LS in R4-2419898.

In this paper, we’d like to share our views on these proposals.

Discussion

Based on the discussion in last meeting, RAN4 has concluded that some large values are left out as cold start for GNSS is not in scope. Based on the topic summary [2], the recommended IDC free duration can range from 1s to [13]s.

If RAN4 only consider the hot start, the TTFF can be 1 or 2 second(s). Current IDC active duration is specified below.

IDC-TDM-Assistance-r18 ::=            SEQUENCE {

cycleLength-r18                       ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30,

ms32, ms35, ms40, ms60, ms64, ms70, ms80, ms96, ms100, ms128, ms160,

ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240},

startOffset-r18                       INTEGER (0..10239),

slotOffset-r18                        INTEGER (0..31),

activeDuration-r18                    CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

...

}

Current maximum value for activeDuration is 1200ms. If RAN4 only consider hot start, it seems current maximum value 1200ms for activeDuration in the IDC-TDM framework is enough or we can add one more value 2000ms.

Observation 1: If RAN4 only consider hot start, it seems current maximum value 1200ms for activeDuration in the IDC-TDM framework is enough or we can add one more value 2000ms.

During the discussion, companies suggest that it’s better to go idle mode, if UE needs more 2 seconds for activeDuration in the IDC-TDM framework. From this perspective, there seems no need to introduce the values more than 2 seconds for activeDuration in the IDC-TDM framework.

Proposal 1: RAN4 can discuss whether to introduce the values more than 2 seconds for activeDuration based on current R18 IDC-TDM framework.

Summary

Observation 1: If RAN4 only consider hot start, it seems current maximum value 1200ms for activeDuration in the IDC-TDM framework is enough or we can add one more value 2000ms.

Proposal 1: RAN4 can discuss whether to introduce the values more than 2 seconds for activeDuration based on current R18 IDC-TDM framework.

- References

[1] R2-2501568, Response LS on simultaneous operation between GNSS and NR NTN, RAN2

[2] R4-2504634, Topic summary for [114bis][309] NR_NTN_Ph3_General_UE_SAN_RF, Moderator (Qualcomm)