# 3GPP TSG-RAN WG4 Meeting # 115	R4-2505534

# St Julian’s, Malta, 19th – 25th May 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the Rel-19 PC2 FDD RedCap UE

Agenda Item:	7.6.2

Document for:	Approval

Introduction

In the RAN106 meeting, the WI [1] for NR PC2 RedCap UE was revised to include the FDD PC2 feature. There are two aspects highlighted here. Firstly, RAN4 may need to discuss the band agnostic way for FDD PC2 RedCap UE. Secondly, it’s noted that wearable device doesn’t support HPUE in FDD bands.

- For FDD bands, specify RF requirements for PC2 RedCap UE - Band specific requirements are specified for the example bands including n3 and n1 when band agnostic way is not applicable (Example bands are selected by considering different sensitivity degradation causes) - Note: Wearable device supporting HPUE is not in the scope for FDD bands in this work item

In the RAN4#114 meeting, based on the WF [2], RAN4 reached some consensus on MPR/AMPR/REFSENS for HD-FDD/REFSENS for 2Rx FD-FDD requirements for PC2 RedCap UE in FDD bands. Only one remaining issue, i.e. 1Rx REFSENS in FD-FDD, need to be further discussed. For the remaining issue, RAN4 made some progress to consider [REFSENS (1Tx (26dBm) /2Rx) + ΔR1R], but need to evaluate how to specify the ΔR1R based on the following two options [3].

Issue 1-1-1: REFSENS for 1Rx in FD-FDD Agreement: Consider the following formats for specifying REFSENS for 1Rx PC2 RedCap in FD-FDD - Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on RSD ranges - Flat 3dB ΔR 1R for all CBW no larger than 20MHz or use values for FDD bands in table 7.3I.2-1 for bands whose PC2 1Tx/2RX RSD is 0dB. - Companies to evaluate REFSENS for other bands. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. - Option2: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on the following conditions - ΔR 1R value may be considered from LTE TS 36.101 Table 7.3.1E-1A for bands and CBW which are already defined in TS 36.101 Table 7.3.1E-1A. The ΔR 1R value is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1. - Companies are encouraged to bring measurement based evaluation for ΔR1R for other bands and CBW which are not covered by LTE TS 36.101 Table 7.3.1E-1A. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. Way forward: It is encouraged for companies to provide analysis for option 1 and 2. Note: Companies are encouraged to evaluate all specified PC2 FDD bands. The WI is intended to be completed in May meeting according to WI plan.

In this paper, we’d like to discuss this remaining issue for FDD PC2 RedCap UE.

Discussion

In the last meeting, RAN4 agreed to consider the following format based on the REFSENS (1Tx (26dBm) /2Rx).

REFSENS (1Tx (26dBm) /2Rx) + ΔR1R

The remaining issue is how to specify the ΔR1R, considering the PC2 RSD range and the other conditions (e.g. LTE category 0 UE REFSENS).

For option 1 in the WF [3] and companies’ evaluation in the discussion summary [4], we can summarize this option as below considering the RSD in Table 7.3.2-1c of TS 38.101-1.

Single antenna port reference sensitivity allowance ΔR1R for FDD PC2 RedCap

| ΔR1R for FDD PC2 RedCap |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | Discussion summary [4] |
| n1 | 2.5 | 3 | 3 | 3 | Proposal 1 and 3 |
| n1 | 3 | 3 | 3 | 3 | Proposal 2 and 4 |
| n3 | 3 | 3 | 3 | 3 | Proposal 2 and 4 |
| n3 | 2.5 | 3 | 3 | 3 | Proposal 3 |
| n3 | 2.7 | 3.3 | 3.3 | 3.3 | Proposal 1 |

Observation 1: the difference ΔR1R for FDD PC2 RedCap in band n1 is whether to consider 2.5dB or 3dB ΔR1R in 5MHz.

Observation 2: the difference ΔR1R for FDD PC2 RedCap in band n3 is equal to 0.3dB for 5, 10, 15, 20MHz.

For option 2, we compared the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1. The REFSENS difference is shown below.

| REFSENS Difference |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
|  | (dBm) | (dBm) | (dBm) | (dBm) |
| 2 | 2.5 | 2.5 | 2.5 | 2.5 |
| 3 | 2.5 | 2.5 | 2.5 | 2.5 |
| 4 | 2.5 | 2.5 | 2.5 | 2.5 |
| 5 | 2.5 | 2.5 |  |  |
| 8 | 2.5 | 2.5 |  |  |
| 13 | 3 | 3 |  |  |
| 20 | 2.5 | 2.5 | 3 | 3 |
| 25 | 2.5 | 2.5 | 2.5 | 2.5 |
| 26 | 2.5 | 2.5 | 2.5 |  |
| 28 | 2.5 | 2.5 | 2.5 | 2.5 |

However, the REFSENS for LTE UE category 0 is defined for PC3, not sure whether RAN4 could consider option 2 ΔR1R = 2.5dB for all the channel bandwidths for PC2 FDD RedCap.

As a compromise, it’s recommended to consider ΔR1R = 3dB for all the channel bandwidths for PC2 FDD RedCap by using a band agnostic method at least for band n1 and n3.

Proposal 1: For a PC2 RedCap UE with 1Rx, the REFSENS requirement is specified by the formula:

REFSENSPC2 RedCap 1Rx=REFSENSPC2 2Rx/1Tx + ΔR1R

The ΔR1R could be 3dB for all the channel bandwidths by using a band agnostic method.

Summary

Observation 1: the difference ΔR1R for FDD PC2 RedCap in band n1 is whether to consider 2.5dB or 3dB ΔR1R in 5MHz.

Observation 2: the difference ΔR1R for FDD PC2 RedCap in band n3 is equal to 0.3dB for 10, 15, 20MHz.

Proposal 1: For a PC2 RedCap UE with 1Rx, the REFSENS requirement is specified by the formula:

REFSENSPC2 RedCap 1Rx=REFSENSPC2 2Rx/1Tx + ΔR1R

The ΔR1R could be 3dB for all the channel bandwidths by using a band agnostic method.

- References

[1] RP-243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom

[2] R4-2502874, WF on NR PC2 RedCap UE in FDD bands, China Telecom, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple

[3] R4-2505231, WF on PC2 RedCap UE RF requirements, China Telecom

[4] R4-2504678, Topic summary for [114bis][120] NR_PC2_RedCap_UE, Moderator (China Telecom)