# 3GPP TSG-RAN WG4 Meeting # 115	R4-2505535

St Julian’s, Malta, 19th – 25th May 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on VSAT requirements for NR NTN Ku band

Agenda Item:	7.11.5

Document for:	Approval

Introduction

In the RAN4#114bis meeting, we have the following way forward [1] for Ku band VSAT.

- Issue 1-2-1: Minimum EIRP requirements - Consider the following as the starting point Operating band; UE Type; Min peak EIRP (dBm). [When communicating with LEO]; [When communicating with GSO]. n248, n247, n509, n508; 4; [64]; [70]. 5; [64]; [70]. - FFS on whether further clarification is needed - Issue 1-3-1: EIS requirement Agreement: - FFS on the following two options - Option 1: - Defining EISREFSENS_10MHz for FR1-NTN Ku bands, and define EISREFSENS_50MHz for FR2-NTN Ku bands - Option 2: - Consider define the common EIS requirement for both FR1-NTN Ku bands and FR2-NTN Ku bands - FSS on the value of the EISREFSENS_10MHz and EISREFSENS_50MHz - The requirement for other channel bandwidth will be scaled based on PRB based

In addition, RAN4 never discuss the minimum ACS/in-band blocking requirements for Ku band VSAT. In this paper, we’d like to share our views on these RF requirements for Ku band VSAT.

Discussion

1) Minimum EIRP requirements:

Referring to the contribution [2], the minimum EIRP for Ku band VSAT could be derived as following:

EIRPmin=-174dBm/Hz+10log10BW+NF+SNR+IM-AntennaGainRx_SAN+FSPL+Atmosphereloss

Where:

Noise Figure is assumed as 3dB.

SNR: -1dB

IM: 2dB

AntennaGainRx_SAN: 58.5dB for 90 deg elevation and 55.5dB

FSPL: 32.44 + 20log(fc_MHz) + 20log(Distance_km)

Atmosphereloss: 2dB

Currently, in this release RAN4 only agreed to specify the following VSAT types which support both GSO and LEO scenarios.

|  | GEO |  | GEO |  |
| --- | --- | --- | --- | --- |
| Elevation angle (Deg) | 30.00 | 90.00 | 30.00 | 90.00 |
| Distance (VSAT-SAN) | 38611.64 | 35786.00 | 38611.64 | 35786.00 |
| UL frequency | 14500.00 | 14500.00 | 14500.00 | 14500.00 |
| FSPL | 207.40 | 206.74 | 207.40 | 206.74 |
| Atmosphere loss (dB) | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN NF (dB) | 3.00 | 3.00 | 3.00 | 3.00 |
| UL BW (MHz) | 100.00 | 100.00 | 200.00 | 200.00 |
| Target SNR (dB) | -1.00 | -1.00 | -1.00 | -1.00 |
| Implementation Margin (dB) | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN Rx Gain (dBi) | 55.50 | 58.50 | 55.50 | 58.50 |
| VSAT Minimum EIRP (dBm) | 63.90 | 60.24 | 66.91 | 63.25 |

Observation 1: based on our evaluations, when VSAT is communicating with GSO SAN, it’s enough for VSAT to specify 70dBm minimum EIRP requirements.

As the VSAT is capable of communicating with GSO SAN, that means the VSAT in Ku band is capable of transmitting 70dBm. Thus, RAN4 can discuss whether it’s enough to only specify minimum EIRP requirement 70dBm for VSAT communicating with both GSO and LEO.

Proposal 1: RAN4 can discuss whether it’s enough to only specify minimum EIRP requirement 70dBm for VSAT communicating with both GSO and LEO.

2) Minimum EIS requirements:

From link budget perspective, the minimum EIS can be derived below.

EISmin= SAN_EIRPdensity + 10log(BW) – FSPL – Atmosphereloss

|  | GEO |  | GEO |  |
| --- | --- | --- | --- | --- |
| Elevation angle (Deg) | 30.00 | 90.00 | 30.00 | 90.00 |
| Distance (VSAT-SAN) | 38611.64 | 35786.00 | 38611.64 | 35786.00 |
| DL frequency | 12700.00 | 12700.00 | 12700.00 | 12700.00 |
| FSPL | 206.25 | 205.59 | 206.25 | 205.59 |
| Atmosphere loss (dB) | 2.00 | 2.00 | 2.00 | 2.00 |
| EIRP density (dBm/MHz) | 70 | 73 | 70 | 73 |
| DL BW (MHz) | 10 | 10 | 50 | 50 |
| VSAT Minimum EIS (dBm) | -128.25 | -124.59 | -121.26 | -117.60 |

From VSAT implementation perspective, the minimum EIS can be derived below.

EISmin = 10*log10(k)+10*log10(BW)+60+IM+Tar_SNR - G/T

Where:

k is Boltzmann constant, 1.38*10-23 J/K

BW is assumed as 50MHz.

IM is implementation margin 2dB

Tar_SNR is -1dB

If G/T for VSAT is larger than 1dB, then the minimum EIS is larger than -121.6dBm for DL 50MHz BW.

Thus, EISREFSENS_50MHz for FR2-NTN Ku bands is -121.26dBm and EISREFSENS_10MHz for FR1-NTN Ku bands is -128.25dBm. We can have the following calculation with different assumed value of G/T.

| G/T (dB) | EISREFSENS_10MHz (dBm) | EISREFSENS_50MHz (dBm) |
| --- | --- | --- |
| 1 | -121.26 | -128.25 |
| 2 | -122.26 | -129.25 |
| 3 | -123.26 | -130.25 |
| 4 | -124.26 | -131.25 |
| 5 | -125.26 | -132.25 |
| 6 | -126.26 | -133.25 |
| 7 | -127.26 | -134.25 |
| 8 | -128.26 | -135.25 |
| 9 | -129.26 | -136.25 |

Proposal 2: RAN4 can consider to specify minimum EIS requirements EISREFSENS_10MHz and EISREFSENS_50MHz for 10MHz BW and 50MHz BW based on the assumed value of G/T with range 1~9dB.

| G/T (dB) | EISREFSENS_10MHz (dBm) | EISREFSENS_50MHz (dBm) |
| --- | --- | --- |
| 1 | -121.26 | -128.25 |
| 2 | -122.26 | -129.25 |
| 3 | -123.26 | -130.25 |
| 4 | -124.26 | -131.25 |
| 5 | -125.26 | -132.25 |
| 6 | -126.26 | -133.25 |
| 7 | -127.26 | -134.25 |
| 8 | -128.26 | -135.25 |
| 9 | -129.26 | -136.25 |

3) Minimum ACS/in-band blocking requirements:

As these requirements are depending on the outcome of coexistence study, we’d like to discuss the framework of ACS requirements.

Firstly, it’s proposed to consider a separate table for FR1-NTN Ku bands and group FR2-NTN Ku bands together with FR2-NTN Ka bands.

Table 10.5.1-1: Adjacent channel selectivity Operating band; Units; Adjacent channel selectivity / Channel bandwidth. 50 MHz; 100 MHz; 200 MHz; 400 MHz. n512, n511, n510; dB; 25; 25; 25; 25. n508, n509; dB; TBD; TBD; TBD; TBD. Table 10.5.1-1a: Adjacent channel selectivity Operating band; Units; Adjacent channel selectivity / Channel bandwidth. 10MHz, 15MHz, 20MHz, 25MHz, 35MHz, 50MHz, 70MHz, 100MHz. n247, n248; dB; TBD.

Generally, the wanted signal value is { REFSENS + Degradation Value }.

And the ACS blocking interference value is { UE noise floor + ACS +10log10(10^( Degradation Value /10)-1) } = { REFSENS - 1 + ACS + 10log10(10^( Degradation Value /10)-1) }

NOTE: UE noise floor + SNR(-1dB) + IM(2dB) = REFSENS => UE noise floor = REFSENS – 1.

Observation 2: to consider a separate table for FR1-NTN Ku bands and group FR2-NTN Ku bands together with FR2-NTN Ka bands when specifying the ACS/in-band blocking requirements.

Summary

Observation 1: based on our evaluations, when VSAT is communicating with GSO SAN, it’s enough for VSAT to specify 70dBm minimum EIRP requirements.

Proposal 1: RAN4 can discuss whether it’s enough to only specify minimum EIRP requirement 70dBm for VSAT communicating with both GSO and LEO.

Proposal 2: RAN4 can consider to specify minimum EIS requirements EISREFSENS_10MHz and EISREFSENS_50MHz for 10MHz BW and 50MHz BW based on the assumed value of G/T with range 1~9dB.

| G/T (dB) | EISREFSENS_10MHz (dBm) | EISREFSENS_50MHz (dBm) |
| --- | --- | --- |
| 1 | -121.26 | -128.25 |
| 2 | -122.26 | -129.25 |
| 3 | -123.26 | -130.25 |
| 4 | -124.26 | -131.25 |
| 5 | -125.26 | -132.25 |
| 6 | -126.26 | -133.25 |
| 7 | -127.26 | -134.25 |
| 8 | -128.26 | -135.25 |
| 9 | -129.26 | -136.25 |

Observation 2: to consider a separate table for FR1-NTN Ku bands and group FR2-NTN Ku bands together with FR2-NTN Ka bands when specifying the ACS/in-band blocking requirements.

- References

[1] R4-2504720, WF for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

[2] R4-2402521, Further discussion on UE RF Tx requirements for NTN in Ka-band, ZTE Corporation