3GPP TSG-RAN4 Meeting #115 	R4-2505542

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title:  	Ambient IoT BS RF requirements

Agenda Item:	7.26.3.1

Document for:	Approval

- Introduction

The WI on solutions for Ambient IoT in NR was approved at RAN#106 [1] and revised at RAN#107[2]. Last meeting, a WF was agreed [3]. This contribution provides our analysis for the remaining RF requirements for Ambient-IoT BS.

- Ambient IoT BS TX requirements

  - Base station output power

38 dBm A-IoT BS output power with ACLR 20/30/40/50dB for Scenario 2-1c in TR38.769 are simulated and results are showed in in our previous paper [4]. Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. The maximum 38dBm output power enables flexible deployment, allowing power levels to be adjusted based on scenario needs.

Proposal 1: Reuse the output power of Medium Range BS for indoor A-IoT BS, Prated,c,AC≤ 38dBm

  - Transmit ON/OFF power

2.3.1 Transmitter OFF power

The NR BS type 1-C NR requirements for transmitter OFF power can be reused.

Proposal 2: For A-IoT BS, the requirements for transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

2.3.2 Modulation quality

Consider effect to device modulation performance, it is need to define modulation depth, RF envelope ripple, RF envelop rise/fall time and RF Pulse width as in the standard of EPC RFID [5].

Figure 3 shows a simulation baseband waveform of R2D with 1 PRB and 4 PRB.The red is CP and the blue is R2D data.

（a）						(b)

Figure 1 Simulation time domain baseband R2D waveform (a) 1PRB; (b) 4PRB

Based on our baseband signal simulation, it can be calculated the following values in Table 4.

Table 1: Simulation results of A-IoT BS RF envelope parameters

| Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- |
| Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
| RF Envelope Ripple | Mh = Ml | 0 |  | 0.10(A–B) | V/m orA/m |
| RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
| RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
| RF Pulsewidth | PW |  | 0.5Tari |  | µs |

Additional considerations such as CP effects, varying chip lengths, digital filtering, and RF impairments may need to be included. The R2D signal uses OOK-4 modulation, appearing as a near-square wave in the time domain. This results in spectral spreading, which can be suppressed with a digital filter—though doing so degrades time-domain signal quality.

Proposal 3: Consider RF impairments, the RF envelope requirements can be defined as Table 2.

Table 2: A-IoT BS RF envelope parameters

| Tari | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| 1/(M*15KHz)*10^6 Where M could be 1, 2, 4, 6, 8, 12, 16, 24 | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Mh = Ml | 0 |  | 0.15(A–B) | V/m orA/m |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
|  | RF Pulsewidth | PW |  | 0.5Tari | 0.65Tari | µs |

  - Unwanted emissions

    - Occupied bandwidth

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also Recommendation ITU-R SM.328.

The value of /2 shall be taken as 0.5%.

The occupied bandwidth for each A-IoT carrier shall be less than the A-IoT BS channel bandwidth.

Proposal 4: The occupied bandwidth for each A-IoT carrier shall be less than the A-IoT BS channel bandwidth.

2.5.2	Adjacent Channel Leakage Power Ratio

Copied from R4-2505097

### Issue 1-5-2: ACLR

- Agreement:

  - Consider following options for ACLR requirements

    - Option 1: ACLR1 is 40dB and ACLR2 is 50dB for A-IOT BS 1PRB transmission BW with 100kHz offset

    - Option 2: ACLR 1 is 30dB and ACLR2 is 40dB for A-IoT BS 1PRB transmission BW with Foffset=100kHz.

  - FFS on whether frequency offset is needed or not for other PRB numbers

For co-existence Scenario 2-1c (NR UE only outdoor) in TR38.769, simulation results show that even with A-IoT BS ACLR of 20dB, there is negligible degradation in NR DL throughput, both for average and cell-edge throughput.

The spectrum of baseband R2D signals before and after FIR filters are simulated, and results are shown in the Figures 1, comparing the unfiltered (blue) and filtered (orange).

(a)                                    (b)

Figure 2: PSD of A-IoT R2D before and after filter,(a) 1 PRB transmission BW;(b) 4PRB transmission BW

Figure 3 shows two R2D channel bandwidth configurations:
(a) 200 kHz channel BW with 1 PRB transmission BW
(b) 800 kHz channel BW with 4 PRB transmission BW

These simulations exclude PA impairments. Simulation results are shown in the Table 1:

Table 3 ACLR simulation result for 1PRB and 4PRB transmission BW （without PA）

| R2D CBW |  | 200kHz | 800kHz |
| --- | --- | --- | --- |
| ACLR1 （without frequency offset） | Before filter | 22dB | 28dB |
|  | After filter | 31dB | 50dB |
| ACLR1 （with 100kHz frequency offset） | Before filter | 35dB | 36dB |
|  | After filter | 90dB | 60dB |

A-IoT BS ALCR relaxation needed because the following reasons:

- Co-existence simulations show negligible degradation even at 20dB ACLR

- Baseband R2D signal (unfiltered) ACLR 22/31dB；

- Stricter ACLR will degrade time-domain signal quality

Proposal 5: Specify 200-800kHz R2D CBW A-IoT carrier ACLR as the following table.

Table 4: A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | Offset(kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 200/400/600/800 | 100 | BWChannel | A-IoT of same BW | Square (BWConfig) | 30 dB |
|  | 100 | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |

Figure 3 Diagram of A-IoT ACLR with 100kHz Foffset

2.5.4	Transmitter intermodulation

Since A-IoT BS and NR BS operating in the same band are not co-located based on R19 A-IoT WI scope, there’s no need to define transmitter intermodulation requirements.

Proposal 6: There is no need to define transmitter intermodulation requirement.

- Ambient IoT BS Rx requirements

  - Reference sensitivity level

The conclusion of reference sensitivity level for A-IoT BS in SI is as follows:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

Copied from R4-2505097 [3]

Issue 2-1-1: Reference sensitivity level

- Agreement:

  - FFS on the definition of channel bandwidth depending on RAN1 conclusions.

    - Consider reference sensitivity requirements for the reader [without frequency shift (R=1)] （114bis，R4-2505097）

    - FFS on side condition of CW for sensitivity requirement（114bis，R4-2505097）

Proposal 7: Consideration： R=1 is  the worst case for sensitivity，suggest to consider reference sensitivity requirements for the reader without frequency shift (R=1)

To determine the D2R modulation SNR, LLS is performed under the assumption in the following table.

Table 6: D2R LLS simulation parameters Applied

| Parameters | Simulation options | HW option |
| --- | --- | --- |
| Carrier frequency | 900 MHz |  |
| SCS | 15 kHz as baseline |  |
| Block structure | Depend on RAN1 |  |
| Channel model | AWGN |  |
| Number of Tx/Rx chains for Ambient IoT device | 1 |  |
| Number of BS antenna elements | 1 |  |
| Reference data rate | 1 kbps (M) |  |
| Message size | Option 1： {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size Option 2： 96 bits | 96 bits |
| BLER target | 10% |  |
| Sampling frequency（SFO） | Option 1: Randomly select a value from the range of [104 ppm~105 ppm] for device 1 Option 2: Use Residual SFO based on companies report | Option1 |
| [OOK/BPSK chip rate] | 7.5 kcps for 2SB 15 kHz Transmission bandwidth |  |
| Receiver bandwidth | Option1: the same as transmission bandwidth. Option 2：18 kHz (consider SFO and BS filter) for 2SB 15 kHz transmission bandwidth Option 3: first sidelobes as receiver bandwidth Companies can report | 18kHz |
| Waveform (CW) | unmodulated single tone |  |
| Modulation | Option1：OOK Option 2：BPSK Companies can report | Option 2 |
| Line code | Manchester encoding |  |
| FEC | 1/3 Convolutional Code |  |
| ADC bit width | Ideal |  |
| D2R receiver | coherent receiver |  |
| Repetition | None |  |
| Preamble configuration | Companies can report | 31 bit m-sequence |
| Midamble configuration | companies can report | No midamble pending on RAN1 |

D2R demodulation SNR simulation result is as follows Figure 5:

Figure 4: D2R demodulation SNR simulation results（BPSK）

Proposal 8: The AWGN D2R modulation SNR is -6dB @ 10% BLER for BPSK.

CW effect to reference sensitivity include 3 main aspects: ideal CW single tone, CW phase noise, reciprocal mixing noise.

For ideal CW single tone:

To evaluate the desens target, maximum input power at A-IoT BS receiver antenna can be calculated as :

CW output power: 33dBm

The distance between CW and A-IoT BS: 20m

Free space path loss for 20m isolation distance: 57dB

Normally the CW node and A-IOT node deploy at the same height, the antenna gain is about -5 dBi at 90°offset from the boresight for both TX antenna and RX antenna.

Maximum input power from CW to A-IoT BS receiver: 33-5-5-57=-34dBm

Analog cancellation in the RF front-end can be applied to reduce the desensitization of the receiver. Assumed analog cancellation capability is 15dB,

The input interference to the A-IoT BS LNA is -34-15=-49dBm, compared with -44dBm narrow-band bloking, A-IoT BS receiver will not be blocked.

For D1T1-B, through spatial isolation(57dB@20m), Analog cancellation （15dB）and high-pass filtering（80dB）, the overall suppression to the carrier-wave interference can reach about (57+15+80) = 152 dB at 20 meters distance. The leakage of CW is 33-152=-119dBm.

A-IoT noise floor：-174+10*lg(15)+30+10=-122.2dBm/15kHz

leakage of CW is 3.2dB higher than noise floor, the desen =10*lg((3.2/10)+1))=4.9dB

To leave some margin, the desens target can be as 6dB.

Proposal 9: Use 6dB as the desensitivity target for reference sensitivity for ideal CW single tone.

For CW phase noise:

The effect of CW phase noise includes:

1) CW leakage level to BS receiver

2) CW phase noise

3) BS’s Phase noise cancellation capability

4) small frequency shift——The worst case occurs with a 15 kHz transmission bandwidth and no frequency shift. Applying a frequency shift reduces phase noise impact—the greater the shift, the lower the impact.

Assume range of residual integral phase noise in 15kHz is 50 to 90dBc, Sensitivity deterioration is calculated as follows:

With 50dBc residual integral phase noise, -34-50=-84dBm

It is 38.2dB (=-84-(-122.2)) higher than noise floor, the desen is 10*lg((38.2/10)+1))=38.2dB;

Table 7: Desen due to CW phase noise

| Residual integral phase noise(dBc) | Higher than -122.2dBm noise floor （dB） | Referense sensitivity Desen （dB） |
| --- | --- | --- |
| 50 | 38.2 | 38.2 |
| 60 | 28.2 | 28.2 |
| 70 | 18.2 | 18.3 |
| 80 | 8.2 | 8.8 |
| 90 | -1.8 | 2.2 |

Observation 1: The impact of CW phase noise on reference sensitivity varies significantly (2.2-38.2 dB range in our calculations) due to differences in phase noise performance and cancellation capabilities.

For reciprocal mixing noise:

In band blocker is -38dBm.

Assume range of l integral phase noise in 15kHz is 50 to 90dBc, Sensitivity deterioration is calculated as follows:

With 50dBc residual integral phase noise, Reciprocal mixing noise =-38-50=-88dBm.

It is 38.2dB (=-84-(-122.2)) higher than noise floor, the desen is 10*lg((38.2/10)+1))=38.2dB;

Table 8: Desen due to reciprocal mixing noise

| Integral phase noise(dBc) | Reciprocal mixing noise(dBm/15kHz) | Higher than -122.2dBm noise floor （dB） | Desen （dB） |
| --- | --- | --- | --- |
| 50 | -88 | 34.2 | 34.2 |
| 60 | -98 | 24.2 | 24.2 |
| 70 | -108 | 14.2 | 14.4 |
| 80 | -118 | 4.2 | 5.6 |
| 90 | -128 | -5.8 | 1 |

Observation 2: The impact of reciprocal mixing noise on reference sensitivity varies significantly (1-34.2 dB range in our calculations from Section 3.1) due to differences in phase noise performance and cancellation capabilities.

Observation 3: Both CW phase noise and reciprocal mixing noise affect reference sensitivity across a wide range, with the exact degradation heavily dependent on phase noise characteristics, cancellation performance, and implementation approach.

  - In-band selectivity and blocking

    - Adjacent Channel Selectivity

For ACS interference aggressor, the potential aggressor is outdoor NR UE.

Copied from R4-2505097 [3]

### Issue 2-3: ACS

- Agreement:

  - Evaluate ACS requirements considering no legacy NR UE indoor assumption

  - For ACS requirements, use legacy NB-IoT requirements as starting point

  - FFS on the frequency offset for ACS requirement

Figure 5 CDF of interference level from outdoor NR UE

Based on simulation, the interference level from outdoor NR UE is -84dBm for case 2-1f, which is much lower than the -52dBm ACS interference level due to penetration loss.There is no need to define ACS requirement.

However, use legacy NB-IoT requirements is also ok.

Proposal 10: Reuse the same frequency offset of legacy NB-IoT requirements.

    - In-band blocking

Similar with ACS situation, the interference level from outdoor NR UE is quite low for case 2-1f. To ensure the A-IoT BS performance, the in-band blocking requirement from NB-IoT standalone Medium Range BS can be reused for A-IoT BS

Proposal 11: Reuse the in-band blocking requirement from NB-IoT standalone Medium Range BS for A-IoT BS.

  - Receiver intermodulation

    - general intermodulation

For general intermodulation, it is assumed that there is no NR interference indoors, so there's no need to define it.

Proposal 12: No need to define general intermodulation for A-IoT BS since it is assumed that there is no NR interference indoors.

    - narrowband intermodulation

Regarding the narrowband intermodulation requirement, it can reuse the requirement of NB Micro BS.

Proposal 13: Reuse FR1 NB-IoT medium range BS Rx narrowband intermodulation requirement for A-IoT BS.

  - In-channel selectivity

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density.

Copied from R4-2502859[4]

### Issue 2-8: ICS

- Agreement:

  - FFS

NB-IoT only defines ICS for in-band operation and does not define it for standalone operation.

No need to define ICS for A-IoT BS.

Proposal 14: There's no need to define ICS for A-IoT BS.

- Summary

In this contribution, we give our opinions of A-IoT BS requirements, and has the following observations and proposals:

For TX requirements:

Proposal 1: Reuse the output power of Medium Range BS for indoor A-IoT BS, Prated,c,AC≤ 38dBm

Proposal 2: For A-IoT BS, the requirements for transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

Proposal 3: Consider RF impairments, the RF envelope requirements can be defined as Table 2.

Table 2: A-IoT BS RF envelope parameters

| Tari | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| 1/(M*15KHz)*10^6 Where M could be 1, 2, 4, 6, 8, 12, 16, 24 | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Mh = Ml | 0 |  | 0.15(A–B) | V/m orA/m |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
|  | RF Pulsewidth | PW |  | 0.5Tari | 0.65Tari | µs |

Proposal 4: The occupied bandwidth for each A-IoT carrier shall be less than the A-IoT BS channel bandwidth.

Proposal 5: Specify 200-800kHz R2D CBW A-IoT carrier ACLR as the following table.

Table 4: A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | Offset(kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 200/400/600/800 | 100 | BWChannel | A-IoT of same BW | Square (BWConfig) | 30 dB |
|  | 100 | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |

Proposal 6: There is no need to define transmitter intermodulation requirement.

For RX requirements:

Proposal 7: Consideration： R=1 is  the worst case for sensitivity，suggest to consider reference sensitivity requirements for the reader without frequency shift (R=1)

Proposal 8: The AWGN D2R modulation SNR is -6dB @ 10% BLER for BPSK.

Proposal 9: Use 6dB as the desensitivity target for reference sensitivity for ideal CW single tone.

Observation 1: The impact of CW phase noise on reference sensitivity varies significantly (2.2-38.2 dB range in our calculations) due to differences in phase noise performance and cancellation capabilities.

Observation 2: The impact of reciprocal mixing noise on reference sensitivity varies significantly (1-34.2 dB range in our calculations from Section 3.1) due to differences in phase noise performance and cancellation capabilities.

Observation 3: Both CW phase noise and reciprocal mixing noise affect reference sensitivity across a wide range, with the exact degradation heavily dependent on phase noise characteristics, cancellation performance, and implementation approach.

Proposal 10: Reuse the same frequency offset of legacy NB-IoT requirements.

Proposal 11: Reuse the in-band blocking requirement from NB-IoT standalone Medium Range BS for A-IoT BS.

Proposal 12: No need to define general intermodulation for A-IoT BS since it is assumed that there is no NR interference indoors.

Proposal 13: Reuse FR1 NB-IoT medium range BS Rx narrowband intermodulation requirement for A-IoT BS.

Proposal 14: There's no need to define ICS for A-IoT BS.

- References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei), RAN#106 Dec.,2024

[2]RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc.，RAN#107 Mar.,2025

[3] R4-2505097 WF on requirements for A-IoT BS and CW，Huawei ,RAN4#114bis, April. 2025

[4] R4-2503217, A-IoT BS RF requirements, Huawei, RAN4 #114bis, April 2025

[5] EPC® Radio-Frequency Identity Generation-2 UHF RFID Standard - Specification for RFID Air Interface Protocol for Communications at 860 MHz - 930 MHz, Release 3.0, Jan 2024