3GPP TSG-RAN WG4 Meeting #115	R4-2505564

Malta, Malta, May 19th – May 23rd, 2025

Agenda item:			7.23.1

Source:	Moderator (Nokia)

Title:	Topic summary for [115][215] FS_NR_AIML_Mob_Part1

Document for:	Information

# Introduction

This summary thread addresses the issues related to AI/ML mobility SI in RAN4. The document is split into 3 high-level topics which are further split into issues.

Moderator’s recommendations are to discuss following issues, listed in the order of priority.

- Issue 2-5: Impacts on beam/cell level measurements

- Issue 2-1: Relative RSRP accuracy definition

- Issue 2-2: Accuracy requirements for the predicted RSRPs

- Issue 2-3: Ground truth definition for RSRP Accuracy for FR1

- Issue 2-4: Impacts of measurement error on prediction accuracy

- Issue 3-1: Performance metric for event prediction use case

- Issue 3-3: Impact on core requirements for event prediction use case

- Issue 3-2: Impact of measurement error for event prediction use case

- Issue 2-6: Impacts on beam/cell level measurements

- Issue 2-7: Inter-frequency prediction scenario

- Issue 2-8: Impact on Core requirements

Furthermore, we will also discuss Issue 1-3 TR-Framework. It may be discussed offline before taking it in an online session.

# Topic #1: General Aspects

This section addresses general aspects and priorities of different scenarios and sub use-cases identified in the draft TR 38.744.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505694 | OPPO | Discussion on general aspects for AI mobility Proposal 1: RAN4 to only study scenarios 2,3,4 in this release. Proposal 2: RAN4 not to consider the combined prediction across multiple domains in this release. Proposal 3: RAN4 to consider all three sub-use cases with no prioritization. Proposal 4: Adopt the text proposal for TR 38.744 in [4] to capture the RAN4 study. |
| R4-2506564 | ZTE Corporation, Sanechips | Discussion on General Aspects of AIML Mobility Proposal 1: Align with RAN2 and RLF prediction will not be studied in Rel-19. Proposal 2: RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. And it is proposed to consider all three sub-use cases for cell level measurement prediction model. |
| R4-2506610 | CMCC | Discussion on general part for AI/ML for mobility Proposal 1: it is proposed to firstly focus on scenario 2, 3, 4. If time allows, scenario 6 can be considered later. Proposal 2: it is proposed to consider all three sub-use cases. |
| R4-2506735 | Huawei, HiSilicon | Discussion on general aspects in AIML mobility Proposal 1: RAN4 will not start the study of RLF use-case in R19. Proposal 2: RAN4 to start the study of other scenarios than scenarios 2,3,4 if RAN2 achieves sufficient progress. Proposal 3: RAN4 to consider all three sub-use cases until RAN2 gives the priority. Proposal 4: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy. Pros and Cons need to be analyzed. - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4. - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4. Proposal 5: RAN4 will study the following two potential options for defining the testing and training dataset. - Option 1: Use the channel model and the sampling rules. - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner. |
| R4-2505949 | Apple | General aspects on AI/ML for mobility in NR Observation 1: RRM measurement predictions provide several advantages, including reducing measurement gaps, increasing the periodicity of SMTC windows, minimizing scheduling and measurement restrictions, decreasing measurement delays, and shortening measurement gap lengths Observation 2: Further discussion is needed in RAN4 regarding future requirements for RLF design: - Should the indication be reported from the UE to the network, or only from the UE's lower layers to higher layers? - If a direct or indirect indication is sent from the UE to the network: - To be determined (FFS): How should such an RLF indication be triggered to the network? - For a direct indication, should the UE report a predicted RLF to the network when all measured and/or predicted downlink radio link quality values on all configured RLM-RS resources fall below a threshold (Qout)? - For an indirect indication, should the UE report all predicted SINR values with corresponding RS indices to the network when the measured and/or predicted downlink radio link quality on all configured RLM-RS resources is worse than Qout? Alternatively, should the UE indicate the predicted SINR with the relevant RS indices as long as at least one configured RLM-RS resource shows measured and/or predicted downlink radio link quality? Proposal 1: RAN4 should evaluate the combined benefits of RRM measurement predictions across multiple domains (such as spatial beam/cell, frequency, and temporal) and dimensions (across various cells and frequencies). Additionally, a working plan should be established to investigate scenarios that maximize these potential advantages. Proposal 2: For measurement event prediction, leverage AI/ML for temporal prediction of {TX, RX} Beam Pair Changes to optimize RRM measurement efficiency. To minimize RRM measurement overhead, the AI/ML model will predict the timing of beam pair changes, enabling the system to conserve resources by focusing measurements only during periods when beam changes are predicted. This approach eliminates the necessity for constant beam monitoring. |
| R4-2507534 | Nokia | On General aspects in AIML Mobility - RAN4 to focus on conducting a detailed analysis of the key RAN2 scenarios #2, #3 and #4 (i.e., FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, frequency domain prediction and FR2 to FR2 intra-frequency temporal domain case A). In Study Item, RAN4 to collect analysis on potential modeling methods and at least capture the collected analysis in the TR. RAN4 to capture the following error modeling principles in the TR: - RF and baseband measurement error modeling may be considered. - A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband. - Alternative modeling approaches are not precluded and can be explored in the Work Item phase. |
| R4-2506547 | vivo | Discussion on general aspects for AI mobility Observation 1: Based on RAN2 latest agreement, for AI/ML for mobility, both the UE-sided model and the NW- sided model are within the scope Proposal 1: In addition to UE sided model, RAN4 also needs to conduct further analysis on potential RRM impact for NW-sided model Proposal 2: RAN4 not to study RRM impact on RLF prediction in Rel-19 Proposal 3: Before performing enough evaluation and reaching an aligned conclusion prediction performance in terms of three sub-use cases, RAN4 not to have any deprioritization given to any sub-use cases |
| R4-2507666 | Ericsson | General discussion on AI/ML for mobility Proposal 1: RAN4 to identify required accuracy for the measurement(s) used as input(s) to the AI/ML model to guarantee the UE predicted measurement meets the requirements for L3-based mobility. Proposal 2: Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes. Proposal 3: Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements. - RF error in dB for FR1 is characterized by Gaussian distribution with μ=0, σ2 = 2.5 truncated at ±2.5. - RF error in dB for FR2 is characterized by Gaussian distribution with μ=0, σ2 = 4 truncated at ±4. Proposal 4: RAN4 to identify requirements for UE reporting measurements for data collection for network sided model for AI/ML based mobility. Proposal 5: RAN4 to either make an agreement to ensure that the agreement on scenario prioritization in the study item phase does not limit discussion on impact on RAN4 requirement for scenario 1, 5, and 6 in the normative phase provided the reporting framework for AI/ML aided mobility supports them or capture all scenarios in section for RAN4 in TR38.744 with a note to clarify that discussion on scenarios 2, 3, and 4 were prioritized in study item but requirements for scenarios 1, 5, and 6 are not precluded to be specified. |
| R4-2505695 | OPPO, Nokia | TP on RAN4 aspects for TR 38.744 |
| R4-2507665 | Ericsson | TP for TR38.744 |

## Open issues summary

### Issue 1-1: Scenario priorities

- Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- Proposals and Observations:

  - Proposal 1 (OPPO, Nokia): RAN4 to only study scenarios 2,3,4 in this release.

  - Proposal 2 (ZTE): RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case.

  - Proposal 3 (CMCC): it is proposed to firstly focus on scenario 2, 3, 4. If time allows, scenario 6 can be considered later.

  - Proposal 4 (Huawei, HiSilicon): RAN4 to start the study of other scenarios than scenarios 2,3,4 if RAN2 achieves sufficient progress.

  - Proposal 5 (Ericsson): RAN4 to either make an agreement to ensure that the agreement on scenario prioritization in the study item phase does not limit discussion on impact on RAN4 requirement for scenario 1, 5, and 6 in the normative phase provided the reporting framework for AI/ML aided mobility supports them or capture all scenarios in section for RAN4 in TR38.744 with a note to clarify that discussion on scenarios 2, 3, and 4 were prioritized in study item but requirements for scenarios 1, 5, and 6 are not precluded to be specified.

- Candidate Options:

  - Option 1: RAN4 to only study scenarios 2,3,4 in this release.

  - Option 2: In addition to scenario 2,3 and 4, RAN4 to start the study of scenario 6 if RAN2 achieves sufficient progress.

  - Option 3: RAN4 to agree that scenario prioritization in the study item phase does not limit discussion on impact on RAN4 requirements for scenario 1, 5, and 6 in the normative phase provided the reporting framework for AI/ML aided mobility supports them.

  - Option 4: RAN4 to capture all scenarios in TR38.744 with a note to clarify that discussion on scenarios 2, 3, and 4 were prioritized in study item but requirements for scenarios 1, 5, and 6 are not precluded to be specified.

    - Option 4a: Inter-frequency inter-cell prediction should consider discussion of non-collocated cell scenarios in future evaluations

- Recommended WF:

  - RAN4 to discuss the above-listed candidate options.

### Issue 1-2: Sub-use case priorities

- RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model). In RAN2, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Proposals and Observations:

  - Proposal 1 (OPPO, ZTE, CMCC, Huawei, vivo): RAN4 to consider all three sub-use cases with no prioritization.

- Recommended WF:

  - RAN4 to consider all three sub-use cases with no prioritization.

### Issue 1-3: TR framework

- Proposals and Observations:

  - Proposal 1 (OPPO, Nokia): Adopt the text proposal for TR 38.744 in [R4-2505695] to capture the RAN4 study.

  - Proposal 2 (Ericsson): Adopt the text proposal for TR 38.744 in [R4-2507665] to capture the RAN4 study.

- Recommended WF:

  - Discuss the TR framework in R4-2505695 and R4-2507665.

### Issue 1-4: Radio Link Failure (RLF) prediction use case

- Proposals and Observations:

  - Proposal 1 (ZTE, Huawei, vivo, CATT): RAN4 not to study RRM impact on RLF prediction in Rel-19

- Recommended WF:

  - RAN4 doesn’t need to define requirements for RLF prediction use case.

### Issue 1-5: Requirements for Data Collection for NW sided model

- Proposals and Observations:

  - Proposal 1 (Ericsson): RAN4 to identify requirements for UE reporting measurements for data collection for network sided model for AI/ML based mobility.

  - Proposal 2 (Huawei): If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed.

- Recommended WF:

  - RAN4 to discuss the above proposals.

# Topic #2: RAN4 requirements for RRM Measurement prediction

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2507432 | Nokia | On RAN4 Impacts for RRM measurement prediction in AIML Mobility Observation 1: TS 38.133 supports relative intra-frequency measurement accuracy evaluations in both inter-cell and intra-cell measurement contexts. For intra-cell cases, this includes comparisons between beams of the same cell, and for inter-cell cases, between cells on same carrier frequency. Proposal 1: For intra-frequency L3-RSRP relative accuracy definition, RAN4 to cover comparison between two RSRP levels from the same cell or different cells. Proposal 2: When defining the relative accuracy for intra-frequency predicted L3-RSRP, RAN4 to consider both of the following cases: Comparison between two predicted values, and comparison between one predicted value and one measured value. Proposal 3: RAN4 consider the following definition of accuracy of relative in intra-frequency; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP level 1 – reported RSRP level 2) – (ground truth of RSRP level 1 – ground truth of RSRP of level 2), where level 1 and level 2 may belong to the same or different cells on the same frequency. [Note: Reported RSRP can be either measured or predicted] Observation 2: Current draft definition of inter-frequency predicted relative L3-RSRP accuracy allows comparison between a predicted value and a measured value only when the predicted value is inferred based on the measured value. Proposal 4: RAN4 to extend the definition of inter-frequency predicted relative L3-RSRP accuracy to include comparisons between a predicted value and a measured value, regardless of whether one is used as the input for the other. Proposal 5: For inter-frequency scenarios, RAN4 consider the following definition relative accuracy; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2. [Note: Reported RSRP can be either measured or predicted] Observation 3: It is expected Case-B temporal domain prediction with applied L3 filtering to include both combination of measured and predicted values and comply with accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement. Proposal 6: For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement. Observation 4: Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE. Observation 5: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered by UE while performing measurements that serve as ground truth. Proposal 7: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions. Proposal 8: RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E). Proposal 9: RAN4 to not consider number of L1/L3 measurements performed during OW and its impacts on the prediction accuracy Observation 6: The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT with multiple UE speed cases. Proposal 10: For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed. Proposal 11: If skipping pattern information is not shared by the UE, RAN4 to only consider MRRT, OW and PW to UE as test configuration and set up a requirement on prediction accuracy. Proposal 12: If skipping pattern information is shared by the UE, TE should select the supported skipping pattern related to the MRRT to test. The test configuration should then include skipping pattern, OW and PW. Proposal 13: For inter-frequency prediction scenario, RAN2 has no sufficient clarity on the definition of correlation coefficient; therefore, RAN4 to wait for the progress in RAN2. Observation 7: Feeding the input from various sources might enhance AIML model performance. Proposal 14: For inter-frequency prediction scenario, in a cluster approach, RRM measurement prediction performance can be enhanced when measurement from a group of cells in one carrier frequency is used by the UE as an input. Therefore, RAN4 should consider the impact of cluster approach. |
| R4-2506548 | vivo | Discussion on RAN4 impacts for RRM measurement prediction Prospoal 1: For intra-frequency L3-RSRP, regarding the definition- ‘Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.’, If interpreted in the following manner, the definitions under both approaches can be considered. - When the beam here refers to L1 beams, it corresponds to the scenario of predicting L3 (at point A or A1) from L1 - When the beam here refers to L3 beams, it corresponds to the scenario of predicting RSRP at point E. Proposal 2: For inter-frequency L3-RSRP, regarding the definition ‘accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].’, - For the case that the reported RSRP of Cell2 is predicted, it needs to firstly clarify which scenario this actually corresponds to - For the cases that the reported RSRP of Cell2 is measured and is a combination of both predicted and measured samples, both of these two cases can be considered Proposal 3:As the key element affecting RSRP prediction accuracy for inter-frequency prediction, correlation coefficient need to consider it when defining metrics. Regarding EPRE difference, as well as frequency distance and frequency separation, hese are coupled with the correlation coefficient and no need to splitting them into more detailed factors for discussion Observation 1: The performance of the cluster-based approach using all cells is better than that using a single cell as input. Proposal 4: For the cluster-based approach, the criteria for selecting cells can influence inter-frequency prediction accuracy. This factor could also be considered as a potential aspect Proposal 5: For inter-frequency prediction, the OW length can be considered as one of the side conditions Proposal 6: For the following conclusion made in RAN4#114bis, RAN4 to update the wording as: For Rel-19 study, RAN4 to not consider L1 beam-level (Point A1) measurement prediction for UE sided model when L1 beam-level (Point A1) predicted results as the report content Proposal 7: On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline: - For BB error modeling: - Link-level simulation approach is considered - For RF error, - For error modeling, the following two modelings can be the candidates. - Alt 1: Uniform distribution - Alt 2: Truncated Gaussian distribution - RF error application, each of Tx beam is independent Proposal 8: Regarding the Ground Truth definition for RSRP accuracy, the analyses of Alt 1 and Alt 2 are shown as below Issues; Impact to Alternative 1; Impact to Alternative 2. 1.; The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction; No with AWGN channel is assumed: If the AWGN channel is considered, it is assumed that the interval duration between time instances in both the observation and prediction windows are aligned, and their corresponding starting points are also aligned. The feasibility lies in the fact that both the measurement and prediction configurations can be adjusted. The timing information can be known to the TE by controlling both the measurement configuration and prediction timing. YES with fading channel is assumed: Under a fading channel, due to dynamic changes in the channel environment, obtaining ground truth may involve significant inaccuracies.; No with certain conditions: This issue emphasizes whether timing information can be obtained on the TE (Transmission Entity) side. Although the exact execution and completion time points of measurements are known on the UE side, if UE reporting is considered, the ways for the TE to be aware of the timing information include: Same as the analysis for Alt 1 (AWGN case) on the left The measurement results reported by the UE include timestamp information indicating when the measurements were performed. 2.; Impact of L3 filtering; YES, in certain scenarios: The filtering coefficient can be network-configured and thus known to the TE (at least for the case where L1 predicts L1 and then calculates L3). However, for the case where L1 directly predicts L3, the L3 filter is embedded in the model, and the consolidation/filtering process is treated as a black box—meaning the TE cannot know the filtering details. For L1 predicts L1 → then calculates L3: The filtering coefficient is a specific value configured by the network and can be known to the TE. For L1 directly predicts L3: The L3 filter is integrated into the model, and the consolidation/filtering process is black box, so the TE has no visibility into it.; YES, in certain scenarios: Similar to Alt 1, for the case where L1 directly predicts L3, the L3 filter is embedded in the model, and the consolidation/filtering process is treated as a black box. During UE measurements, the filtering coefficient may be based on network configuration. However, during prediction, the process may depend on UE implementation, leading to potential inconsistencies between: The filtering coefficient and the specific process used for the obtained measured value, And the actual model implementation (which remains opaque to the TE). This discrepancy could impact the alignment between measurement and prediction behaviors.. 3.; Number of samples of L1 filtering; L1 filtering and the number of samples for filtering are not known by the TE and entirely depend on UE implementation.; NO. 4.; Timing window, if it exists, where UE can’t measure the ground truth in Alt2; NO; In Temporal Case B, ground truth may be missing if measurements are skipped. How to obtain the true measurement values at the corresponding moments requires further analysis.. 5.; Channel condition; Under fading channels, due to the dynamic changes in the channel environment, the acquisition of ground truth may be highly inaccurate.; NO. 6.; Avoid UE cheat in the test; NO; No feasible solution has been found yet, even in the discussion of the R19 AI interface beam management test.. 7.; S(I)NR variations in the test at different time instances (for Alt2); NO; No with certain conditions the signal environment corresponding to the measured value is the signal environment at the moment of the predicted time instance, and there is no influence caused by changes in channel conditions or excessively low channel conditions. |
| R4-2506736 | Huawei, HiSilicon | Discussion on impacts for RRM measurement prediction Proposal 1: RAN4 to start the discussion on scenario 2, 3 and 4 with high priority. Scenario 6 (FR2 to FR2 intra-frequency spatial domain for the 1st goal) can be carried out after sufficient progress are achieved on high priority scenarios. Proposal 2: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed. Proposal 3: For intra-frequency temporal domain prediction case A, observation window length (or called as prediction evaluation period) and prediction window length may need to be specified. Proposal 4: To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing. Proposal 5: For intra-frequency temporal domain prediction case B, RSRP prediction accuracy is to be defined under the side condition of MRRT with limited PW length. Proposal 6: For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy is to be specified under certain conditions, - channel correlation coefficient - FFS on cluster/single cell approach, depending on whether explicit indication and corresponding UE capability are specified in RAN2. Proposal 7: cluster approach or single cell approach would impact on the value of prediction accuracy. Proposal 8: The processing time of inference function may need to be considered in measurement reporting delay. Proposal 9: For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C. Proposal 10: RAN4 to discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases. Observation 1: Fading channel is to be employed for AI mobility measurement prediction. Observation 2: For either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a L3 predicted result is unknown to TE. Observation 3: The number of L1 measurement samples is unknown to TE. Proposal 11: If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles: - The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE. - The number of L1 measurement samples is unknown to TE. Proposal 12: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions. - how to avoid UE cheating can wait the progress in R19 AI interface beam management test. Proposal 13: The below definition of beam-level relative predicted accuracy of beam-level L3 RSRP is needed if sub-use cases of L3 filtered beam-level measurement predication are considered in RAN4 in R19, otherwise the definition can be deprioritized. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. Proposal 14: For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, - if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, or - if cell 1 is the target cell based on intra-cell prediction in temporal domain Proposal 15: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified. Proposal 16: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements. Proposal 17: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface. |
| R4-2507667 | Ericsson | On requirements for RRM measurement prediction for AI/ML based mobility Observation 1: For a fixed MRRT, prediction accuracy for temporal prediction case-B may differ based on the measurement skipping pattern within OW. Proposal 1: RAN4 to study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B. Proposal 2: Reformulate the term “Accuracy of predicted relative L3-RSRP” in the agreements reached in RAN4#114bis to “Relative accuracy of predicted L3-RSRP measurements”. Proposal 3: Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), if cell 2 is measured cell to infer measurement for cell 1 (target cell) in the same carrier frequency. Proposal 4: Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different carrier frequency than cell 2 but in the same FR as cell 2 and cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2. Proposal 5: Definition for relative accuracy of predicted L3-RSRP measurements in intra-frequency scenarios where beam 1 and beam 2 are from the same cell is not needed. Proposal 6: Relative accuracy of predicted L3-RSRP measurements for temporal prediction is defined as: - Relative accuracy of predicted L3-RSRP measurements = (reported predicted RSRP1 of a cell/beam at time T1 – reported RSRP2 of the same cell/beam at time T2) – (ground truth of RSRP1 of a cell/beam at time T1 – ground truth of RSRP2 of the same cell/beam at time T2). - Reported RSRP2 can be predicted or measured. - T1 and T2 refer to time instances for temporal domain prediction. Observation 2: RLF prediction is out of Rel. 19 study item scope. |
| R4-2506611 | CMCC | Discussion on RAN4 impacts for RRM measurement prediction Proposal 1: for accuracy of predicted relative L3-RSRP for inter-frequency, it is proposed that the reported RSRP of cell2 can be predicted or measured. Proposal 2: whether to consider Top K/N (%), N>=1 as the performance metrics for AI/ML-based L3-RSRP prediction is pending on whether the case that model output is cell ID is considered in RAN2. Proposal 3: Table 2 provide the comparison between Alternative 1 and Alternative 2 for ground truth definition for FR1. Table 2 Comparison between Alternative 1 and Alternative 2 for ground truth definition for FR1 Alternative 1 (Ground truth for FR1 will be based on the transmitted or reception power); Alternative 2 (Ground truth for FR1 will be based on the reported measurement value under certain conditions). Impact of L3 filtering; If L3 filtering is considered, how to perform L3 filtering is based on L3 filtering formula with filtering coefficient configured, and TE has no such information; N/A. Impact of L1 filtering; How to perform L1 filtering, e,g, the number of samples, whether sliding or non-sliding filtering in use, is up to UE implementation, and TE has no such information; N/A. Timing window; N/A; For Case B (target goal is to reduce measurement overhead), the time instance in prediction window will not be measured, which means UE cannot measure and report the ground truth.. Channel condition; N/A; For fading channel, the channel condition at T1 to get ground truth and the channel condition at T2 to predict is different, which result in mismatch. UE cheating issue; N/A; Since both the ground truth and predicted value are reported by UE, there may be cheating issue. Proposal 4: for cluster approach, it is proposed to follow RAN2 agreements that model takes measurements from more than one cell as inputs. Proposal 5: it is proposed to discuss whether to introduce RAN4 requirements about MRRT and MRRS. Proposal 6: for RRM measurement prediction, the measurement delay requirements need to be defined/updated - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements |
| R4-2507801 | Qualcomm Incorporated | Impact of AI based mobility on RRM measurement prediction Definition of ground truth for AI/ML-based RRM measurement prediction Observation 1: It is not reasonable to assume that the same L3-RSRP measurement result at Point C in Figure 9.2.4-1 of TS38.300 should be observed across all UEs, particularly when the channel evolves over time and the measurement reference signals are contaminated due to low SNR, UE mobility, inter-cell/inter-sector interference, etc. Observation 2: It is not reasonable to assume that the predicted strongest neighbor cell would have as high SNR as the predicted strongest beam from the serving cell. Considering the geographical location of the UE, where the AI/ML-based neighbor cell’s RSRP prediction is of interest, is where the UE is not close to the serving cell but near the cell boundary, even the serving cell’s SNR is not expected to be high enough Observation 3: Considering that the models are not likely to be trained solely based on synthetic data generated by simulators, both the model input corresponding to the observation window and the ground truth of the model outputs corresponding to the prediction window will essentially be data sets generated based on actual UE measurement results, not so-called error-free ideal measurement results. Observation 4: The ground truth being the actual measurement result is also in line with the way NW and UE monitor the performance of the AI/ML-based RRM prediction. The effective KPI for performance monitoring is the difference between the prediction result and the actual measurement result on the performance monitoring reference signals, as there is no practical way of defining so-called error-free ideal measurement results. Observation 5: The UE’s L3-RSRP measurement accuracy performance will be ensured by the current requirements and test framework. Thus, the UE’s reported measurement results in more dynamic environments can serve as the benchmark for AI/ML-based L3-RSRP prediction accuracy performance requirements. - For testing, the following two-step test may thoroughly verify AI/ML-based L3-RSRP prediction performance: - Existing L3-RSRP Measurement Accuracy: - It is tested under static channel conditions (i.e., no RSRP variation during the entire test run), hence there is no ambiguity in the definition of the ground truth (or reference) RSRP. - With this, it can be claimed that the reported RSRP from the UE can serve as the ground truth for AI/ML-based L3-RSRP prediction in the second step below. - New Test Case for AI/ML-based L3-RSRP Prediction under Time-varying RSRP Conditions: - UE can be tested under multiple different RSRP variation profiles. - X% of the data set can be used for AI/ML model training. Note that it does not mean online training. - The rest of the data set can be used for AI/ML-based L3-RSRP prediction testing. - At time instance T0, UE reports AI/ML-based L3-RSRP prediction results based on previous observations. The reports correspond to future time instances. - UE and NW will be in sync in terms of how far into the future the measurement prediction results correspond to. - The accuracy/error can be defined as the difference between 'reported prediction results' and 'reported actual measurement results' corresponding to the same or closely timed instances. - The above is expected to allow UE to collect actual measurement results on/offline (measurement errors and UE implementation specifics are inherited in the data set) for model training and testing. Observation 6: For the same channel realization, it was observed that the IIR-filtered L3-RSRP can vary depending on the UE L3-RSRP measurement rates and the start position of the L3-RSRP measurement in the time domain. Therefore, a margin to accommodate uncertainties in the actual UE L3 measurement rate, etc., needs to be introduced on top of the ground truth if the ground truth for L3-RSRP is defined based on expected measurement results. Proposal 1: RAN4 to consider the following options for the definition of ground truth of L3-RSRP for the predicted absolute L3-RSRP accuracy (reported predicted L3-RSRP – ground truth of L3-RSRP). If the down selection needs to be made during the study item phase, RAN4 should consider Option 2-A, Option 2-B, and Option 1 in that order of priority. Note that the possible solutions and potential issues for each option are included within the respective options as well. - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP) - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate. The value of the margin needs a further study, and it can vary for different measurement configurations, etc. - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc. - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition as the prediction-based L3-RSRP report (UE-reported L3-RSRP with noise) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms. - To avoid the UE from cheating the test, TE can either create another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction or run two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report. - This definition is the closest to the way the UE evaluates and monitors AI/ML performance based on the LCM framework. - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough. - Another round of tests would be needed for the L3-RSRP measurement report - RAN4 to strive to not differentiate the definition of the ground truth between FR1 and FR2. Performance metrics for AI/ML-based RRM measurement prediction Observation 7: The relative cell level strength metric is important for event-based L3-RSRP reports and conditional handovers, as they are widely used in the field for measurement gap configuration, handover preparation, and handover execution decisions. Among multiple event types, the most commonly used event type is Event-A3, which is defined based on the relative cell level strength between the serving cell and a neighbor cell. Proposal 2: RAN4 to update the definition of relative L3-RSRP agreed in RAN4#114bis as follows: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - Whether and how to define the requirements on the accuracy of predicted relative beam level L3-RSRP, when the two beams are from the same cell, should be discussed further during the work item phase. - Note that whether it becomes the same as the accuracy of predicted relative beam level L3-RSRP would depend on how the definition of the ground truth is established. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2 or another cell on a different frequency than cell 2 but in the same FR as cell 2. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - The applicability of the relative RSRP accuracy definition should be confined within the same frequency range Observation 8: AI/ML-based RRM prediction is not expected to entirely replace actual UE measurements on neighbor cells, particularly during handover, due to the required downlink timing and frequency synchronization procedure with the target cell. If the AI/ML model can accurately predict the N strongest neighbor cells, the serving cell may reconfigure the measurement objects accordingly, allowing the UE to perform actual measurements on those cells. Observation 9: The RAN2 agreement also hints that the network would not solely rely on AI/ML-based RRM prediction results but would require the report of actual measurement results when deciding UE mobility. Proposal 3: RAN4 to consider the following as one of the performance metrics for AI/ML-based L3-RSRP prediction. If the down selection needs to be made during the study item phase, RAN4 should consider Top-K(%) and Top-K/1(%) in that order of priority. - Top-1 (%): The percentage of ‘the Top-1 strongest cell is Top-1 predicted cell’ - Top-K/1 (%): The percentage of ‘the Top-1 strongest cell is one of the Top-K predicted cells’ - Top-1/K (%): The percentage of ‘the Top-1 predicted cell is one of the Top-K strongest cells’ - Top-K (%): The percentage of the maximum ground truth L3-RSRP among the Top-K predicted cells being larger than ‘the ground truth L3-RSRP of the strongest cell’ - X dB - Note: Top-1 strongest cell and Top-K strongest cells are determined by the metric of ground truth L3-RSRP. Inter-frequency prediction scenario Proposal 4: RAN4 to not consider EPRE difference as one of the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario. RRM prediction performance and impact of measurement error Proposal 5: RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM, e.g. - Whether and how to accommodate the impact of measurement inaccuracies for a cluster-based approach. - Whether and how to model the impact of imperfect RF calibration for inter-frequency RRM prediction. - Whether different aspects need to be considered for Case A and Case B in intra-cell/intra-frequency RRM prediction. - Note that the effective differences between Case A and Case B, from a UE measurement and prediction perspective, might be limited to the measurement rate within the observation window and the prediction range into the future. Case A and Case B may have different impacts on RRM requirements, particularly about measurement/scheduling restrictions. Potential impact on RRM requirements Proposal 6: RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g. - Common aspects: - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc. - Measurement and scheduling restrictions in the prediction window. - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation. - Side conditions such as requirement applicability rules. - Overall budget in terms of mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements. - Scenario 2: FR1 to FR1 intra-frequency temporal domain case B, Intra-cell - Uniform measurement approach for cells in the same frequency layer - Scenario 3: FR1 to FR1 inter-frequency, Inter-cell - CSSF - MG - Scenario 4: FR2 to FR2 intra-frequency temporal domain case A, Intra-cell Uniform measurement approach for cells in the same frequency layer |
| R4-2507123 | NTT DOCOMO, INC. | Discussions on requirements for RRM measurement prediction on AIML mobility Observation 1: If reported RSRP is based on measured value, it will be the same value as ground truth of RSRP, or almost same value if taking measurement error into account. In consequence, the relative RSRP accuracy becomes absolute RSRP accuracy of cell 1. Observation 2: If combination of both predicted and measured samples is applied for reported RSRP of cell2, the requirement value on accuracy of predicted relative L3-RSRP is between the predicted samples and the measured samples. Proposal 1: Predicted samples are applied for the reported RSRP of cell 2 to keep relative RSRP accuracy meaningful. Proposal 2: No need to consider the combination of predicted and measured samples from RAN4 accuracy requirement perspective. |
| R4-2506563 | ZTE Corporation, Sanechips | Discussion impacts on requirements of AI/ML mobility Observation1: Both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. And according to RAN2’s agreements, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4). Proposal 1：RAN4 should study the impacts on requirements for L3 cell-level prediction first and the prediction accuracy for L3 beam-level prediction can use the conclusion of L3 cell level as baseline. Proposal 2: RAN4 should discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases. Proposal 3: For temproal domain cell level prediction, no need to consider predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell. Proposal 4: If RAN4 consider the relative accuracy for spatial domian or L3 beam level prediction, predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell is needed. Proposal 5: For the case that cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, the reported RSRP of cell2 should be measured results. Proposal 6: RAN4 should study the RRM impact of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances. Proposal 7: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements. And RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility. |
| R4-2505696 | OPPO | Study of impacts on RAN4 requirements for RRM measurement prediction Proposal 1: For the definition of relative accuracy for intra-frequency measurement, not to consider RSRP difference between different beams from the same cell. Proposal 2: For the definition of relative accuracy for inter-frequency measurement, not to consider RSRP difference between predicted results and measured results, i.e., only consider the difference between two predicted results. Proposal 3: Use Alt2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1. Proposal 4: RAN4 to consider certain channel model, beam consolidation configuration and L3 filter configuration as possible side conditions when defining the accuracy requirements and testing. Proposal 5: Consider the following options to model the measurement error: - Option 1: For RF error, use the truncated Gaussian distribution under ±X dB; for baseband error, use the link level simulation assumptions to generate L3-RSRP difference as baseband error. - Option 2: For RF and baseband error, use the truncated Gaussian distribution under ±X dB and ±Y dB, respectively. - Option 3: For the sum of RF and baseband error, use the truncated Gaussian distribution under ±X dB. Proposal 6: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length. Proposal 7: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values. Proposal 8: RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation. Proposal 9: RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction. Proposal 10: Wait for more progress from RAN1/2 AI mobility and RAN4 AI BM discussion for LCM related requirements. |
| R4-2506306 | LG Electronics Inc. | Discussion of RAN4 impacts for RRM measurement prediction Proposal 1: The beam-level relative L3-RSRP accuracy requirement is not necessary for intra-frequency L3-RSRP. Proposal 2: Regaring inter-frequency L3-RSRP, for the predicted relative RSRP accuracy requirement, the reported RSRP of cell2 should be predicted value. Proposal 3: Alternative 2 is more reasonable and feasible method for ground truth definition for RSRP accuracy for FR1. Proposal 4: RAN4 to consider only Gaussian error model regardless what RF error model or BB error model is. |
| R4-2506171 | Tejas Network Limited | Study of RAN4 impacts for RRM measurement prediction for AIML mobility in NR Observation 1: Predicting multiple inter-frequency measurements can be challenging due to varying channel conditions across different frequencies Proposal 1: RAN4 to clarify the difference between ground truth RSRP and reported RSRP Proposal 2: RAN4 should explore potential enhancements in inter-frequency measurement prediction by combining with temporal domain prediction. Proposal 3: It is proposed that RAN4 not study the impact of cluster-based and non-cluster-based inter-frequency prediction scenarios. Proposal 4: No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario. - Proposal 5: RAN4 should study the mobility enhancements based on L3 cell-level measurement predictions in conjunction with the relaxation of L1 beam-level measurements. |
| R4-2505766 | Xiaomi | Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility Proposal 1: For intra-frequency, the definition of the accuracy of predicted relative L3-RSRP shall be based on different UE capabilities: - If UE support to predict for multiple cells: Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency - If UE doesn’t support to predict for multiple cells: Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported measured RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency Proposal 2: Accuracy of predicted relative L3-RSRP for different beams of the same cell is not needed. Proposal 3: For inter-frequency, if UE support to predict for multiple cells, the relative RSRP can be compared between two different predicted RSRPs. Otherwise, the relative RSRP can be compared between one predicted RSRP and one measured RSRP from two different cells. Observation 1: When timing mismatch increases to 200ms, RSRP difference increases to 5.5dB. Observation 2: Prediction interval for sliding and non-sliding widow is different, which will have impact on degerming the reported time instance. Proposal 3: Timing between predicated L3-RSRP and ground truth L3-RSRP needs to be aligned, how to align timing is highly dependent on reporting scheme designed by RAN2. If timing alignment is infeasible, incorporate additional margins into accuracy thresholds. Observation 3: L1 filtering sample number will have impact on deriving ideal L3-RSRP value from TE side. Observation 4: if 3 samples are used to derive L1 filtered RSRP, RSRP delta will be within 3dB for 90% cases when compared with that case that 5 samples are used. Proposal 4: L1 filtering number needs to be aligned, otherwise, extra margin needs to be considered. Observation 5: If the L3 filtering factor α is not zero, the L1/L3 filtering method will influence the derivation of the ideal L3-RSRP value from the TE side. Observation 6: At the same time T, ideal L3 filtered RSRP difference between non-sliding and sliding mode can be small than 1.8dB for k=4 in 90% cases. If K=0, no L3 filtering is applied and there is no RSRP difference. Proposal 5: L3 filtering impact can be removed by setting k=0, extra margin needs to be considered. Proposal 6: If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports. Test Equipment (TE) can employ multiple anti-cheating strategies such as: - Multi-instance channel scrambling: Randomizing the order of predefined channel sequences. - Insertion of redundant channels: Introducing randomly generated non-functional channels. - Randomized temporal offset: Reconfiguring channels at an unknown time offset within a specified Proposal 7: Observation window and prediction window defined in RAN2 will have RRM impact: - Observation window will have impact on measurement duration - Prediction window will have impact on scheduling restriction interval to disrupt UE's pattern recognition. Observation 7: Observation window and prediction length is dependent on different L1/L3 filtering method. Observation 8: Observation window and prediction length is dependent on different sub-cases. Observation 9: Observation window and prediction length is dependent on different patterns. Proposal 8: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window. Observation 10: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period. Proposal 9: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period. Observation 11: EPRE may not have impact on prediction accuracy if EPRE is relative static. Observation 12: The channel correlation coefficient is significantly influenced by factors such as antenna pattern differences, bandwidth variations, LOS probability variations, frequency scaling of large-scale parameters, cluster delays and angles, independent shadowing, and blockage. Observation 13: For cluster based approach, intra-frequency cluster Focuses on more about temporal relation while Inter-frequency cluster focus more about frequency domain relation. How to choose multiple cells in the same cluster will have impact on the final prediction accuracy. Observation 14: Extended measurement intervals degrade inter - frequency prediction performance by causing data obsolescence, which fails to capture rapid channel changes and results in inaccurate RSRP predictions. Proposal 10: RAN4 to study the factor and impact of measurement error on RRM prediction. |
| R4-2506507 | MediaTek Inc. | Discussion on RAN4 impacts for RRM measurement prediction Proposal 1: For cell-level RRM measurement prediction, only use cell level RSRP (Point C) as the performance metric for all the three sub-use cases. Observation 1: For scenario 3 (inter-frequency measurement prediction), it is not possible set a fixed correlation coefficient factor and then generating the channels of measured frequency layer or predicted frequency layer fulfil the correlation coefficient. Proposal 2: For inter-frequency prediction scenario, it is not feasible to study the impact of correlation coefficient unless RAN4 define a common dataset. Proposal 3: During the SI, RAN4 not to do down-selection on the two Alternatives of ground truth definition. No matter which alternative finally adopted, the impact of the instantaneous fluctuation at different occasions due to fast fading shall be considered. Observation 2: For scenario 3 (inter-frequency measurement prediction), it is possible to inform UE cell ID of the inter-frequency neighbor cell(s) to be predicted for collocated cases while may be not possible for non-collocated cases. Proposal 4: For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown. Proposal 5: Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring. Proposal 6: For AI mobility, in FR1, measurement error should be added as: - BB error: - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR. - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement. - RF error: - Modelled as Gaussian distribution under ±3 dB RF error (μ=0, σ2=3). - A random RF error is generated for all the beams of the same frequency layer per SMTC. Proposal 7: Regarding measurement report delay requirements for AI mobility, leverage the conclusion on AI/ML BM as much as possible. Not to discuss the impact on measurement report delay requirements for AI mobility in SI to avoid duplicated discussions. Proposal 8: Regarding the impact on measurement delay requirements for AI mobility Scenario 2, leverage the conclusion on AI/ML BM case 2 as much as possible. Not to discuss the impact on measurement delay requirements for AI mobility Scenario 2 in SI to avoid duplicated discussions. Proposal 9: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved. Proposal 10: Scenario 4 has no impact on measurement delay requirements. |
| R4-2505985 | CATT | Discussion on impacts of AIML RRM measurement prediction on RRM requirements Proposal 1: RAN4 do not study radio link failure use case in Rel-19. Proposal 2: For scenario 4 (2nd study goal), the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes. Proposal 3: For scenarios with 1st study goal, RAN4 discuss whether a relaxed prediction accuracy requirement is defined. Proposal 4: RAN4 to capture the relative RSRP prediction definition for intra-frequency beam L3-RSRP in the TR as one of study outcomes. Proposal 5: The reported RSRP of cell 2 can be measured at least. RAN4 further discuss it in WI phase after RAN2 finish the design on observation window and prediction window configuration. Proposal 6: RAN4 to deprioritize the study on impact of L3 beam-level measurement prediction at point E to align with RAN2 prioritization. Proposal 7: RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed. |
| R4-2505950 | Apple | Study of RAN4 impacts for RRM measurement prediction for AIML Mobility in NR Observation 1: Scenario #2 for AI/ML in Release-19 Mobility is similar to Release-18 AI/ML beam prediction (Case-2), except that BM Case-2 allows set B to differ from set A. They are identical when set B = set A - Observation 2: In Temporal Domain Case B, if measurements in the prediction window are intentionally skipped, the corresponding L3-RSRP values for those time instances may not be available. This absence of reported measurements undermines their viability as ground truth for validation, necessitating alternative approaches or re-evaluation of measurement protocols for such scenarios. - Observation 3: Relative RSRP accuracy is essential for use cases where predictions rank or compare multiple cells (e.g., identifying the strongest neighbor cell for handover). However, RAN2’s current simulations and requirements focus on absolute RSRP accuracy (deviation between predicted and ground truth RSRP for a single cell). - Observation 4: Existing AI/ML models and test frameworks in RAN2 do not yet address scenarios where predictions output RSRP for multiple cells simultaneously - Proposal 1: RAN4 to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E) Proposal 2: RAN4 to investigate the impact of measurement errors (RF and baseband) in AI/ML model input data on the accuracy of predicted measurements. Additionally, RAN4 should investigate whether RAN2 simulation results and conclusions regarding achievable prediction accuracy remain valid under such measurement errors for scenario 2. Proposal 3: For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C). Proposal 4: Investigate the translation of RAN2 HO KPIs into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR. Proposal 5: To support the increase of MGRP, RAN4 should investigate the feasibility of predicting measurement results for multiple cells per frequency layer. Additionally, it should examine the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps. Proposal 6: In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells. Proposals 7: Scenario 3 (Frequency Domain Prediction) can be extended to incorporate multi-cell and multi-frequency layer inputs, as well as time-domain samples as well as multiple cell outputs. This integration would enable measurement gap reduction by leveraging cross-layer and temporal correlations in the data. Proposal 8: RAN2’s assumption of collocated cells for frequency domain prediction may represent an unrealistic scenario. RAN4 to investigate frequency domain prediction methodologies for non-collocated cells, ensuring alignment with real-world deployment conditions. (scenario #3) Proposal 9: For frequency-domain prediction, RAN4 to evaluate whether an AI/ML model trained at one site can generalize effectively to another site. Additional conditions must be identified to ensure consistency between training and inference phases for generalization purposes. (scenario #3) Proposal 10: RAN4 should evaluate how unknown cell predictions impact AI/ML model accuracy (e.g., false positives/negatives in cell identification). Additionally, RAN4 must define criteria for classifying cells as known or unknown in AI/ML training and inference phases, such as time thresholds or signal stability metrics. Finally, RAN4 should assess whether AI/ML models can reliably predict measurements for unknown cells using historical or cross-frequency correlation data Proposal 11: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures. (scenario #4) Proposal 12: RAN4 to evaluate the assumptions for single-link temporal domain prediction accuracy (single-beam operation) versus tracking multiple beams, including potential beam changes (best Tx/Rx pair) within the observation/prediction window. Proposal 13: RAN4 to investigate the extension of Scenario 4 to a multi-link temporal domain prediction across different cells and optimum TX/RX pairs Proposal 14: For the multi-link RSRP temporal-prediction extension of Scenario RAN4 to investigate the impact of cell dependent measurement errors (different SINRs and channel conditions) to the prediction accuracy of the AI/ML models (scenario #4) Proposal 15: RAN4 to evaluate whether an AI/ML model trained at one site can be effectively applied to another site under Scenario 4 and its multi-link extension. Additional conditions must be defined to ensure consistency between training and inference phases for generalization purposes - Proposal 16: RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. RAN4 should validate any RAN2 conclusions in the presence of realistic measurements errors. Proposal 17 : RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions. Proposal 18: Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, hardware compatibility, signal propagation characteristics) must be defined to ensure consistency between training and inference phases for robust generalization. Proposal 19: Define the ground truth of L3-RSRP as the measured RSRP from time t₀ to t₁ (observation window), under the condition that the channel characteristics during the prediction window (t₁ to t₂) remain identical to those in the observation window. The goal is to establish a reproducible and scenario-agnostic validation framework for AI/ML-based L3-RSRP prediction accuracy, particularly addressing limitations in scenarios where real-time UE measurements are unavailable or unreliable. - Proposed Methodology for Proposal 19: Ground truth definition - Channel-Consistency The channel conditions (e.g., large-scale fading, Doppler spread, interference levels, and multipath profiles) must remain unchanged between the observation window (t₀–t₁) and the prediction window (t₁–t₂). This ensures that the RSRP measured in the observation window (t₀–t₁) accurately reflects the conditions under which predictions are validated in t₁–t₂. - Measurement Protocol: - During the observation window (t₀–t₁), RSRP is measured continuously using UE or test equipment (TE) under controlled channel emulation. - In the prediction window (t₁–t₂), the AI/ML model’s predicted RSRP is compared against the pre-measured ground truth from t₀–t₁, assuming identical channel conditions. - Emulation and Reproducibility: - A channel emulator must replicate time-invariant channel parameters (e.g., path loss, shadowing, delay spread) across both windows. - For FR1, consistency in large-scale fading (e.g., path loss models) is prioritized. For FR2, beam-specific parameters (e.g., beam alignment, polarization) must also remain static. - Validation requires a correlation coefficient ≥ 0.95 between channel realizations in t₀–t₁ and t₁–t₂ to confirm stationarity. - Temporal Granularity: - The duration of t₀–t₁ and t₁–t₂ is defined based on UE mobility: - Low mobility (e.g., pedestrian): 20–50 ms windows. - High mobility (e.g., vehicular): 5–10 ms windows to minimize Doppler-induced variations. - Proposal 20: RAN4 to consider following steps for defining RSRP relative accuracy requirements: - Step 1: Prioritize Absolute RSRP Accuracy - Define absolute RSRP accuracy as the primary KPI for initial AI/ML model validation. - Metrics include: - Mean Absolute Error (MAE) between predicted and ground truth RSRP. - Tolerance thresholds (e.g., ±3 dB for FR1, ±4 dB for FR2). - Scope: Apply to current RAN2-supported scenarios (single-cell output). - Step 2: Monitor RAN2 Progress on Multi-Cell Output Scenarios - RAN4 to collaborate with RAN2 to investigate AI/ML models that predict RSRP for multiple cells(e.g., neighbor cell lists). - Identify use cases where relative RSRP accuracy becomes critical (e.g., handover optimization, carrier aggregation). - Step 3: Define Relative RSRP Accuracy Requirements (Conditional) - If RAN4/RAN2 standardizes multi-cell output prediction scenarios, RAN4 will: - Develop relative RSRP accuracy metrics, such as: - Rank order accuracy (correctly ranking cells by predicted RSRP). - Differential error thresholds (e.g., ±1 dB tolerance for relative differences between top-K cells). - Align test methodologies with multi-cell prediction dynamics (e.g., beam/cell correlation, interference coupling). - Proposal 21: RAN4 to clarify the exact meaning of the correlation coefficient between measured and predicted frequency layers, and how it relates to multi-frequency channel generation - Proposal 22: RAN4 to investigate the key features not addressed in the RAN2 study in order to determine overall feasibility, as outlined in the table Proposal 23: RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility. |

## Open issues summary

In this topic different companies’ proposals about the RRM requirements for RRM Measurements prediction use case are summarized.

### Issue 2-1: Relative RSRP accuracy definition

- Background:

Following agreement was reached in RAN4#114bis meeting:

Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

- Proposals and Observations:

  - Proposal 1 (Nokia): For intra-frequency L3-RSRP relative accuracy definition, RAN4 to cover comparison between two RSRP levels from the same cell or different cells.

  - Proposal 2 (Nokia): When defining the relative accuracy for intra-frequency predicted L3-RSRP, RAN4 to consider both of the following cases: Comparison between two predicted values, and comparison between one predicted value and one measured value..

  - Proposal 3 (Nokia): : RAN4 consider the following definition of accuracy of relative in intra-frequency; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP level 1 – reported RSRP level 2) – (ground truth of RSRP level 1 – ground truth of RSRP of level 2), where level 1 and level 2 may belong to the same or different cells on the same frequency. [Note: Reported RSRP can be either measured or predicted]

  - Proposal 4 (Nokia): RAN4 to extend the definition of inter-frequency predicted relative L3-RSRP accuracy to include comparisons between a predicted value and a measured value, regardless of whether one is used as the input for the other.

  - Proposal 5 (Nokia): For inter-frequency scenarios, RAN4 consider the following definition relative accuracy; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported  RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2. [Note: Reported RSRP can be either measured or predicted]

  - Proposal 6 (vivo): For intra-frequency L3-RSRP, regarding the definition- ‘Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.’, If interpreted in the following manner, the definitions under both approaches can be considered.

    - When the beam here refers to L1 beams, it corresponds to the scenario of predicting L3 (at point A or A1) from L1

    - When the beam here refers to L3 beams, it corresponds to the scenario of predicting RSRP at point E

  - Proposal 7 (vivo): For inter-frequency L3-RSRP, regarding the definition

‘accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].’,

      - For the case that the reported RSRP of Cell2 is predicted, it needs to firstly clarify which scenario this actually corresponds to

      - For the cases that the reported RSRP of Cell2 is measured and is a combination of both predicted and measured samples, both of these two cases can be considered

  - Proposal 8 (Huawei): For inter-frequency L3-RSRP,

    - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

    - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2,

      - if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, or

      - if cell 1 is the target cell based on intra-cell prediction in temporal domain

  - For intra-frequency L3-RSRP,

    - The below definition of beam-level relative predicted accuracy of beam-level L3 RSRP is needed if sub-use cases of L3 filtered beam-level measurement predication are considered in RAN4 in R19, otherwise the definition can be deprioritized.

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

  - Proposal 9 (Ericsson): Reformulate the term “Accuracy of predicted relative L3-RSRP” in the agreements reached in RAN4#114bis to “Relative accuracy of  predicted L3-RSRP measurements”.

  - Proposal 10 (Ericsson): Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), if cell 2 is measured cell to infer measurement for cell 1 (target cell) in the same carrier frequency.

  - Proposal 11 (Ericsson): Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different carrier frequency than cell 2 but in the same FR as cell 2 and cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2.

  - Proposal 12 (Ericsson): Definition for relative accuracy of predicted L3-RSRP measurements in intra-frequency scenarios where beam 1 and beam 2 are from the same cell is not needed.

  - Proposal 13 (Ericsson): Relative accuracy of predicted L3-RSRP measurements for temporal prediction is defined as:

    - Relative accuracy of predicted L3-RSRP measurements = (reported predicted RSRP1 of a cell/beam at time T1 – reported RSRP2 of the same cell/beam at time T2) – (ground truth of RSRP1 of a cell/beam at time T1 – ground truth of RSRP2 of the same cell/beam at time T2).

    - Reported RSRP2 can be predicted or measured.

    - T1 and T2 refer to time instances for temporal domain prediction.

  - Proposal 14 (CMCC): for accuracy of predicted relative L3-RSRP for inter-frequency, it is proposed that the reported RSRP of cell2 can be predicted or measured

  - Proposal 15 (Qualcomm): RAN4 to update the definition of relative L3-RSRP agreed in RAN4#114bis as follows:

    - For intra-frequency L3-RSRP,

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

        - Whether and how to define the requirements on the accuracy of predicted relative beam level L3-RSRP, when the two beams are from the same cell, should be discussed further during the work item phase.

        - Note that whether it becomes the same as the accuracy of predicted relative beam level L3-RSRP would depend on how the definition of the ground truth is established.

    - For inter-frequency L3-RSRP,

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2 or another cell on a different frequency than cell 2 but in the same FR as cell 2.

        - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition.

        - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples.

    - The applicability of the relative RSRP accuracy definition should be confined within the same frequency range

  - Proposal 16 (NTT DOCOMO): Predicted samples are applied for the reported RSRP of cell 2 to keep relative RSRP accuracy meaningful.

  - Proposal 17 (NTT DOCOMO): No need to consider the combination of predicted and measured samples from RAN4 accuracy requirement perspective.

  - Proposal 18 (ZTE): For temproal domain cell level prediction, no need to consider predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell.

  - Proposal 19 (ZTE): If RAN4 consider the relative accuracy for spatial domian or L3 beam level prediction, predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell is needed.

  - Proposal 20 (ZTE): For the case that cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, the reported RSRP of cell2 should be measured results.

  - Proposal 21 (OPPO): For the definition of relative accuracy for intra-frequency measurement, not to consider RSRP difference between different beams from the same cell.

  - Proposal 22 (OPPO): For the definition of relative accuracy for inter-frequency measurement, not to consider RSRP difference between predicted results and measured results, i.e., only consider the difference between two predicted results

  - Proposal 23 (LG Electronics): The beam-level relative L3-RSRP accuracy requirement is not necessary for intra-frequency L3-RSRP.

  - Proposal 24 (LG Electronics): Regarding inter-frequency L3-RSRP, for the predicted relative RSRP accuracy requirement, the reported RSRP of cell2 should be predicted value.

  - Proposal 25 (Tejas): RAN4 to clarify the difference between ground truth RSRP and reported RSRP

  - Proposal 26 (Tejas): RAN4 should explore potential enhancements in inter-frequency measurement prediction by combining with temporal domain prediction.

  - Proposal 27 (Xiaomi): For intra-frequency, the definition of the accuracy of predicted relative L3-RSRP shall be based on different UE capabilities:

    - If UE support to predict for multiple cells:

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency

    - If UE doesn’t support to predict for multiple cells:

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported measured RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency

  - Proposal 28 (Xiaomi): Accuracy of predicted relative L3-RSRP for different beams of the same cell is not needed.

  - Proposal 29 (Xiaomi): For inter-frequency, if UE support to predict for multiple cells, the relative RSRP can be compared between two different predicted RSRPs. Otherwise, the relative RSRP can be compared between one predicted RSRP and one measured RSRP from two different cells.

  - Proposal 30 (CATT): RAN4 to capture the relative RSRP prediction definition for intra-frequency beam L3-RSRP in the TR as one of study outcomes.

  - Proposal 31 (Apple): Define Relative RSRP Accuracy Requirements (Conditional)

    - If RAN4/RAN2 standardizes multi-cell output prediction scenarios, RAN4 will:

      - Develop relative RSRP accuracy metrics, such as:

        - Rank order accuracy (correctly ranking cells by predicted RSRP).

        - Differential error thresholds (e.g., ±1 dB tolerance for relative differences between top-K cells).

      - Align test methodologies with multi-cell prediction dynamics (e.g., beam/cell correlation, interference coupling).

- Candidate options:

For intra-frequency case, Relative accuracy of predicted L3-RSRP =:

  - Option 1a:   (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

  - Option 1b:  (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

    - Whether and how to define the requirements on the accuracy of predicted relative beam level L3-RSRP, when the two beams are from the same cell, should be discussed further during the work item phase.

    - Note that whether it becomes the same as the accuracy of predicted relative beam level L3-RSRP would depend on how the definition of the ground truth is established.

  - Option 2: (reported predicted L3-RSRP level 1 – reported RSRP level 2) – (ground truth of RSRP level 1 – ground truth of RSRP of level 2), where level 1 and level 2 may belong to the same or different cells on the same frequency. [Note: Reported RSRP can be either measured or predicted]

  - Option 3: (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.’, If interpreted in the following manner, the definitions under both approaches can be considered

    - When the beam here refers to L1 beams, it corresponds to the scenario of predicting L3 (at point A or A1) from L1

    - When the beam here refers to L3 beams, it corresponds to the scenario of predicting RSRP at point E

  - Option 4: (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), if cell 2 is measured cell to infer measurement for cell 1 (target cell) in the same carrier frequency.

  - Option 5a (If UE supports to predict for multiple cells): (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency

  - Option 5b (If UE doesn’t support to predict for multiple cells): (reported predicted L3-RSRP of cell 1 – reported measured RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency

- For inter-frequency case, Relative accuracy of predicted L3-RSRP =:

  - Option 1a: = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

  - Option 1b: Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2 or another cell on a different frequency than cell 2 but in the same FR as cell 2.

    - if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, or

    - if cell 1 is the target cell based on intra-cell prediction in temporal domain

    - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition.

    - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples.

  - Option 2: (reported predicted L3-RSRP of cell 1 – reported  RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2. [Note: Reported RSRP can be either measured or predicted]

  - Option 3: reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].

    - For the case that the reported RSRP of Cell2 is predicted, it needs to firstly clarify which scenario this actually corresponds to

    - For the cases that the reported RSRP of Cell2 is measured and is a combination of both predicted and measured samples, both of these two cases can be considered

  - Option 4: (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different carrier frequency than cell 2 but in the same FR as cell 2 and cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2

  - Option 5: For inter-frequency, if UE support to predict for multiple cells, the relative RSRP can be compared between two different predicted RSRPs. Otherwise, the relative RSRP can be compared between one predicted RSRP and one measured RSRP from two different cells.

- For temporal prediction case, Relative accuracy of predicted L3-RSRP =:

Option 1: Relative accuracy of predicted L3-RSRP measurements for temporal prediction is defined as:

- Relative accuracy of predicted L3-RSRP measurements = (reported predicted RSRP1 of a cell/beam at time T1 – reported RSRP2 of the same cell/beam at time T2) – (ground truth of RSRP1 of a cell/beam at time T1 – ground truth of RSRP2 of the same cell/beam at time T2).

    - Reported RSRP2 can be predicted or measured.

    - T1 and T2 refer to time instances for temporal domain prediction.

- Recommended WF

- Discuss above listed Candidate Options separately for intra-freq and inter-freq cases.

### Issue 2-2: Accuracy requirements for the predicted RSRPs

- Proposals and Observations:

  - Proposal 1 (Ericsson): Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes.

  - Proposal 2 (Nokia): For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

  - Proposal 3 (ZTE): RAN4 should discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

- Recommended WF

- Discuss above listed proposals.

### Issue 2-3: Ground truth definition for RSRP Accuracy for FR1

- Background:

Following agreement was reached in RAN4#114bis meeting:

Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Furthermore, the following agreement was reached in RAN4#114 meeting:

- Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

- Proposals and Observations:

  - Proposal 1 (Nokia): Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

  - Proposal 2 (Huawei): Proposal 12: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions.

    - how to avoid UE cheating can wait the progress in R19 AI interface beam management test.

  - Proposal 3 (Qualcomm): RAN4 to consider the following options for the definition of ground truth of L3-RSRP for the predicted absolute L3-RSRP accuracy (reported predicted L3-RSRP – ground truth of L3-RSRP). If the down selection needs to be made during the study item phase, RAN4 should consider Option 2-A, Option 2-B, and Option 1 in that order of priority. Note that the possible solutions and potential issues for each option are included within the respective options as well.

    - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate. The value of the margin needs a further study, and it can vary for different measurement configurations, etc.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

    - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition as the prediction-based L3-RSRP report (UE-reported L3-RSRP with noise)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms.

        - To avoid the UE from cheating the test, TE can either create another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction or run two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report.

        - This definition is the closest to the way the UE evaluates and monitors AI/ML performance based on the LCM framework.

    - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough.

        - Another round of tests would be needed for the L3-RSRP measurement report

    - RAN4 to strive to not differentiate the definition of the ground truth between FR1 and FR2.

  - Proposal 3 (OPPO): Use Alt2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1

  - Proposal 4 (LG Electronics): Alternative 2 is more reasonable and feasible method for ground truth definition for RSRP accuracy for FR1.

  - Proposal 5 (Xiaomi): If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports. Test Equipment (TE) can employ multiple anti-cheating strategies such as:

    - Multi-instance channel scrambling: Randomizing the order of predefined channel sequences.

    - Insertion of redundant channels: Introducing randomly generated non-functional channels.

    - Randomized temporal offset: Reconfiguring channels at an unknown time offset within a specified

  - Proposal 6 (Apple): Define the ground truth of L3-RSRP as the measured RSRP from time t₀ to t₁ (observation window), under the condition that the channel characteristics during the prediction window (t₁ to t₂) remain identical to those in the observation window. The goal is to establish a reproducible and scenario-agnostic validation framework for AI/ML-based L3-RSRP prediction accuracy, particularly addressing limitations in scenarios where real-time UE measurements are unavailable or unreliable

  - Proposal 7 (vivo): Regarding the Ground Truth definition for RSRP accuracy, the analyses of Alt 1 and Alt 2 are shown as below

|  | Issues | Impact to Alternative 1 | Impact to Alternative 2 |
| --- | --- | --- | --- |
| 1. | The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction | No with AWGN channel is assumed: If the AWGN channel is considered, it is assumed that the interval duration between time instances in both the observation and prediction windows are aligned, and their corresponding starting points are also aligned. The feasibility lies in the fact that both the measurement and prediction configurations can be adjusted. The timing information can be known to the TE by controlling both the measurement configuration and prediction timing. YES with fading channel is assumed: Under a fading channel, due to dynamic changes in the channel environment, obtaining ground truth may involve significant inaccuracies. | No with certain conditions: This issue emphasizes whether timing information can be obtained on the TE (Transmission Entity) side. Although the exact execution and completion time points of measurements are known on the UE side, if UE reporting is considered, the ways for the TE to be aware of the timing information include: Same as the analysis for Alt 1 (AWGN case) on the left The measurement results reported by the UE include timestamp information indicating when the measurements were performed |
| 2. | Impact of L3 filtering | YES, in certain scenarios: The filtering coefficient can be network-configured and thus known to the TE (at least for the case where L1 predicts L1 and then calculates L3). However, for the case where L1 directly predicts L3, the L3 filter is embedded in the model, and the consolidation/filtering process is treated as a black box—meaning the TE cannot know the filtering details. For L1 predicts L1 → then calculates L3: The filtering coefficient is a specific value configured by the network and can be known to the TE. For L1 directly predicts L3: The L3 filter is integrated into the model, and the consolidation/filtering process is black box, so the TE has no visibility into it. | YES, in certain scenarios: Similar to Alt 1, for the case where L1 directly predicts L3, the L3 filter is embedded in the model, and the consolidation/filtering process is treated as a black box. During UE measurements, the filtering coefficient may be based on network configuration. However, during prediction, the process may depend on UE implementation, leading to potential inconsistencies between: The filtering coefficient and the specific process used for the obtained measured value, And the actual model implementation (which remains opaque to the TE). This discrepancy could impact the alignment between measurement and prediction behaviors. |
| 3. | Number of samples of L1 filtering | L1 filtering and the number of samples for filtering are not known by the TE and entirely depend on UE implementation. | NO |
| 4. | Timing window, if it exists, where UE can’t measure the ground truth in Alt2 | NO | In Temporal Case B, ground truth may be missing if measurements are skipped. How to obtain the true measurement values at the corresponding moments requires further analysis. |
| 5. | Channel condition | Under fading channels, due to the dynamic changes in the channel environment, the acquisition of ground truth may be highly inaccurate. | NO |
| 6. | Avoid UE cheat in the test | NO | No feasible solution has been found yet, even in the discussion of the R19 AI interface beam management test. |
| 7. | S(I)NR variations in the test at different time instances (for Alt2) | NO | No with certain conditions the signal environment corresponding to the measured value is the signal environment at the moment of the predicted time instance, and there is no influence caused by changes in channel conditions or excessively low channel conditions |

  - Proposal 8 (CMCC): Consider Table below to provide the comparison between Alternative 1 and Alternative 2 for ground truth definition for FR1

|  | Alternative 1 (Ground truth for FR1 will be based on the transmitted or reception power) | Alternative 2 (Ground truth for FR1 will be based on the reported measurement value under certain conditions) |
| --- | --- | --- |
| Impact of L3 filtering | If L3 filtering is considered, how to perform L3 filtering is based on L3 filtering formula with filtering coefficient configured, and TE has no such information | N/A |
| Impact of L1 filtering | How to perform L1 filtering, e,g, the number of samples, whether sliding or non-sliding filtering in use, is up to UE implementation, and TE has no such information | N/A |
| Timing window | N/A | For Case B (target goal is to reduce measurement overhead), the time instance in prediction window will not be measured, which means UE cannot measure and report the ground truth. |
| Channel condition | N/A | For fading channel, the channel condition at T1 to get ground truth and the channel condition at T2 to predict is different, which result in mismatch |
| UE cheating issue | N/A | Since both the ground truth and predicted value are reported by UE, there may be cheating issue |

  - Proposal 9 (MediaTek): During the SI, RAN4 not to do down-selection on the two Alternatives of ground truth definition. No matter which alternative finally adopted, the impact of the instantaneous fluctuation at different occasions due to fast fading shall be considered.

- Candidate Options:

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

  - Option 1: RAN4 should consider Alt 1

  - Option 2: RAN4 should consider Alt 2

  - Option 3: RAN4 should not prioritize any alternative during SI phase. Discuss Proposal 7 and 8 to provide analysis on potential issue and possible solution of the two Alternatives.

- Recommended WF

- Discuss above listed Candidate Options

### Issue 2-4: Impacts of measurement error on prediction accuracy

- Background:

The following agreement was reached in RAN4#114 meeting:

Agreement: For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

Furthermore, following WF was suggested in RAN4#114bis meeting:

WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

- Proposals and Observations:

  - Proposal 1 (vivo): On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline:

    - For BB error modeling:

      - Link-level simulation approach is considered

    - For RF error,

      - For error modeling, the following two modelings can be the candidates.

        - Alt 1: Uniform distribution

        - Alt 2: Truncated Gaussian distribution

    - RF error application, each of Tx beam is independent

  - Proposal 2 (Ericsson): Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements.

    - RF error in dB for FR1 is characterized by Gaussian distribution with 〖μ=0,σ〗^2  = 2.5 truncated at ±2.5.

    - RF error in dB for FR2 is characterized by Gaussian distribution with 〖μ=0,σ〗^2  = 4 truncated at ±4.

  - Proposal 3 (Nokia): In Study Item, RAN4 to collect analysis on potential modeling methods and at least capture the collected analysis in the TR.

  - Proposal 4 (Nokia): RAN4 to capture the following error modeling principles in the TR:

    - RF and baseband measurement error modeling may be considered.

    - A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband.

    - Alternative modeling approaches are not precluded and can be explored in the Work Item phase.

  - Proposal 5 (Qualcomm): RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM, e.g.

    - Whether and how to accommodate the impact of measurement inaccuracies for a cluster-based approach.

    - Whether and how to model the impact of imperfect RF calibration for inter-frequency RRM prediction.

    - Whether different aspects need to be considered for Case A and Case B in intra-cell/intra-frequency RRM prediction.

    - Note that the effective differences between Case A and Case B, from a UE measurement and prediction perspective, might be limited to the measurement rate within the observation window and the prediction range into the future. Case A and Case B may have different impacts on RRM requirements, particularly about measurement/scheduling restrictions.

  - Proposal 6 (OPPO): Consider the following options to model the measurement error:

    - Option 1: For RF error, use the truncated Gaussian distribution under ±X dB; for baseband error, use the link level simulation assumptions to generate L3-RSRP difference as baseband error.

    - Option 2: For RF and baseband error, use the truncated Gaussian distribution under ±X dB and ±Y dB, respectively.

    - Option 3: For the sum of RF and baseband error, use the truncated Gaussian distribution under ±X dB.

  - Proposal 7 (LG Electronics): RAN4 to consider only Gaussian error model regardless what RF error model or BB error model is.

  - Proposal 8 (Xiaomi): RAN4 to study the factor and impact of measurement error on RRM prediction.

  - Proposal 9 (MediaTek): For AI mobility, in FR1, measurement error should be added as:

    - BB error:

      - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR.

      - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement.

    - RF error:

      - Modelled as Gaussian distribution under ±3 dB RF error (〖μ=0,σ〗^2=3).

      - A random RF error is generated for all the beams of the same frequency layer per SMTC.

  - Proposal 10 (Apple): RAN4 to investigate the impact of measurement errors (RF and baseband) in AI/ML model input data on the accuracy of predicted measurements. Additionally, RAN4 should investigate whether RAN2 simulation results and conclusions regarding achievable prediction accuracy remain valid under such measurement errors for scenario 2.

  - Proposal 11 (Apple): For the multi-link RSRP temporal-prediction extension of Scenario RAN4 to investigate the impact of cell dependent measurement errors (different SINRs and channel conditions) to the prediction accuracy of the AI/ML models (scenario #4)

  - Proposal 12 (Apple): RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. RAN4 should validate any RAN2 conclusions in the presence of realistic measurements errors

  - Proposal 13 (Apple): RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions.

- Recommended WF

  - RAN4 to capture following factors to be considered for error models to consider the impact of measurement errors on prediction accuracy

    - RF and baseband measurement error modeling may be considered.

    - A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband.

    - Alternative modeling approaches are not precluded and can be explored in the Work Item phase.

### Issue 2-5: Impacts on beam/cell level measurements

- Background:

The following agreement was achieved in RAN4#112bis meeting:

Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Furthermore, following agreement was reached in RAN4#114bis meeting:

Agreement: - For Rel. 19 study: - RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

The figure below from TS 38.300 (Figure 9.2.4-1) should be considered as a reference for this discussion.

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

  - Proposal 2 (vivo): For the following conclusion made in RAN4#114bis, RAN4 to update the wording as:

    - For Rel-19 study, RAN4 to not consider L1 beam-level (Point A1) measurement prediction for UE sided model when L1 beam-level (Point A1) predicted results as the report content

  - Proposal 3 (ZTE): RAN4 should study the impacts on requirements for L3 cell-level prediction first and the prediction accuracy for L3 beam-level prediction can use the conclusion of L3 cell level as baseline.

  - Proposal 4 (Tejas): RAN4 should study the mobility enhancements based on L3 cell-level measurement predictions in conjunction with the relaxation of L1 beam-level measurements.

  - Proposal 5 (CATT): RAN4 to deprioritize the study on impact of L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

  - Proposal 6 (Apple): RAN4 to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E)

- Candidate Options:

  - L3 beam-level (Point E) measurement:

    - Option 1: Do not consider L3 beam-level (Point E)

    - Option 2: Consider L3 beam-level (Point E) measurement prediction as second priority

    - Option 3: RAN4 should consider studying the impacts on requirements for L3 beam-level (Point E)

  - L1 beam-level (Point A1):

    - Option 1: Update the agreement reached in RAN4#114bis meeting as:

      - For Rel-19 study, RAN4 to not consider L1 beam-level (Point A1) measurement prediction for UE sided model when L1 beam-level (Point A1) predicted results as the report content

- Recommended WF:

  - Discuss above listed options for L3 beam-level (Point E) and L1 beam-level (Point A1) separately.

### Issue 2-6: Impacts from observation and prediction windows

- Background:

The following agreement was reached in RAN4#114 meeting:

Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 to not consider number of L1/L3 measurements performed during OW and its impacts on the prediction accuracy

  - Proposal 2 (Nokia): For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed.

  - Proposal 3 (Nokia): If skipping pattern information is not shared by the UE, RAN4 to only consider MRRT, OW and PW to UE as test configuration and set up a requirement on prediction accuracy.

  - Proposal 4 (Nokia): If skipping pattern information is shared by the UE, TE should select the supported skipping pattern related to the MRRT to test. The test configuration should then include skipping pattern, OW and PW.

  - Proposal 5 (vivo): For inter-frequency prediction, the OW length can be considered as one of the side conditions

  - Proposal 6 (Huawei): For intra-frequency temporal domain prediction case A, observation window length (or called as prediction evaluation period) and prediction window length may need to be specified.

  - Proposal 7 (Ericsson): RAN4 to study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B.

  - Proposal 8 (OPPO): RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

  - Proposal 9 (OPPO): RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

  - Proposal 10 (Xiaomi): Observation window and prediction window defined in RAN2 will have RRM impact:

    - Observation window will have impact on measurement duration

    - Prediction window will have impact on scheduling restriction

  - Proposal 11 (Xiaomi): For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

  - Proposal 12 (CATT): The reported RSRP of cell 2 can be measured at least. RAN4 further discuss it in WI phase after RAN2 finish the design on observation window and prediction window configuration.

  - Proposal 13 (CATT): RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed.

  - Proposal 14 (Apple): Investigate the translation of RAN2 HO KPIs into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR.

- Candidate Options:

  - Option 1: RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed

  - Option 2: For intra-frequency intra-cell prediction scenarios, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed.

  - Option 3: In addition to OW, PW, MRRT and UE speed, RAN to also study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B

- Recommended WF:

Discuss above listed candidate options.

### Issue 2-7: Inter-frequency prediction scenario

- Background:

The following agreement was reached in RAN4#114bis meeting:

Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

- Proposals and Observations:

  - Proposal 1 (Nokia): For inter-frequency prediction scenario, RAN2 has no sufficient clarity on the definition of correlation coefficient; therefore, RAN4 to wait for the progress in RAN2.

  - Proposal 2 (Nokia): For inter-frequency prediction scenario, in a cluster approach, RRM measurement prediction performance can be enhanced when measurement from a group of cells in one carrier frequency is used by the UE as an input. Therefore, RAN4 should consider the impact of cluster approach.

  - Proposal 3 (vivo): As the key element affecting RSRP prediction accuracy for inter-frequency prediction, correlation coefficient need to consider it when defining metrics. Regarding EPRE difference, as well as frequency distance and frequency separation, hese are coupled with the correlation coefficient and no need to splitting them into more detailed factors for discussion

  - Proposal 4 (vivo): For the cluster-based approach, the criteria for selecting cells can influence inter-frequency prediction accuracy. This factor could also be considered as a potential aspect

  - Proposal 5 (Huawei): For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy is to be specified under certain conditions,

    - channel correlation coefficient

    - FFS on cluster/single cell approach, depending on whether explicit indication and corresponding UE capability are specified in RAN2.

  - Proposal 6 (Huawei): cluster approach or single cell approach would impact on the value of prediction accuracy.

  - Proposal 7 (CMCC): for cluster approach, it is proposed to follow RAN2 agreements that model takes measurements from more than one cell as inputs.

  - Proposal 8 (Qualcomm): RAN4 to not consider EPRE difference as one of the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario.

  - Proposal 9 (OPPO): RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

  - Proposal 10 (Tejas): It is proposed that RAN4 not study the impact of cluster-based and non-cluster-based inter-frequency prediction scenarios.

  - Proposal 11 (Tejas): No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario.

  - Proposal 12 (Xiaomi): For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

  - Proposal 13 (MediaTek): For inter-frequency prediction scenario, it is not feasible to study the impact of correlation coefficient unless RAN4 define a common dataset.

  - Proposal 14 (Apple):RAN2’s assumption of collocated cells for frequency domain prediction may represent an unrealistic scenario. RAN4 to investigate frequency domain prediction methodologies for non-collocated cells, ensuring alignment with real-world deployment conditions. (scenario #3)

  - Proposal 15 (Apple): RAN4 to clarify the exact meaning of the correlation coefficient between measured and predicted frequency layers, and how it relates to multi-frequency channel generation (7.6.5 in TR 38.901)

- Candidate Options:

- Regarding EPRE difference

  - Option 1: RAN4 to not consider EPRE difference as one of the factors

  - Option 2: RAN4 to consider EPRE difference as one of the factors

- Regarding Correlation Coefficient

  - Option 1: RAN2 has no sufficient clarity on the definition of correlation coefficient; therefore, RAN4 to wait for the progress in RAN2

  - Option 2: RAN4 to consider channel correlation coefficient for the definition of performance metric for inter-frequency scenarios

  - Option 3: It is not feasible to study the impact of correlation coefficient unless RAN4 define a common dataset.

  - Option 4: To consider the framework of multi-frequency simulation in section 7.6.5 in TR 38.901 for inter frequency evaluations and drop the correlation coefficient

- Regarding Cluster Approach

  - Option 1: RAN4 to consider the impact of cluster approach

  - Option 2: RAN4 to consider the impact of cluster approach

  - Option 3: RAN4 to consider the impact of cluster approach depending on whether explicit indication and corresponding UE capability are specified in RAN2

  - Option 4: Extend the clustering approach to support multi-output cell prediction per frequency layer, enabling the possibility of skipping measurement gaps

- Impact on CSSF:

  - RAN4 to consider the impact on CSSF calculation

  - RAN4 to not consider the impact on CSSF calculation

- Non-collocated Scenario:

  - RAN4 to consider non-collocated scenarios for inter-frequency inter-cell prediction

- Recommended WF:

Discuss above listed candidate options.

### Issue 2-8: Impact on Core requirements

- Proposals and Observations:

  - Proposal 1 (Huawei): The processing time of inference function may need to be considered in measurement reporting delay.

  - Proposal 2 (CMCC): for RRM measurement prediction, the measurement delay requirements need to be defined/updated

    - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy

    - For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements

  - Proposal 3 (ZTE): Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements. And RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility

  - Proposal 4 (OPPO): RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

  - Proposal 5 (OPPO): RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

  - Proposal 6 (OPPO): RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

  - Proposal 7 (MediaTek): Regarding measurement report delay requirements for AI mobility, leverage the conclusion on AI/ML BM as much as possible.  Not to discuss the impact on measurement report delay requirements for AI mobility in SI to avoid duplicated discussions.

  - Proposal 8 (MediaTek): Regarding the impact on measurement delay requirements for AI mobility Scenario 2, leverage the conclusion on AI/ML BM case 2 as much as possible. Not to discuss the impact on measurement delay requirements for AI mobility Scenario 2 in SI to avoid duplicated discussions.

  - Proposal 9 (MediaTek): For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

  - Proposal 10 (MediaTek): Scenario 4 has no impact on measurement delay requirements.

  - Proposal 11(MediaTek): For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown.

  - Proposal 12 (MediaTek): Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring.

  - Proposal 13 (Apple): Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, hardware compatibility, signal propagation characteristics) must be defined to ensure consistency between training and inference phases for robust generalization.

- Candidate Options:

  - Option 1: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A and/or B

  - Option 2: RAN4 to discuss the measurement period requirements for inter-frequency case

  - Option 3: RAN4 to discuss the measurement period requirements for both inter-frequency case and intra-frequency temporal case A and/or B

  - Option 4: RAN4 should consider inference delay in measurement reporting delay.

  - Option 5: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A and B considering the impact of OW and/or PW length and/or skipped measurement instances.

  - Option 6: RAN4 should wait for further progress in RAN2/AIML BM WI before starting discussion on core requirements in RAN4

  - Option 7: RAN4 to investigate the generalization capability of AI/ML models across cells operating under distinct network conditions, and define critical parameters needed to ensure consistency between training and inference phases

- Recommended WF

- Discuss above listed options

### Issue 2-9: LCM related requirements

- Proposals and Observations:

  - Proposal 1 (OPPO): Wait for more progress from RAN1/2 AI mobility and RAN4 AI BM discussion for LCM related requirements.

  - Proposal (Apple): RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

- Recommended WF

  - LCM related discussions should be held in RAN4 potentially at a later stage. RAN2 needs to progress on this topic.

# Topic #3: RAN4 requirements for measurement event prediction

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505697 | OPPO | Proposal 1: For indirect prediction, if the report includes the predicted RSRPs, the requirements for measurement prediction can be reused. Proposal 2: For indirect prediction, RAN4 not to define delay requirements for predicted event triggered reporting. Proposal 3: For indirect prediction, wait for more progress from measurement prediction to discuss whether the predicted RSRP reporting delay is needed. Proposal 4: For indirect prediction, the following can be considered as possible KPI to evaluate the performance: - Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.) - Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges) - Alt 3: The handover failure rate per UE under certain time durations Proposal 5: For direct and indirect prediction, RAN4 to discuss whether and how to define new quantities or requirements based on further RAN2 progress. |
| R4-2505767 | Xiaomi | Observation 1: In legacy, RAN4 defined event triggered L3 reporting delay in core part. Proposal 1: Measurement event prediction may have impact on predicated event triggered L3 reporting. Observation 2: The time interval from measurement to prediction increase, the prediction accuracy decreases. The prediction accuracy of individual measurement results in a time window could be quite different. Proposal 2: For RSRP accuracy requirement for event triggered reporting, if there are multiple predicted RSRPs in one report, we propose to evaluate RSRP accuracies continuously over multiple time instances to enhance the performance of event reporting. Proposal 3: For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances. Observation 3: In RAN2, performance metric F1 score is defined for event prediction accuracy, which considered both precision and recall. Observation 4: In RAN2, a timing difference between predicated event and ground truth event is defined. Observation 5: In legacy RAN4, in test for verify event prediction accuracy, only precision = 90% is used and no timing difference between predicated event and ground truth event is considered. Proposal 4: RAN4 to only consider precision and not consider recall in event prediction accuracy KPI. |
| R4-2505951 | Apple | Proposal 1: RAN4 to discuss the modelling of measurement errors and their impact to evaluate AI/ML event prediction benefits compared to sample-and-hold and the impact to false detection of events (higher cost) Proposal 2: For indirect measurement event prediction, RAN4 to discuss the modeling and impact of measurement errors to the prediction accuracy (i.e., F1 score) Proposal 3: RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions. Proposal 4: Investigate the translation of RAN2 HO KPIs into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures. Proposal 5: Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, signal propagation characteristics, SINR) must be defined to ensure consistency between training and inference phases for robust generalization. Proposal 6: RAN4 should consider the following performance metrics for AI/ML-based measurement event prediction: - Baseline Metrics from L3-RSRP Prediction - Retain one or multiple existing performance metrics defined for AI/ML-based L3-RSRP prediction - Event-Specific Temporal Accuracy - Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP. - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2. - Combined or Down-Selected Metrics - The above metrics (L3-RSRP prediction accuracy + temporal gap) may be: - Jointly used for comprehensive performance assessment, or - Independently selected depending on use-case priorities. |
| R4-2505986 | CATT | Proposal 1: RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction, based on RAN2 agreement. Proposal 2: RAN4 to define delay requirement for indirect case and discuss the details in WI phase. Proposal 3: RAN4 to define accuracy requirement for indirect case and reuse the accuracy requirement for RRM measurement prediction, when the report includes the predicted RSRPs. No accuracy requirements need to be defined for each final result report. Proposal 4: Successful prediction rate can be used as a test metric for AI/ML model performance monitoring during a time period for both direct and indirect measurement event prediction. Proposal 5: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details in WI phase. |
| R4-2506508 | MediaTek Inc. | Observation 1: For measurement event prediction, it is too early to discuss the metrics to define requirements for in RAN4 as the reported quantities are quite open in RAN2. |
| R4-2506549 | vivo | Observation 1: In legacy, the time when the event condition is met must always precede the UE’s transmission of the measurement report, ensuring that the UE can confirm the event condition is satisfied during measurement. While for the measurement event prediction, the expected occurrence time of the measurement event corresponds to a future moment. It will be later than the UE’s transmission of the prediction report Proposal 1: RAN4 not to study the delay requirement for the predicted event triggered reporting Proposal 2: F1 score under certain fixed ETD conditions can be served as the performance metric for evaluating the accuracy of measurement event prediction. Proposal 3: If the report includes the predicted RSRPs corresponding to the predicted event occurrence time, RAN4 will define requirement for the predicted RSRPs |
| R4-2506565 | ZTE Corporation, Sanechips | Proposal 1: Measurement error should be considered into measurement event prediction, which will impact the prediction accuracy. Proposal 2: For measurement event prediction, RAN4 should at least consider scenarios 2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A). Proposal 3: For indirect case, if the report includes the predicted RSRPs, RAN4 should define requirements for the predicted RSRPs and the requirements for measurement prediction can be used as baseline. Proposal 4: RAN4 should consider whether and how to define prediction accuracy for event prediction use case based on RAN2’s progress. |
| R4-2506612 | CMCC | Proposal 1: for indirect measurement event prediction, if the report includes the predicted RSRPs, it is proposed to define requirements for the predicted RSRPs, and the conclusion on RRM measurement prediction can be reused. Proposal 2: for the final output of measurement event prediction (e.g. the probability of event occurrence for direct prediction or expected occurrence time of a event for indirect prediction), it is proposed to discuss whether and how to define RRM requirements. Proposal 3: Whether measurement error is considered for event prediction use case can be considered case by case. - For indirect measurement event prediction, the conclusion on how to handle the measurement error for RRM prediction mode can be reused. - For direct measurement event prediction, whether to consider the impact due to measurement error depends on the model input. - If the model input does not include measured RSRP, no need to consider measurement error. Otherwise, measurement error may have impact on prediction perfromance. |
| R4-2506737 | Huawei, HiSilicon | Proposal 1: If the starting point of legacy handover delay is reused (i.e, when UE receives HO CMD), the AI event prediction has no impact on the requirements of handover delay. Proposal 2: RAN4 to discuss the following options for indirect event prediction Option 1: Define accuracy for indirect event prediction Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD (time difference between the ground truth event and the predict event), under the condition that F1 score consistently is above a value. Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score, under the conditions of certain fixed ETD. More specifically, the qualified UE shall satisfy F1 score consistently is above a value, under the condition that event is predicted within certain ETD. Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied. Proposal 3: In indirect event prediction, if the MR includes the intermediate predicted RSRP corresponding to the predicted event occurrence time, the intermediate RSRP shall satisfy the measurement prediction accuracy. Proposal 4: RAN4 to discuss the following options for defining accuracy for direct event prediction: Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that F1 score is consistently above a value. Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1]. |
| R4-2507124 | NTT DOCOMO, INC. | Observation 1: The RRM requirements on predicted RSRPs have been discussed in the AIML air and RRM measurement prediction. Proposal 1: Need to define requirement for the predicted RSRPs by reusing outcomes from other AIML discussions. Proposal 2: RAN4 reuses relative RSRP requirement for the predicted RSRPs. |
| R4-2507433 | Nokia | Observation 1: The following scenarios have been considered in RAN2 for indirect measurement event prediction using temporal domain predictions: 1) Temporal domain Case A: The UE predicts a probable future measurement event, which may be used in mobility optimization decisions, but there is no measurement reduction. 2) Temporal domain Case B: The UE is predicting measurements for measurement reduction, which should not have significant impact on the mobility performance. Observation 2: For indirect measurement event prediction, the choice between temporal domain Case A and Case B may impose different performance requirements, necessitating further RAN4 evaluation. Proposal 1: RAN4 to study the requirements and performance metrics for indirect measurement event prediction temporal domain Case A and Case B. Observation 3: Using performance metric F1 score, the choice between periodic and event-based measurement event prediction significantly impacts the clarity of initial points and intervals for counter updates in F1 score, and final F1 score results. Observation 4: If there is more than one target cell candidate, it should be clarified if the prediction accuracy counters are to be evaluated for each target candidate cell individually, or if predicting the right target candidate cell is included in the F1 score calculation. Observation 5: Testing the UE with F1 score as the metric for A3 measurement event prediction is challenging due to significant testing overhead. Proposal 2: RAN4 to study a simpler metric for A3 measurement event prediction. Observation 6: For a measurement event prediction with time domain case A which is used in mobility optimization predicting A3 events ahead of the actual ground-truth trigger is often desirable. Proposal 3: For the time domain case A measurement event prediction, RAN4 to consider prediction of A3 event early enough a head of ground truth (actual event) as performance metric for true event prediction. Proposal 4: For the time domain case B measurement event prediction, RAN4 may consider a similar performance metric definition as that of temporal domain case A. Observation 7: Reporting predicted measurements alongside the predicted event provides greater operational flexibility, transparency, and reusability for the network. Observation 8: Similar accuracy definition and requirements have been considered for Even-based measurement reporting and periodic measurement in TS 38.133. Proposal 5: For event-based measurement reporting based on indirect event prediction, RAN4 to consider a similar accuracy definition as measurement prediction. |
| R4-2507668 | Ericsson | Observation 1: Reporting delay requirement defined for the legacy event triggered reporting may not hold true for the event prediction case. Proposal 1: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting for indirect event prediction use case. Proposal 2: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting for direct event prediction use case provided the predicted event triggered reporting based on direct event prediction is supported by RAN2 framework. Observation 2: Temporal domain prediction of RRM measurements is reused for indirect event prediction use cases. Observation 3: Accuracy of temporal domain prediction has an impact on the accuracy indirect event prediction use case. Proposal 3: RAN4 to consider same accuracy requirement for temporal prediction of RRM measurement and indirect event prediction use case. Proposal 4: Performance requirement for direct event prediction needs to be defined for event prediction provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility. |
| R4-2507802 | Qualcomm Incorporated | Proposal 1: RAN4 should clarify that 'delay' in the agreement made in RAN4#114bis refers to how far ahead of the actual event occurrence time the UE should be able to predict the event. - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [‘delay’ and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs Proposal 2: For the definition of ground truth of predicted event occurrence time accuracy (reported predicted event occurrence time – ground truth of the event occurrence time) in the time domain, the ground truth of the event occurrence time should be defined by the same principles as the ground truth of predicted L3-RSRP. If a down selection is made during the study item phase for the definition of ground truth of predicted L3-RSRP, the same principle should be inherited for the definition of ground truth of predicted event occurrence time. The options for the definition of ground truth of predicted L3-RSRP are listed below (options are duplicated below for reference from our companion paper submitted for AI 7.21.2.1): - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP) - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate. The value of the margin needs a further study, and it can vary for different measurement configurations, etc. - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc. - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition as the prediction-based L3-RSRP report (UE-reported L3-RSRP with noise) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms. - To avoid the UE from cheating the test, TE can either create another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction or run two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report. - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough. - Another round of tests would be needed for the L3-RSRP measurement report - RAN4 to strive to not differentiate the definition of the ground truth between FR1 and FR2. Observation 1: For the performance metric of event prediction in the L3-RSRP domain, one or both of the performance metrics in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%), which are covered by another agenda item, should be considered. Proposal 3: RAN4 to consider the following as one of the performance metrics for AI/ML-based measurement event prediction: - One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%). - The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP. - The above two metrics may jointly serve as a performance assessment metric for event prediction, or one can be down-selected. The choice of the metric and the details could further depend on the following aspects to be discussed and decided by RAN2: - Whether the AI model is for indirect or direct event prediction. - What metrics are to be reported to the serving cell. |

## Open issues summary

In this section, different companies’ proposals about the RRM requirements for AI/ML Mobility use cases are summarized.

Issue 3-1: Performance metric for event prediction use case

- Background:

The following agreement was reached inRAN4#114bis meeting:

Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

- Proposals and Observations:

  - Proposal 1 (Nokia):

    - For event-based measurement reporting based on indirect event prediction, RAN4 to consider a similar accuracy definition as measurement prediction.

    - RAN4 to study a simpler metric for A3 measurement event prediction

      - For the time domain case A measurement event prediction, RAN4 to consider prediction of A3 event early enough a head of ground truth (actual event) as performance metric for true event prediction.

    - For the time domain case B measurement event prediction, RAN4 may consider a similar performance metric definition as that of temporal domain case A.

  - Proposal 2 (Ericsson):

    - RAN4 to consider same accuracy requirement for temporal prediction of RRM measurement and indirect event prediction use case.

    - Performance requirement for direct event prediction needs to be defined for event prediction provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility

  - Proposal 3 (Xiaomi): RAN4 to only consider precision and not consider recall in event prediction accuracy KPI.

    - For RSRP accuracy requirement for event triggered reporting, if there are multiple predicted RSRPs in one report, we propose to evaluate RSRP accuracies continuously over multiple time instances to enhance the performance of event reporting.

    - For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances.

  - Proposal 4 (OPPO): For indirect prediction,

    - If the report includes the predicted RSRPs, the requirements for measurement prediction can be reused.

    - The following can be considered as possible KPI to evaluate the performance:

      - Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.)

      - Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges)

      - Alt 3: The handover failure rate per UE under certain time durations

  - Proposal 5 (Huawei):

    - In indirect event prediction, if the MR includes the intermediate predicted RSRP corresponding to the predicted event occurrence time, the intermediate RSRP shall satisfy the measurement prediction accuracy.

    - RAN4 to discuss the following options for defining accuracy for direct event prediction:

      - Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that F1 score is consistently above a value.

      - Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1].

    - RAN4 to discuss the following options for indirect event prediction

      - Option 1: Define accuracy for indirect event prediction

      - Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD (time difference between the ground truth event and the predict event), under the condition that F1 score consistently is above a value.

      - Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score, under the conditions of certain fixed ETD. More specifically, the qualified UE shall satisfy F1 score consistently is above a value, under the condition that event is predicted within certain ETD.

      - Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

  - Proposal 6 (CATT): Successful prediction rate can be used as a test metric for AI/ML model performance monitoring during a time period for both direct and indirect measurement event prediction.

    - RAN4 to define accuracy requirement for indirect case and reuse the accuracy requirement for RRM measurement prediction, when the report includes the predicted RSRPs. No accuracy requirements need to be defined for each final result report.

    - RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction, based on RAN2 agreement.

  - Proposal 7 (Qualcomm):

    - RAN4 to consider the following as one of the performance metrics for AI/ML-based measurement event prediction:

      - One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%).

      - The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP.

      - The above two metrics may jointly serve as a performance assessment metric for event prediction, or one can be down-selected. The choice of the metric and the details could further depend on the following aspects to be discussed and decided by RAN2:

        - Whether the AI model is for indirect or direct event prediction.

        - What metrics are to be reported to the serving cell.

  - Proposal 8 (CMCC):

    - For indirect measurement event prediction, if the report includes the predicted RSRPs, it is proposed to define requirements for the predicted RSRPs, and the conclusion on RRM measurement prediction can be reused.

  - Proposal 9 (Apple):

    - Investigate the translation of RAN2 HO KPIs into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures.

    - Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, signal propagation characteristics, SINR) must be defined to ensure consistency between training and inference phases for robust generalization.

    - RAN4 should consider the following performance metrics for AI/ML-based measurement event prediction:

      - Baseline Metrics from L3-RSRP Prediction

        - Retain one or multiple existing performance metrics defined for AI/ML-based L3-RSRP prediction

      - Event-Specific Temporal Accuracy

        - Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP.

        - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2.

      - Combined or Down-Selected Metrics

        - The above metrics (L3-RSRP prediction accuracy + temporal gap) may be:

        - Jointly used for comprehensive performance assessment, or

        - Independently selected depending on use-case priorities.

  - Proposal 10 (vivo):

    - F1 score under certain fixed ETD conditions can be served as the performance metric for evaluating the accuracy of measurement event prediction.

    - If the report includes the predicted RSRPs corresponding to the predicted event occurrence time, RAN4 will define requirement for the predicted RSRPs

- Candidate options:

- For indirect case:

  - Option 1: RAN4 to study the applicability of existing measurement event criteria and reporting requirements to event prediction use case

  - Option 2: RAN4 to define a new performance metric for event prediction

    - Option 2-1: RAN4 to study a simpler metric for A3 measurement event prediction

    - Option 2-2: If the MR includes the intermediate predicted RSRP corresponding to the predicted event occurrence time, the intermediate RSRP shall satisfy the measurement prediction accuracy

    - Option 2-3: define successful prediction rate as a test metric for AI/ML model performance monitoring

    - Option 2-4: RAN4 to consider the following as one of the performance metrics for AI/ML-based measurement event prediction:

      - One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%).

      - The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP.

    - Option 2-5: Investigate the translation of RAN2 HO KPIs into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study

    - Option 2-6: Use F1 score under certain fixed ETD conditions as a performance metric for evaluating the accuracy of measurement event prediction

    - Option 2-7: RAN4 should consider the following performance metrics for AI/ML-based measurement event prediction:

      - Baseline Metrics from L3-RSRP Prediction

        - Retain one or multiple existing performance metrics defined for AI/ML-based L3-RSRP prediction

      - Event-Specific Temporal Accuracy

        - Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP.

        - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2.

      - Combined or Down-Selected Metrics

        - The above metrics (L3-RSRP prediction accuracy + temporal gap) may be:

        - Jointly used for comprehensive performance assessment, or

        - Independently selected depending on use-case priorities.

    - Option 2-8: RAN4 to only consider precision and not consider recall in event prediction accuracy KPI.

      - For RSRP accuracy requirement for event triggered reporting, if there are multiple predicted RSRPs in one report, evaluate RSRP accuracies continuously over multiple time instances to enhance the performance of event reporting.

      - For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances.

    - Option 2-9: RAN4 to investigate the generalization capability of AI/ML models across cells operating under distinct network conditions, and define critical parameters needed to ensure consistency between training and inference phases

    - Option 3: RAN4 to consider same accuracy requirement for temporal prediction of RRM measurement and indirect event prediction use case

- For direct case:

  - Option 1: Performance requirement for direct event prediction needs to be defined for event prediction provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility.

- Recommended WF

- Discuss above listed Candidate Options.

Issue 3-2: Impact of measurement error for event prediction use case

- Proposals and Observations:

  - Proposal 1 (ZTE): Measurement error should be considered into measurement event prediction, which will impact the prediction accuracy.

  - Proposal 2 (CMCC): Whether measurement error is considered for event prediction use case can be considered case by case.

    - For indirect measurement event prediction, the conclusion on how to handle the measurement error for RRM prediction mode can be reused.

    - For direct measurement event prediction, whether to consider the impact due to measurement error depends on the model input.

    - If the model input does not include measured RSRP, no need to consider measurement error. Otherwise, measurement error may have impact on prediction performance.

  - Proposal 3 (Apple):

    - RAN4 to discuss the modelling of measurement errors and their impact to evaluate AI/ML event prediction benefits compared to sample-and-hold and the impact to false detection of events (higher cost)

    - For indirect measurement event prediction, RAN4 to discuss the modeling and impact of measurement errors to the prediction accuracy (i.e., F1 score)

    - RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions.

Candidate options:

  - Option 1: Measurement error to be considered for event prediction use case during WI phase. Error model can be similar to that of RRM measurement prediction use case.

  - Option 2: Measurement error to be considered for event prediction use case during WI phase. Error model may be different from the one used for RRM measurement prediction use case.

  - Option 3: Measurement error not to be considered for event prediction use case.

- Recommended WF

- Discuss above listed Candidate Options.

Issue 3-3: Impact on core requirements for event prediction use case

- Proposals and Observations:

  - Proposal 1 (Xiaomi): Measurement event prediction may have impact on predicated event triggered L3 reporting.

  - Proposal 2 (OPPO):

    - For indirect prediction, RAN4 not to define delay requirements for predicted event triggered reporting

    - For indirect prediction, if the report includes the predicted RSRPs, the requirements for measurement prediction can be reused. For indirect prediction, RAN4 to wait for more progress from measurement prediction to discuss whether the predicted RSRP reporting delay is needed.

    - For indirect prediction RAN4 to discuss whether and how to define new quantities or requirements based on further RAN2 progress.

  - Proposal 3 (CATT): With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details in WI phase.

    - RAN4 to define delay requirement for indirect case and discuss the details in WI phase.

  - Proposal 4 (Huawei): If the starting point of legacy handover delay is reused (i.e, when UE receives HO CMD), the AI event prediction has no impact on the requirements of handover delay

  - Proposal 5 (NTT Docomo):

    - Need to define requirement for the predicted RSRPs by reusing outcomes from other AIML discussions.

    - RAN4 reuses relative RSRP requirement for the predicted RSRPs.

  - Proposal 6 (vivo):

    - RAN4 not to study the delay requirement for the predicted event triggered reporting

  - Proposal 7 (ZTE):

    - For indirect case, if the report includes the predicted RSRPs, RAN4 should define requirements for the predicted RSRPs and the requirements for measurement prediction can be used as baseline.

    - RAN4 should consider whether and how to define prediction accuracy for event prediction use case based on RAN2’s progress.

  - Proposal 8 (Ericsson):

    - RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting for indirect event prediction use case

    - RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting for direct event prediction use case provided the predicted event triggered reporting based on direct event prediction is supported by RAN2 framework

  - Proposal 9 (Qualcomm):

    - RAN4 should clarify that 'delay' in the agreement made in RAN4#114bis refers to how far ahead of the actual event occurrence time the UE should be able to predict the event.

    - Potential RRM requirement impact for event prediction use case - for indirect case:

      - RAN4 to study the requirement for the predicted event triggered reporting

        - RAN4 to study the requirement for [‘delay’ and accuracy]

        - RAN4 to discuss the metric for the event prediction

      - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time],

        - Candidate options:

          - Option 1: RAN4 will define requirement for the predicted RSRPs

          - Option 2: RAN4 will not define requirement for the predicted RSRPs

  - Proposal 10 (CMCC): For the final output of measurement event prediction (e.g. the probability of event occurrence for direct prediction or expected occurrence time of a event for indirect prediction), it is proposed to discuss whether and how to define RRM requirements.

  - Proposal 11 (MediaTek): Too early to discuss the metrics to define requirements for in RAN4 as the reported quantities are open in RAN2.

- Candidate options:

  - Option 1: RAN4 to wait for further progress in RAN2 before starting discussion on core requirements in RAN4 for event prediction use case.

  - Option 2: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting

  - Option 3: RAN4 not to define delay requirements for predicted event triggered reporting

- Recommended WF

- Discuss above listed Candidate Options.

Issue 3-4: Scenario priorities for event prediction

- Proposals and Observations:

  - Proposal 1 (ZTE): For measurement event prediction, RAN4 should at least consider scenarios 2(FR1 to FR1 intra-frequency temporal domain case B), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A)

  - Proposal 2 (Nokia): RAN4 to study the requirements and performance metrics for indirect measurement event prediction temporal domain Case A and Case B

- Candidate options:

  - Option 1: RAN4 to consider

    - scenarios 2(FR1 to FR1 intra-frequency temporal domain case B),

    - scenario 3 (FR1 to FR1 inter-frequency (frequency domain), and

    - scenario 4 (FR2 to FR2 intra-frequency temporal domain case A).

      - Option 1a: Consider also non-collocated scenarios

- Recommended WF

  - RAN4 to discuss if Option 1 is agreeable.

Issue 3-5: Ground-truth definition for event prediction

- Proposals and Observations:

  - Proposal 1 (Qualcomm): For the definition of ground truth of predicted event occurrence time accuracy (reported predicted event occurrence time – ground truth of the event occurrence time) in the time domain, the ground truth of the event occurrence time should be defined by the same principles as the ground truth of predicted L3-RSRP. If a down selection is made during the study item phase for the definition of ground truth of predicted L3-RSRP, the same principle should be inherited for the definition of ground truth of predicted event occurrence time. The options for the definition of ground truth of predicted L3-RSRP are listed below (options are duplicated below for reference from our companion paper submitted for AI 7.21.2.1):

    - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate. The value of the margin needs a further study, and it can vary for different measurement configurations, etc.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

    - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition as the prediction-based L3-RSRP report (UE-reported L3-RSRP with noise)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms.

        - To avoid the UE from cheating the test, TE can either create another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction or run two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report.

    - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough.

        - Another round of tests would be needed for the L3-RSRP measurement report

    - RAN4 to strive to not differentiate the definition of the ground truth between FR1 and FR2.

- Recommended WF

  - RAN4 to wait for the conclusion on the ground-truth definition in RRM measurement prediction before starting discussion on this issue.