3GPP TSG-RAN WG4 Meeting #115                                          R4-2505584 Malta, May 19 - 23, 2025

Source: 	Huawei, HiSilicon

Title: 	On power domain enhancements for single carrier

Agenda Item:	7.1.2.2.1

Document for:	Approval

Introduction

During RAN4#114bis, power domain enhancement for single carrier has been extensively discussed. The progress can be found in [1] and we reproduce those agreements here.

- Issue 1-1-1: RB based or CBW based extension - WF - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5 - The extended RBs are aligned with common RB grid - Issue 1-1-2: Fixed or flexible extension - Agreement in main session: - Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature - In addition, consider a limited number of additional fixed values with capability - Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - FFS in next meeting - Issue 1-1-4: Guard band for extended CBW - Options - Option 1: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band. - Option 2: NRB based extension and associated guard band: - maximum bandwidth extension: The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext. - flexible NRB-based extension: Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext - Option 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW. The GBs of extended UE CBW reuse the size of original GB of UE CBW. - Option 4: The GBs of the extended CBW shall be the remaining portion obtained by subtracting the RB numbers from the specified channel bandwidth when the extended CBW is selected from Table 5.3.2-1 in 3GPP TS 38.101-1. - Others - WF - FFS in next meeting - Issue 1-2-1: A-MPR handling - Proposals - Option 1: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. - Option 2: - If A-MPR is specified for a given RB allocation X, then the UE performs max(MPRenh(X), A-MPR(X)) - If A-MPR is not specified for a given RB allocation, then the UE performs max(MPR(X), A-MPR(X)) - Option 3: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function based on the specified A-MPR and the initial UE CBW MPR and thus set its backoff ignoring any MPR reduction. - Option 4: max(MPRenh(X), A-MPR(X)), where A-MPR(X) is determined based on a given RB allocation, and RB region with respect to initial CBW, WF and modulation etc, and MPRenh(X) is determined based on RB region/etc. within the extended UE CBW. - Others - WF - Only apply the power domain enhancement scheme to channels which are not subject to A-MPR in Rel-19. - Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW Issue1 Issue 2; Option1. Allocate the extended RBs from the GB of the original UE; Option2. Allocate the extended RBs from the edge of the UE CBW. Option1. Individual evaluation of the extended RBs & Horizontal line; Case1; Case2. Option2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line; Case3; Case4. Note: It is assumed that extended RBs are RB aligned with the UE and BS RBs for the listed cases.. - WF: - Case 1 and case 3 could be further down selected - GB of UE CBW may or may not be scheduled to other UEs - Extended IBE can be applied in the GB of UE CBW - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW - Issue 1-2-3: Derivation of the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - Option 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs. - Option 3: Others - WF - Discuss together with issue 1-2-2, and down select the options in next meeting. - Issue 1-2-4: Relaxation of the extended IBE - Proposals - Proposal 1: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. (Xiaomi) - Proposal 2: (vivo) - If the extended RBs are allocated from the GB of the original UE (case1/3), then the IBE in the GB should be relaxed to ensure the MPR of outer allocation reduces to the inner level: - Way 1: Skip the IBE evaluation for the extended RBs allocated in the GB. - Way2: Relax EVM in the IBE formula to that for BPSK for extended RBs. IBE_general = max(PRB - 30 dB, ) - WF - For case 1 in issue 1-2-2, use BPSK EVM in the IBE formula for all extended RBs - For case 3 in issue 1-2-2, FFS the relaxation - Issue 1-3-1: NW signaling - WF - FFS in next meeting - Issue 1-3-2: UE capability - Agreement in main session: - An optional UE capability to indicate whether it supports the CBW extension based MPR reduction feature - To discuss the following aspects - Whether to have separate UE capabilities for scenario 1 and scenario 2 - Whether to indicate multiple values of fixed extension capability

In this contribution, we would like to share our views on this topic concentrating on the most controversial issues.

Discussion

- 2.1 Last pieces of the puzzle

As those FFS points are somehow interrelated, we should try to start from the most critical one. In essence, MPR reduction gain is mainly provided by the relaxation of OOB requirements, what kind of IBE requirements apply to the extended CBW actually play a less important role because IBE normally is not the bottleneck for meeting Inner MPR with target waveform. In this sense, CBW based extension is a bit more straightforward than RB based extension because there is no need to worry about the guard band for extended CBW anymore. But for RB based indication, UE also needs to know the guard band for extended CBW in order to figure out where to apply the shifted OOB.

Figure 1. UE PSD with 100MHz transmitting bandwidth

From the above figure, it can be observed that the emissions in adjacent channel exhibit a roll-off rather than upturned trend as the frequency moves away from the centre. Therefore, the emission leakage to the adjacent channel would depend largely on the applied requirements for the near frequency of the UE CBW. Since RAN4 already agreed that the extended IBE can be applied in the legacy guard band, how to define the guard band for the extended CBW becomes insignificant. In conclusion, we have the following proposal.

Proposal 1: The limited number of fixed values smaller than the 0.5*UE CBW/NRB shall be selected from Table 5.3.2-1 in TS 38.101-1, assuming the same SCS as the one configured for the UE CBW.

Proposal 2: Since the extended RBs are allocated from the legacy guard band, the guard band for the extended CBW GBext = GB1+GB2, where GB1 is the guard band for the UE CBW and GB2 is the guard band defined for the channel bandwidth corresponding to the extended CBW.

Regarding how to get the actual NRB which is the input for new Inner/Outer allocations determination, a straightforward solution would be referring to the same table, i.e. Table 5.3.2-1 in TS 38.101-1, as we proposed for extended CBW. Hence the function for deriving total NRB can be replaced to the following one:

N'RB=Nextended_low+NRB+Nextended_high,

where Nextended is the maximum transmission bandwidth configuration of the extended CBW corresponding to the length of the extended CBW, expresses in MHz, configured by network and the subcarrier spacing of this component carrier. The rest of the suffix is to distinguish the extended CBW at the lower frequency side from the higher one.

Proposal 3: Adopt following equation for deriving the total maximum transmission bandwidth configuration, where Nextend_low/high is the maximum transmission bandwidth configuration corresponding to the subcarrier spacing of the component carrier and the extended CBW, expresses in MHz, located at lower or higher frequency side, respectively.

N'RB=Nextended_low+NRB+Nextended_high

For better illustration, we can provide an example as:

- The assigned UE CBW is 20MHz with 30kHz SCS

- Both lower and higher extended CBW are configured as 10MHz

Then N'RB would be 24+51+24=99 (which is not the NRB=106 defined for 40MHz). Then actually the UE would be exempted on some RB allocations in a way to still apply the Outer MPR.

Figure 2. Example on how to determine the new Inner RB allocations with configured N'RB

As to how to derive the new Inner region, following method to accommodate both symmetric and asymmetric is proposed.

- Step 1 (legacy specification): For a given RB allocation (RBStart, LCRB), label it as Inner/Outer/Edge according to the legacy definition.

- Step 2 (dedicated for Rel-19 MPR reduction): If this is an Outer RB allocation, below criteria constructed by the extended CBW step in.

  - RB’Start_Low = max(1, floor(LCRB/2)) - NRB_low, to translate the RBStart_Low along the lower side

  - RB’Start_High = NRB – RBStart_Low – LCRB + NRB_high, to translate the RBStart_High along the higher side

  - If RB’Start_Low  ≤  RBStart  ≤  RB’Start_High, and, LCRB  ≤  ceil(N’RB/2) then this Outer RB allocation belongs to new Inner

Corresponding spec changes are as below.

Proposal 4: Adopt the following derivation method of new inner RB region in extended CBW.

- Step 1 (legacy specification): For a given RB allocation (RBStart, LCRB), label it as Inner/Outer/Edge according to the legacy definition.

- Step 2 (dedicated for Rel-19 MPR reduction): If this is an Outer RB allocation, below criteria constructed by the extended CBW step in.

  - RB’Start_Low = max(1, floor(LCRB/2)) - NRB_low, to translate the RBStart_Low along the lower side

  - RB’Start_High = NRB – RBStart_Low – LCRB + NRB_high, to translate the RBStart_High along the higher side

  - If RB’Start_Low  ≤  RBStart  ≤  RB’Start_High, and, LCRB  ≤  ceil(N’RB/2) then this Outer RB allocation belongs to new Inner

For the extended IBE, our preference is to adopt the Case 1 with the BPSK EVM replacement, so that enough margin can be reserved since it was agreed that the extended IBE can be applied in the legacy guard band where frequency jitter exists.

Proposal 5: Adopt Case 1 for the extended IBE definition and use BPSK EVM in the IBE formula for all extended RBs.

- 2.2 UE capability and network signalling

Per discussion in last meeting, there are two aspects should be considered:

- Whether to have separate UE capabilities for scenario 1 and scenario 2

- Whether to indicate multiple extension ratios

The intention of the first aspect can be understood but it would be strange to reflect such differentiation by UE capability, because the extended CBW shall be configured by the network and the UE will determine the applied MPR for a specific RB allocation accordingly. As for the second aspect, how flexible such extension configuration can be will definitely impact the implementation/verification cost. If there are few suitable spectrum allocations, pursuit of that flexibility will become an overdesign. However, RAN4 agreed to consider a limited set of extension ratios. As long as a dedicated and optional UE capability can be assigned for flexible extension in additional to the default case i.e. fixed half CBW extension, we think the concern for additional implementation/verification complexity can be alleviated. In conclusion, we have the following proposal on UE capability.

Proposal 6: UE capability for Rel-19 single CC MPR reduction can be structured as below.

- UE capability #1: To indicate whether the UE supports half UE CBW extension.

- UE capability #2: To indicate whether the UE supports other ratio (< 1/2) of UE CBW extension.

- For these two optional UE capabilities, the first one is the prerequisite of the second one. The extension is UE CBW agnostic i.e. it shall be applicable to the maximum channel bandwidth that this UE indicates.

For network signalling, it depends on how many extension ratios will be allowed pending on RAN4 discussion. For instance, if one value e.g. 1/4 is allowed in addition to the default ratio, 2 bits is needed to configure {1/2, 1/4, 0}*CBW extension for one side. Since the asymmetric extension should also be covered, 4 bits would be enough. Note that the network configuration shall align with the UE capability indication, 1/4 is an invalid configuration for the UE only supports default ratio extension.

Proposal 7: Network signalling for the extension ratio configuration can be structured as below.

- Assigning the same bit length for each side of the UE CBW to cover the asymmetric extension case. Bit length is pending on RAN4 discussion on how many extension ratios can be supported.

- The network configuration shall be aligned with UE capability. The extension ratio other than 1/2 shall not be configured to the UE that only supports half UE CBW extension.

- 2.3 UE feature list

We summarize our UE feature list proposal as in below table, note that it covers both MPR reduction for single CC and MPR applicability enhancement for UL CA [2, 3].

Proposal 8: Adopt the following UE feature list for Rel-19 WI NR_ENDC_RF_Ph4 objective Power boosting and/or MPR reduction.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | MPR reduction for single carrier-fixed | Indicates whether UE supports MPR reduction for single UL carrier with symmetric and asymmetric 1/2*UE CBW extension. |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | Yes (only FR1) | N/A | This capability is applicable for single UL CC case | Optional |
|  | 46-2 | MPR reduction for single carrier-flexible | Indicates whether UE supports MPR reduction for single UL carrier with symmetric and asymmetric 1/X*UE CBW extension, where 1/X is a set of ratios smaller than 1/2 [{1/4, …}]. | 46-1 | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | Yes (only FR1) | N/A | This capability is applicable for single UL CC case | Optional |
|  | 46-3 | MPR enhancement for activated carrier | Indicates whether UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA. |  | Yes | N/A | The FR1 UE shall support MPR requirement based on UL CA configuration even for single activated CC. | Per BC | No | Yes (only FR1) | N/A |  | Optional |
|  | 46-4 | FR2 MPR-Improvement Downlink Independent | Indicates whether UE supports reduced MPR by removing dependence on DL CA configuration. |  | Yes | N/A | The FR2 UE only supports MPR requirement with dependence of DL CA configuration. | Per BC | Yes | Yes (only FR2) | N/A |  | Optional |
|  | 46-5 | FR2 MPR Improvement Activation Dependent | Indicates whether UE supports MPR based on activation status of its configured CCs |  | Yes | N/A | The FR2 UE shall support MPR requirement based on UL CA configuration. | Per BC | Yes | Yes (only FR2) | N/A |  | Optional |

Conclusion

In this contribution we discussed on the Rel-19 MPR reduction for single CC. We have the following proposals:

Proposal 1: The limited number of fixed values smaller than the 0.5*UE CBW/NRB shall be selected from Table 5.3.2-1 in TS 38.101-1, assuming the same SCS as the one configured for the UE CBW.

Proposal 2: Since the extended RBs are allocated from the legacy guard band, the guard band for the extended CBW GBext = GB1+GB2, where GB1 is the guard band for the UE CBW and GB2 is the guard band defined for the channel bandwidth corresponding to the extended CBW.

Proposal 3: Adopt following equation for deriving the total maximum transmission bandwidth configuration, where Nextend_low/high is the maximum transmission bandwidth configuration corresponding to the subcarrier spacing of the component carrier and the extended CBW, expresses in MHz, located at lower or higher frequency side, respectively.

N'RB=Nextended_low+NRB+Nextended_high

Proposal 4: Adopt the following derivation method of new inner RB region in extended CBW.

- Step 1 (legacy specification): For a given RB allocation (RBStart, LCRB), label it as Inner/Outer/Edge according to the legacy definition.

- Step 2 (dedicated for Rel-19 MPR reduction): If this is an Outer RB allocation, below criteria constructed by the extended CBW step in.

  - RB’Start_Low = max(1, floor(LCRB/2)) - NRB_low, to translate the RBStart_Low along the lower side

  - RB’Start_High = NRB – RBStart_Low – LCRB + NRB_high, to translate the RBStart_High along the higher side

  - If RB’Start_Low  ≤  RBStart  ≤  RB’Start_High, and, LCRB  ≤  ceil(N’RB/2) then this Outer RB allocation belongs to new Inner

Proposal 5: Adopt Case 1 for the extended IBE definition and use BPSK EVM in the IBE formula for all extended RBs.

Proposal 6: UE capability for Rel-19 single CC MPR reduction can be structured as below.

- UE capability #1: To indicate whether the UE supports half UE CBW extension.

- UE capability #2: To indicate whether the UE supports other ratio (< 1/2) of UE CBW extension.

- For these two optional UE capabilities, the first one is the prerequisite of the second one. The extension is UE CBW agnostic i.e. it shall be applicable to the maximum channel bandwidth that this UE indicates.

Proposal 7: Network signalling for the extension ratio configuration can be structured as below.

- Assigning the same bit length for each side of the UE CBW to cover the asymmetric extension case. Bit length is pending on RAN4 discussion on how many extension ratios can be supported.

- The network configuration shall be aligned with UE capability. The extension ratio other than 1/2 shall not be configured to the UE that only supports half UE CBW extension.

Proposal 8: Adopt the following UE feature list for Rel-19 WI NR_ENDC_RF_Ph4 objective Power boosting and/or MPR reduction.

- References

- R4-2505099, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #114bis, Wuhan, China, April 7-11, 2025.

- R4-2505144, “LS on introduction of new UE capabilities for FR2 MPR improvement”, Samsung, Qualcomm, 3GPP TSG-RAN4 Meeting #114bis, Wuhan, China, April 7-11, 2025.

- R4-2505212, “DraftCR to 38.101-1 MPR applicability for FR1 intra-band contiguous UL CA”, ZTE, Skyworks, Samsung, Huawei, vivo Qualcomm, 3GPP TSG-RAN4 Meeting #114bis, Wuhan, China, April 7-11, 2025.