3GPP TSG-RAN WG4 Meeting #115                                           R4-2505590

Malta, May 19 - 23, 2025

Source: 	Huawei, HiSilicon

Title: 	On potentially new requirements for SBFD

Agenda Item:	7.25.3.1

Document for:	Approval

Introduction

During RAN4#114bis, potentially new requirements for SBFD have been discussed. Following conclusions and FFS points are reproduced from [1].

- In-channel adjacent subband leakage Agreement: - The requirement (In-channel adjacent subband leakage ratio) is renamed as “in-channel adjacent subband leakage requirement” (by removing “ratio”). ##### Assumption for in-channel adjacent subband leakage requirement Way forward: - FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage. ##### Absolute limit for in-channel adjacent subband leakage Way forward: - Absolute limit for in-channel adjacent subband leakage: - FFS by considering the following exemplary cases by performing analysis including measurement or simulation: - FR1: - DU/UD: 80MHz (DL) – 20MHz (UL) - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL) - Other cases are not precluded - FR2-1: - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL) - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL) - Other cases are not precluded - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit. ##### Necessity of in-channel adjacent subband leakage ratio Way forward: - No need to further discuss the necessity of in-channel adjacent subband leakage ratio, based on agreement from RAN4#114. ##### In-channel adjacent subband blocking requirement Way forward: - New requirement for in-channel adjacent subband blocking: - The requirement can be derived by considering co-site inter-sector case and/or inter-site case - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification. - The range of [-35dBm to -27dBm] can be used as the starting point for further study. ##### Assumption of gNB-to-gNB CLI handling Way forward: - FFS the assumption of gNB-to-gNB CLI handling for two cases: - Inter-site case: - Note: for the blocking requirement, RX digital beamforming cannot be considered. - Co-site inter-sector case: - Note: Inter-sector is near field interaction.

In this contribution, we would like to share our views regarding this topic.

Discussion

In-channel adjacent subband leakage

Since it was agreed to define in-channel adjacent subband leakage and the discussion direction is to check whether absolute ACLR can be considered as baseline, we would like to provide some analysis accordingly. In the current spec, there are ACLR and CACLR defined for different cases, and for each of them the relative limit and absolute limit are defined separately while the verification is only required with whichever less stringent.

Viewing the Rel-18 SI simulation assumptions, the 45 dB ACLR was selected for the example 5GHz operating frequency. If we consider the BS Tx power as to output 53dBm within 100MHz RF bandwidth, then it is equivalent to 33dBm/MHz, so roughly -12dBm/MHz could be the residual noise level in the adjacent channel. However, such level would be higher if we consider U6G bands due to the ACLR requirement becomes 38dB.

Observation 1: Assuming 45dB ACLR and 53dBm BS output power over 100MHz, roughly -12dBm/MHz level residual noise level in the adjacent channel can be expected. However, it will get higher for U6G since the ACLR requirement is relaxed to 38dB.

Regarding the assumption for in-channel adjacent subband leakage, it is not clear what kind of assumption is for study. At least one argument that goes without saying could be endlessly adding assumption may kill implementation flexibility eventually, which must not be the purpose of feature design.

Observation 2: Seemingly no need to introduce assumption into the core part spec if existing requirement can be reused.

To make this requirement simpler, what matters is the interference leaked to the UL frequency, thus we can accept to reuse the legacy absolute limit as it is, so we have the following proposal on this requirement.

Proposal 1: Suggest to reuse ACLR absolute basic limit given in Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2-1, for the new in-channel adjacent subband leakage.

Potential adjacent subband blocking

Regarding in-channel adjacent subband blocking, it has been agreed that new requirement is needed. But the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes should also be considered. To our understanding, such DL sub-band interference might cause some nonlinearity to LNA or other RF components but that interference is anyway coming from the same operator so some methods e.g. beam nulling/coordination can be applied in order to mitigate the interference.

Observation 3: Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

In our view, the source interference for this requirement only consists of the inter-site interference and the co-site inter-sector interference, the self-interference shall be precluded.

Proposal 2: For in-channel adjacent subband blocking, the source interference only consists of the inter-site interference and the co-site inter-sector interference. The self-interference shall be precluded.

For the co-site inter-sector case, the interference level can be around -27dBm/100MHz, which is from 53dBm/100MHz output power minus 80dBc isolation. During the feasibility study in Rel-18, interference mitigation capability e.g. beam nulling is varied among companies. But if 10dB can be considered here, the required MCL can be reduced to 70dB so that the potential deployment restriction could be reduced. Anyway, if RAN4 can go with the worst case, such interference level would be -27dBm/100MHz.

For the inter-site case, following simulation based on our co-ex simulation platform is conducted to provide some inputs for showing the interference level, note that CLI handling is NOT included.

Figure 1. CDF of in-channel adjacent subband blocking w/o CLI handling

According to the result, it is -28.03 dBm/100MHz @99%-tile. In our view, 15dB gNB-to-gNB CLI handling can be considered here, so we think -43dBm/100MHz is reasonable but it is negligible comparing to the co-site inter-sector interference level. Therefore we have the following proposal.

Proposal 3: Specify the in-channel adjacent subband blocking as -27dBm/100MHz.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following observation and proposals:

Observation 1: Assuming 45dB ACLR and 53dBm BS output power over 100MHz, roughly -12dBm/MHz level residual noise level in the adjacent channel can be expected. However, it will get higher for U6G since the ACLR requirement is relaxed to 38dB.

Observation 2: Seemingly no need to introduce assumption into the core part spec if existing requirement can be reused.

Observation 3: Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

Proposal 1: Suggest to reuse ACLR absolute basic limit given in Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2-1, for the new in-channel adjacent subband leakage.

Proposal 2: For in-channel adjacent subband blocking, the source interference only consists of the inter-site interference and the co-site inter-sector interference. The self-interference shall be precluded.

Proposal 3: Specify the in-channel adjacent subband blocking as -27dBm/100MHz.

- References

- R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #114bis, Wuhan, China, April 7-11, 2025.