3GPP TSG-RAN WG4 Meeting #115                                           R4-2505591

Malta, May 19 - 23, 2025

Source: 	Huawei, HiSilicon

Title: 	Overview on conformance testing for SBFD

Agenda Item:	7.25.3

Document for:	Approval

Introduction

According to the work plan, RAN4 can start to think about the conformance testing for SBFD in this meeting. (Partially copied from the proposed Work plan for Evolution of NR Duplex Operation – RAN4 Performance Part)

| Schedule and TU | Work plan |
| --- | --- |
| May, 2025 (0.5 RF + 0.25 RD TU for Perf. part) | RAN4 (performance) - Begin discussion on - BS RF conformance requirements to support SBFD operation at gNB - Discussion on RF conformance requirement scope. - RRM performance requirements to support SBFD operation - Discussion on RRM performance requirement scope. - SBFD-capable BS and/or SBFD-aware UE demodulation performance requirements to support SBFD operation - Discussion on demodulation performance requirement scope. |
| August, 2025 RAN4#116 (0.5 RF + 0.25 RD TU for Perf. part) | RAN4 (performance) - Continue discussion on - BS RF conformance requirements to support SBFD operation at gNB - Confirm the scope for BS RF conformance requirements. - Identify and resolve key test issues for BS RF conformance testing. - CR drafting work-splitting confirmed for the conformance testing general requirements and test cases for SBFD-capable BS - RRM performance requirements to support SBFD operation - Confirm the scope for RRM performance requirements considering the UE-to-UE CLI handling and impacted RRM core requirements. - Identify and resolve key issues for RRM performance testing. - CR drafting work-splitting on RRM performance testing general requirements and test cases - SBFD-capable BS and/or SBFD-aware UE demodulation performance requirements to support SBFD operation - SBFD-capable BS and/or SBFD-aware UE demodulation performance requirements to support SBFD operation. - Identify and resolve key issues for demodulation performance testing. - CR drafting work-splitting on demodulation performance testing general requirements and test cases |

In this contribution, we would like to share our high level views regarding the SBFD BS RF conformance requirement scope in the light of current RF requirements discussion progress.

Discussion

Views regarding the SBFD BS RF conformance requirement scope

Review the RF core part discussion so far, we list the agreements (or way forward) that may need dedicated considerations for SBFD conformance testing.

|  | Objective | Agreement/Way forward |
| --- | --- | --- |
| General | Subband configuration | “All declared configurations shall be tested” |
| BS TX requirements | Output power and radiated transmit power | PSD scaling for normal and SBFD slots/symbols: - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols. - Vendors can declare different TX power values for normal and SBFD slots/symbols. |
|  | Output power dynamics | - Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement. - The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured. |
|  | Transmit ON/OFF power | - Transmitter ON/OFF power should apply to normal symbol and it is not applicable within SBFD symbol. |
|  | Transmitted signal quality | - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. - Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity. |
| BS RX requirements | OTA sensitivity degradation sue to SI | - 0.7 dB degradation on top of existing minimum requirements for OTA sensitivity. |

As we can see, there are plenty of core requirements shall be specifically considered for SBFD since a fundamental difference between TDD and SBFD is that there will be at least one DL channel and UL channel configured simultaneously in time domain. Thus we think one issue that needs to be examined is whether existing FRC and test model can be reused for SBFD.

Observation 1: The existing UL FRC can be reused for SBFD testing.

Observation 2: How to accommodate SBFD time domain pattern with the test model need to be further studied.

Another thing that worth mentioning is that the source interference signal for specific requirement may need to be checked. For instance, the 0.7dB OTA desensitization was agreed as only self-interference can be counted. Thus any other interferer shall not be generated for the test.

Observation 3: The interference signal generation for specific requirement shall be aligned with the assumptions for the corresponding requirement.

Conclusion

In this contribution we share our initial thinking on the conformance testing. We have the following observations:

Observation 1: The existing UL FRC can be reused for SBFD testing.

Observation 2: How to accommodate SBFD time domain pattern with the test model need to be further studied.

Observation 3: The interference signal generation for specific requirement shall be aligned with the assumptions for the corresponding requirement.

- References