3GPP TSG-RAN WG4 Meeting #115			R4-2505592

St Julian’s, Malta, 19-23 May 2025

Agenda item:	7.33.5

Title:	Discussion on UE RF requirements for NTN NB-IoT TDD mode

Source:	Nordic Semiconductor ASA

Document for:	Discussion

1. Introduction

In RAN1#114b in Wuhan the following was agreed for in-band SEM:

Agreement: As a starting point, RAN4 can consider for discussions: - Option 1: Since similar frequency range between 249 and 254, A-MPR requirement for 249 shall follow simulation results in TS 36.764 and A-MPR requirements (to be defined) for 254 in TS 36.102. 1a/ As starting point, for 1-tone allocation (priority 1) – see TR 36.764: Table 6.2.3.2-3: 1-Tone A-MPR for NS_04N power class 3 Modulation; π/4 QPSK. Tone positions for 15kHz 1-Tone allocation; 0/11; 1/10; 2/3/4/5/6/7/8/9. A-MPR; ≤ 0.5 dB; 0 dB; 0 dB. Table 6.2.3.2-4: 1-Tone A-MPR for NS_05N power class 3 Modulation; π/4 QPSK. Tone positions for 15kHz 1-Tone allocation; 0/11; 1/10; 2/9; 3/4/5/6/7/8. A-MPR; ≤ 1.5 dB; ≤ 1.0 dB; 0 dB; 0 dB. 1b/ As starting point, for multiple-tone allocation (priority 2) – see TR 36.764: Table 6.2.3.2-1: A-MPR for NS_04N power class 3 Modulation; QPSK. Tone positions for 3 Tones allocation; 0-2; 3-5 and 6-8; 9-11. A-MPR; ≤ 0.7 dB; 0.2 dB; ≤ 0.7 dB. Tone positions for 6 Tones allocation; 0-5 and 6-11. A-MPR; 0 dB; 0 dB. Tone positions for 12 Tones allocation; 0-11. A-MPR; 0 dB. Table 6.2.3.2-2: A-MPR for NS_05N power class 3 Modulation; QPSK. Tone positions for 3 Tones allocation; 0-2; 3-5 and 6-8; 9-11. A-MPR; ≤ 1.5 dB; 0.5 dB; ≤ 1.5 dB. Tone positions for 6 Tones allocation; 0-5 and 6-11. A-MPR; ≤ 0.7 dB; ≤ 0.7 dB. Tone positions for 12 Tones allocation; 0-11. A-MPR; 0 dB. 1c/ Other options not precluded. - Option 2: Re-evaluate A-MPR for 249 based on PC3/PC5 (priority for PC3) assumption using at least the following references: ETSI EN 301 441, ITU-R M.1343-1

In this contribution we further analyse ETSI harmonized standard requirements.

2. Discussion

In-band spectral emission mask

The ETSI requirement [2] for unwanted emissions within the band 1610 MHz to 1628.5 MHz applies for the nominated bandwidth which is defined in [2] to be specified by the applicant. Therefore, RAN4 should not make the generalization of defining “nominated bandwidth” equals “channel bandwidth”. And if UE declares nominated BW to be X, such UE shall not access channels which are less than X/2 from the edge of band, i.e. 1616-1626.5 MHz. If a UE declares nominated bandwidth of 400kHz, the standard NB-IoT SEM from Table 6.5B.3.2-1 from TS36.102 can apply. Because at 100kHz from channel BW edge emission limit is -5dBm which matches NS_05N.

Proposal-1: Support Option 2: Additional spectrum emission masks (i.e. NS_04N and NS_05N) should not be defined for band 249.

EIRP density within the operational band

According to our investigation, no aeronautical radionavigation systems are deployed in the frequency range of 1616-1626.5MHz. Therefore, the operational band’s mean EIRP density requirement in [1] of under -3 dB(W/4kHz) applies for the band 249. Therefore, agreed 23dBm transmit power is well within the requirements.

Observation-1: Current specification satisfies the operational band’s mean EIRP density requirement.

NPDSCH demodulation performance

36.102 defined NPDSCH demodulation performance requirements.

In RAN4#114b, it has been agreed that target SNR for TDD band is -1dB. Therefore, the NPDSCH Minimum performance tables shall be updated accordingly in 36.102.  We suggest assuming 4 repetitions for NPDSCH and set R_max=4 and G=16. The MCS (iTBS, iSF) to be selected is FFS.

Proposal-2: Revisit the minimum performance requirements for NPDSCH for R17 NTN

- assume 4 NPDSCH repetitions, -1dB target SNR, R_max=4 and G=16, FFS: MCS (iTBS, iSF)

3. Conclusion

Proposal-1: Support Option 2: Additional spectrum emission masks (i.e. NS_04N and NS_05N) should not be defined for band 249.

Observation-1: Current specification satisfies the operational band’s mean EIRP density requirement.

Proposal-2: Revisit the minimum performance requirements for NPDSCH for R17 NTN

- assume 4 NPDSCH repetitions, -1dB target SNR, R_max=4 and G=16, FFS: MCS (iTBS, iSF)

4. References

[1] Radio regulations, ITU, 2024

[2] ETSI EN 301 441, V2.1.1, 2016