3GPP TSG-RAN WG4 # 115	R4-2505593

Saint Julian’s, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	7.16.3

Source:	MediaTek Inc.

Title:	Discussion on Spatial Channel Modelling

Document for:	Discussion

# Introduction

The Rel-19 study item [1] investigates practical spatial channel modelling methodologies for both SU-MIMO and MU-MIMO demodulation and CSI reporting requirements. It has been identified that the legacy TDL-based models and spatial correlation matrices poorly match with realistic propagation conditions that have been used as the design assumption for the NR downlink precoder (PMI) codebooks.

For spatially selective channel model development, both CDL (Clustered Delay Line) and TDL models are considered as baseline. It has been demonstrated in this study item that by the multi-cluster TX-RX beam steering approach, the simple TDL models can be extended to have desired spatial properties similar to CDL models.

# Discussion

# 2.1 Comparison of Channel Statistics

In this section, we propose a set of channel statistics for comparison and alignment. Furthermore, we illustrate and compare those statistics for both CDL and multi-cluster TDL channels.

Performance Metrics

Way Forward:

Channel Model candidates to be compared in two aspects:

- Performance Properties

- Channel Properties (captured in an annex)

Performance Properties include:

- [TPUT with follow precoding]

- [TPUT with fixed precoding]

- [TPUT with random precoding]

- [Metrics Statistics and Spread]

- [SNR Operating Point]

- [Convergence time]

Channel Properties include:

- [Eigenmodes and Eigenvectors]

- [Power Angular Distribution]

- [Time Coherence]

- [Frequency Coherence]

- [PMI Preference]

It has turned out that tput performance alignment and comparison with CDL channels between companies is difficult. Therefore, we propose to carry out initial alignment/comparison via simpler alternative metrics that omit the uncertainty resulting from different receiver algorithms implementations. These alternative metrics should be based on measured channel statistics that can be recorded and cross-checked between companies.

Proposal #1: We propose the following metrics for alignment between companies and to compare the properties of different channel models:

- Spatial domain: Power Spectral Density (PSD) or Power Angular Distribution (PAD) of the channel in TX and RX dimensions

- Time domain fading: Time Coherence (TC) function measured as channel correlation between symbols

- Frequency domain: Frequency Coherence (FC) function measured as channel correlation between subcarriers

To specify the channel metrics, we define the following notation. Arrange MIMO channel as a complex-valued 7-dimensional array:

hn,k,m1,m2,l,f, t , where

  - n=0,1,…, Nrx1-1 is RX-antenna index within one polarization group,

  - k=0,1 is the RX-antenna polarization group index,

  - m1=0,1,…, Ntx1-1 is 1st TX-antenna index within one polarization group,

  - m2=0,1,…, Ntx2-1 is 2nd TX-antenna index within one polarization group,

  - l=0,1 is the TX-antenna polarization group index,

  - f is frequency (subcarrier) index for channel frequency response (CFR), and

  - t is time (symbol) index.

Note that we assume Ntx1 is the TX-array size in horizontal direction and Ntx2 in vertical direction. For 4TX and 8TX cases, Ntx2=1. It is also worth noting that channel impulse response (CIR) representation can alternatively be used (except for FC) by replacing the frequency dimension with delay tap index dimension.

We define the spatial power spectral density (PSD) in RX-domain (AOA) as

PSDrxθ=Ek,m1,m2,l, f,t1Nrx1n=0Nrx1-1hn,k,m1,m2,l, f, t∙e-j∙θ∙n2

and PSD in TX-domain (AOD) as

PSDtxθ=En,k,m2,l, f,t1Ntx1m1=0Ntx1-1hn,k,m1,m2,l, f, t∙e-j∙θ∙m12

Here, the range for phase shift θ is (-π,π) or equivalently (0,2π). FFT can be utilized for fast calculation of PSD. Notation E(index list) denotes statistical mean over the listed indexes/dimensions.

As an alternative to spatial PSDθ, power angular distribution PADφ can be specified as function of AOA/AOD by substituting θ=π∙sinφ in the definition of PSD. Note however that while the physical range of AOD/AOA is (-180,180) degrees, the meaningful range of φ is only (-90,90) degrees. This is due to the fact that from the channel realization h only, it is not possible to distinguish the whole physical angle range. For example, PAD90-x=PAD(90+x), which means that the power responses of physical angles 90-x and 90+x fall on top of each other. Therefore, we prefer applying PSD instead of PAD. Plotting PAD would give a slightly false impression of the physical meaning of the metric.

We define complex-valued time coherence (TC) as

TC∆t=En,k,m1,m2,l, f,th*n,k,m1,m2,l, f, t∙hn,k,m1,m2,l, f, t+∆t

where ∆t=0, 1, 2,…Nt symbols (or multiples of symbols).

We define complex-valued frequency coherence (FC) as

FC∆f=En,k,m1,m2,l, f,th*n,k,m1,m2,l, f, t∙hn,k,m1,m2,l, f+∆f, t

where ∆f=0, 1, 2,…Nf subcarriers (or multiples of subcarriers).

As can be seen, the definitions of the proposed metrics are simple. Therefore, we believe they should be easy to implement and align as well. In the following, we plot our measured metrics for the CDL-C and the multi-cluster TDL models 1, 2, and 3.

Figure 1 depicts the spatial PSD of CDL-C A2/A4 with AAV Options 1 and 3. As can be seen that the TX side is much more spatially selective than the RX side. The difference between A2 (non-truncated) and A4 (truncated) models as well as between the two AAV options is visible but minor.

Figures 2 and 3 show the absolute value of FC for CDL-C A2 and A4, respectively. As can be seen, the frequency coherence properties of the 24-cluster A2 and the 12-cluster A4 models somewhat differ. Furthermore, for A4, the choice of AAV has an impact. This is probably due to the fact that the AAV 1 and 3 include slightly different sets of clusters selected from the original A2, and the delay scaling is different as well.

Figures 4 and 5 show the TC of CDL-C A2 and A4, respectively. As can be seen, TC is complex-valued. This implies that the Doppler spectrum is not symmetric around zero frequency. The shapes of TC for A2/A4 and AAV 1/3 slightly differ, but no major systematic difference is seen.

Observation #1: The spatial selectivity and the time coherence (fading) properties of CDL-C models A2 and A4 – with both AAV options 1 and 3 – remain similar.

Observation #2: The frequency coherence properties of CDL-C models A2 and A4 are different due to the different channel power delay profiles.

Figure 1: Measured spatial PSD at TX (left) and at RX (right) for CDL-C channel.

Figure 2: Measured frequency coherence (FC) of CDL-C A2.

Figure 3: Measured frequency coherence (FC) of CDL-C A4.

Figure 4: Measured time coherence (TC) of CDL-C A2.

Figure 5: Measured time coherence (TC) of CDL-C A4 with 3 km/h.

Figure 6 depicts the spatial PSD of the 4-cluster TDL channel models 1, 2, and 3 specified in the simulation plan [3] and also provided in Tables 1, 2, and 3 of this document. Note that the models were defined to be symmetrical so that the phase shifts at the TX and RX side are the same. Therefore, also the PSD at the TX and the RX sides are identical. In general, multi-cluster TDL models need not be symmetrical. As can be seen, the channels are clearly spatially selective. The spatial selectivity pattern can be tailored to suit any test purpose by properly selecting the cluster-specific phase shifts and powers.

Figure 7 shows the absolute value of FC for the three multi-cluster TDL models. As can be seen, the frequency coherence properties are identical for all the models. This is due to the fact that the channel tap delays and powers are directly inherited from the underlying TDL-C model.

Figure 8 shows the TC for the three multi-cluster TDL models. As can be seen, time coherence is real-valued and follows the shape of Bessel function. This is due to the symmetric Jakes Doppler spectrum inherited from the underlying TDL-C channel.

Observation #3: The spatial selectivity properties of the multi-cluster TDL models depend on the cluster parameter setting.

Observation #4: The frequency and time coherence properties of the simulated multi-cluster TDL-C models follow the properties of the underlying legacy TDL-C model.

Figure 6: Measured spatial PSD of multi-cluster TDL-C channels.

Figure 7: Measured FC (absolute value) of multi-cluster TDL-C channels.

Figure 8: Measured TC (real part) of multi-cluster TDL-C channels. The imaginary part is very close to zero.

# 2.2 CDL channels

In this section, we discuss the fixed PMI selection, simulation results, and the channel model details of CDL channels.

# 2.2.1 Fixed PMI selection in CDL channels

PDSCH ‘Fixed’ PMI Selection

Way Forward:

For alignment a random PMI will be used, for comparison cases with a ‘fixed’ PMI FFS on approach.

We believe that it is impossible to align fixed PMI simulation results if all companies choose different PMIs. Furthermore, selecting some subset of the precoders considerably complicates the test implementation and the agreement process in RAN4. Moreover, it is outside of the scope/context of PDSCH demod testing to evaluate the statistics of PMI selection algorithms. Therefore, we propose that collectively chosen fixed PMI is applied based on companies’ proposals. For 8TX-8RX-8L, our proposal is fixed PMI (4,0,0). One suitable alternative is PMI (6,0,0).

We see two potential reasons to justify applying fixed PMI testing. The first reason is that achieving 100% tput with random precoding and LMMSE demodulation may be challenging. The second reason is that by using a fixed PMI with 8 layers, the CW-specific performance difference can be selected/adjusted as desired. Neither of the reasons are valid in case of 4-layer testing. Therefore, we propose to apply fixed PMI testing to 8TX-8RX-8L only.

Proposal #2: We propose that a collectively chosen fixed PMI, based on companies’ proposals, is applied for fixed PMI testing with 8TX-8RX-8L only.

Proposal #3: For 8TX-8RX-8L demod testing with fixed PMI, we propose PMI (4,0,0) to be used.

# 2.2.2 Simulations/alignment in CDL channels

We have simulated PDSCH demodulation performance with random and fixed PMI. Results are available in separate simulation result document [3]. We can make the following observations.

Observation #5: Demodulation in CDL-C A2/A4 with both AAV options is feasible with 4T4R4L and random precoding.

Observation #6: Demodulation in CDL-C A2/A4 with both AAV options is feasible with 8T8R8L, with both random precoding and fixed Type1 precoding.

Observation #7: Reaching 100% tput in CDL-C A2/A4 with 8T8R8L and random precoding requires relatively high SNR: more than 30dB with A4/AAV1.

Observation #8: AAV option 3 (no AAV) provides improved demod performance compared to AAV option 1, especially in the truncated CDL-C model A4.

Proposal #4: We propose to prioritize testing in the truncated CDL-C channel model A4 with AAV option 3. This is the simplest combination, and it avoids the controversial ray-splitting feature.

# 2.2.3 CDL channel model derivation

In this section, we discuss the steps to derive the new CDL channel model tables, and the sub-cluster issue.

Angles of Departure and Angles of Arrival

Agreement

For PDSCH comparison μϕ,desired =  μϕ,model

For PMI further discussion required.

For TR 38.753, RAN4 to explicitly specify angle wrapping rules.

One observation about the AS scaling procedure for AOA/AOD is that it is affected by how angle wrapping to keep angles between -180 and +180 degrees is applied. This holds especially for AOA where the AS is so large that angle wrapping has effect. No method for angle wrapping is specified in the currently agreed equation:

.

Observation #9: The CDL angle scaling result is affected by how AOA/AOD angles are wrapped (or not wrapped) to stay between -180 and +180. Wrapping rules are not specified in the currently agreed equations.

We propose to explicitly specify how to apply wrapping to avoid ambiguity and different interpretations. Furthermore, we propose to apply the scaling not to the ray-angles, but to the cluster angles that are tabulated. This is because if scaling/wrapping is applied per ray, the rays of one cluster may deviate from each other.

Proposal #5: For CDL angle scaling, we propose to explicitly specify angle wrapping rules for AOA/AOD. Furthermore, we propose to apply scaling to the cluster-angles instead of the ray-angles as shown in equations (1) and (2) below.

ϕn,AOA,scaled=wrapASdesiredASmodel∙wrapϕn,AOA-μϕ,model+μϕ,desired	(1)

CASA,scaled=ASdesiredASmodel∙CASA						(2)

Here, function y = wrap(x) makes y to fall between -180 and +180 degrees so that y = (x+180) mod 360 - 180. Note that this is the scaling/wrapping rule we used to obtain an exact match with the earlier agreed A2 model.

In CDL models, while AOA/AOD fall between [-180, 180] degrees, ZOA/ZOD fall between [0, 180] degrees. Thus, ZOA/ZOD cannot be wrapped, but they can be clipped as specified in TR 38.901 Section 7.7.5.1. However, in the scaling process of CDL-C addressed in this study item, no clipping was needed. In case of heavy AS scaling, clipping could have an effect, but it also probably results in undesired spatial properties. It is up to discussion whether to specify clipping for ZOA/ZOD in TR 38.753.

Observation #10: In CDL models, ZOA/ZOD angles fall between [0, 180] degrees. Thus, ZOA/ZOD cannot be wrapped, but they can be clipped as specified in TR 38.901 Section 7.7.5.1.

Finally, the scaled/wrapped/clipped cluster angles are quantized and tabulated for documentation. We propose to apply rounding the angles to the closest value with the granularity of 0.1 degrees.

Proposal #6: We propose to round CDL cluster angles to the closest value with the granularity of 0.1 degrees.

CDL Table Truncation

Way Forward

As a starting point, a single CDL Table taken forward to include truncation and cluster combining (details included in Annex A). The number of clusters used will be 12.

This will be assessed during RAN4#115 if it can support 8 layers; and if a untruncated option is required.

Truncated CDL Model

Agreement

The configuration table of the truncated CDL model (commonly referred to as ‘A4’) is derived by the procedure detailed in Annex A, and includes truncation, as well as, combining of clusters with equal angular profiles under the earliest delay. The configuration is based on 38.901 CDL link level tables after delay and angle scaling.

The implementation of the truncated CDL model treats all clusters the same, and uses 20 rays per cluster (i.e., no ray splitting or sub-clustering is applied)

Note: Companies welcome to bring results for both truncated and untruncated results.

According to our simulations, in truncated channel model A4, both PDSCH demod and follow-PMI testing performance remain well feasible. Furthermore, according to our analysis, the truncation has a minor effect on the spatial selectivity (PSD) of the channel. Most importantly, using sub-cluster merging helps to avoid the tedious ray-splitting feature, which significantly simplifies the model specification.

Proposal #7: We support the CDL table truncation process via sub-cluster merging and cluster selection, and we endorse model A4 as the test channel.

Proposal #8: We propose not to include untruncated CDL model variants such as A2 in TR 38.753.

However, in case untruncated CDL models (like A2) are still needed as an option in TR 38.753, we believe there are two options to address the sub-cluster ray-splitting problem:

- Merge the sub-clusters (as in A4) so that the number of clusters is reduced from 24 to 20, or

- Explicitly specify how ray-splitting is done in the context of 827-based randomness reduction.

Proposal #9: In case an untruncated CDL model is kept as an option in TR 38.753, merge the sub-clusters (as in A4) so that the number of clusters is reduced from 24 to 20. Alternatively, explicitly specify how ray-splitting is done in the context of 827-based randomness reduction.

Ray Splitting

Way Forward

New approach to CDL truncation has been taken which includes cluster combining, and no ray splitting is required for the truncated CDL.

One feature of the CDL models is so-called sub-clusters that involve cluster-splitting and ray-splitting as specified in Table 7.5-5 of TR 38.901. We would like to define these two terms just to clarify our meaning:

- Cluster-splitting: splitting one strong cluster into three sub-clusters with different delays and powers.

- Ray-splitting: splitting the 20 rays between the three sub-clusters so that the first gets 10, the second 6, and the third gets 4 rays.

The motivation for ray-splitting is to avoid fading correlation between the sub-clusters. There is some disagreement between companies whether ray-splitting needs to be done in the context of link-level CDL models or not. We believe that if ray-splitting is meaningful in system-level models, it is even more important in link-level models. In our view, this detail cannot be left open for interpretation.

Proposal #10: If sub-clusters should remain in CDL models of TR 38.753, specify how ray-splitting is done in the context of 827-based deterministic ray-coupling as shown in Figure 9.

Figure 9: Ray-splitting proposal for sub-clusters with fixed ray angle coupling pattern.

# 2.4 Multi-cluster TDL channels

TDL Model Extensions

Way Forward:

Companies to assess Cluster Models 1, 2, and 3 – focus on option 2 and 1 for alignment, with 4 clusters for all cases, based on companies’ preference.

It is encouraged that all companies assess all options to understand the pros and cons of each option

We have simulated the TDL extensions as detailed in [3], and the results are shown in our simulation results [4]. For a detailed description of all multi-cluster TDL models see [5]. All PDSCH simulation cases turned out to be feasible in the sense that 100% tput can be achieved. According to the simulation results, for 8TX-8RX-8L with fixed precoder (0,0,0), cluster models 1 and 3 provide very similar tput performance, while cluster model 2 requires 5dB higher SNR to achieve 70% total tput.

We provide harmonized table formats to specify TDL cluster models 1, 2 and 3. Examples of the tables for the models specified in [4] are shown below in Tables 1, 2 and 3. We propose to use the new formats in TR 38.753.

Proposal #11: We propose new harmonized table formats for multi-cluster TDL channels to be used in TR 38.753.

The three introduced cluster models are different approaches to specify parameters for the generic TX-RX beam steering functionality. Model 1 does not allow splitting the TDL delay profile into clusters, which is a desired complexity reduction feature. On the other hand, model 2 is limited in the sense that it does not allow multiple underlying TDL models or cluster power adjustments.

Cluster model 3 combines the complexity reduction and flexibility features. We recognize that if we allow a flexible number of underlying TDL channel models (instead of fixing the number to exactly two), cluster model 3 becomes a generic model that includes cluster models 1 and 2 as special cases. Therefore, if so desired, it is possible to reduce the number of cluster models by introducing just one: generalized model 3.

Observation #11: If we allow a flexible number of underlying TDL channel models (instead of fixing the number to exactly two), TDL multi-cluster model 3 becomes a generic model that includes cluster models 1 and 2 as special cases.

Proposal #12: We propose to prioritize cluster model 3 as the most versatile multi-cluster TDL model. Furthermore, we propose RAN4 to consider unifying cluster models 1, 2, and 3 into one generic model in the model description of TR 38.753.

Table 1: Cluster model 1 with TDL-C and 4 clusters

| Delay profile | Cluster index | Power (dB) | θtx1 | θrx1 |
| --- | --- | --- | --- | --- |
| TDLC 300 | 1 | 0 | 0 | 0 |
| TDLC 300 | 2 | 0 | -π/2 | -π/2 |
| TDLC 300 | 3 | -9 | -π | -π |
| TDLC 300 | 4 | -9 | -3π/2 | -3π/2 |

Table 2: Cluster model 2 with TDL-C and 4 clusters

| TDLC 300 delay profile |  |  | TX-RX beam steering |  |  |
| --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |
| 3 | 70 | -7.7 | 2 | -π/2 | -π/2 |
| 4 | 190 | -2.5 |  |  |  |
| 5 | 195 | -2.4 |  |  |  |
| 6 | 200 | -9.9 |  |  |  |
| 7 | 240 | -8.0 |  |  |  |
| 8 | 325 | -6.6 |  |  |  |
| 9 | 520 | -7.1 | 3 | -π | -π |
| 10 | 1045 | -13.0 |  |  |  |
| 11 | 1510 | -14.2 | 4 | -3π/2 | -3π/2 |
| 12 | 2595 | -16.0 |  |  |  |

Table 3: Cluster model 3 with 2 TDL-C channels and 4 clusters

| TDLC 300 delay profile |  |  | TX-RX beam steering |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | TDL channel 1 Power: 0 dB |  |  | TDL channel 2 Power: -9 dB |  |  |
|  |  |  | Cluster index | θtx1 | θrx1 | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 | 3 | -π | -π |
| 2 | 65 | 0 |  |  |  |  |  |  |
| 3 | 70 | -7.7 |  |  |  |  |  |  |
| 4 | 190 | -2.5 |  |  |  |  |  |  |
| 5 | 195 | -2.4 | 2 | -π/2 | -π/2 | 4 | -3π/2 | -3π/2 |
| 6 | 200 | -9.9 |  |  |  |  |  |  |
| 7 | 240 | -8.0 |  |  |  |  |  |  |
| 8 | 325 | -6.6 |  |  |  |  |  |  |
| 9 | 520 | -7.1 |  |  |  |  |  |  |
| 10 | 1045 | -13.0 |  |  |  |  |  |  |
| 11 | 1510 | -14.2 |  |  |  |  |  |  |
| 12 | 2595 | -16.0 |  |  |  |  |  |  |

# Conclusion

In this paper we provided our views on the spatial channel modelling. The following observations and proposals are made:

Proposal #1: We propose the following metrics for alignment between companies and to compare the properties of different channel models:

- Spatial domain: Power Spectral Density (PSD) or Power Angular Distribution (PAD) of the channel in TX and RX dimensions

- Time domain fading: Time Coherence (TC) function measured as channel correlation between symbols

- Frequency domain: Frequency Coherence (FC) function measured as channel correlation between subcarriers

Observation #1: The spatial selectivity and the time coherence (fading) properties of CDL-C models A2 and A4 – with both AAV options 1 and 3 – remain similar.

Observation #2: The frequency coherence properties of CDL-C models A2 and A4 are different due to the different channel power delay profiles.

Observation #3: The spatial selectivity properties of the multi-cluster TDL models depend on the cluster parameter setting.

Observation #4: The frequency and time coherence properties of the simulated multi-cluster TDL-C models follow the properties of the underlying legacy TDL-C model.

Proposal #2: We propose that a collectively chosen fixed PMI, based on companies’ proposals, is applied for fixed PMI testing with 8TX-8RX-8L only.

Proposal #3: For 8TX-8RX-8L demod testing with fixed PMI, we propose PMI (4,0,0) to be used.

Observation #5: Demodulation in CDL-C A2/A4 with both AAV options is feasible with 4T4R4L and random precoding.

Observation #6: Demodulation in CDL-C A2/A4 with both AAV options is feasible with 8T8R8L, with both random precoding and fixed Type1 precoding.

Observation #7: Reaching 100% tput in CDL-C A2/A4 with 8T8R8L and random precoding requires relatively high SNR: more than 30dB with A4/AAV1.

Observation #8: AAV option 3 (no AAV) provides improved demod performance compared to AAV option 1, especially in the truncated CDL-C model A4.

Proposal #4: We propose to prioritize testing in the truncated CDL-C channel model A4 with AAV option 3. This is the simplest combination, and it avoids the controversial ray-splitting feature.

Observation #9: The CDL angle scaling result is affected by how AOA/AOD angles are wrapped (or not wrapped) to stay between -180 and +180. Wrapping rules are not specified in the currently agreed equations.

Proposal #5: For CDL angle scaling, we propose to explicitly specify angle wrapping rules for AOA/AOD. Furthermore, we propose to apply scaling to the cluster-angles instead of the ray-angles as shown in equations (1) and (2) below.

ϕn,AOA,scaled=wrapASdesiredASmodel∙wrapϕn,AOA-μϕ,model+μϕ,desired	(1)

CASA,scaled=ASdesiredASmodel∙CASA						(2)

Observation #10: In CDL models, ZOA/ZOD angles fall between [0, 180] degrees. Thus, ZOA/ZOD cannot be wrapped, but they can be clipped as specified in TR 38.901 Section 7.7.5.1.

Proposal #6: We propose to round CDL cluster angles to the closest value with the granularity of 0.1 degrees.

Proposal #7: We support the CDL table truncation process via sub-cluster merging and cluster selection, and we endorse model A4 as the test channel.

Proposal #8: We propose not to include untruncated CDL model variants such as A2 in TR 38.753.

Proposal #9: In case an untruncated CDL model is kept as an option in TR 38.753, merge the sub-clusters (as in A4) so that the number of clusters is reduced from 24 to 20. Alternatively, explicitly specify how ray-splitting is done in the context of 827-based randomness reduction.

Proposal #10: If sub-clusters should remain in CDL models of TR 38.753, specify how ray-splitting is done in the context of 827-based deterministic ray-coupling as shown in Figure 9.

Figure 9: Ray-splitting proposal for sub-clusters with fixed ray angle coupling pattern.

Proposal #11: We propose new harmonized table formats for multi-cluster TDL channels to be used in TR 38.753.

Observation #11: If we allow a flexible number of underlying TDL channel models (instead of fixing the number to exactly two), TDL multi-cluster model 3 becomes a generic model that includes cluster models 1 and 2 as special cases.

Proposal #12: We propose to prioritize cluster model 3 as the most versatile multi-cluster TDL model. Furthermore, we propose RAN4 to consider unifying cluster models 1, 2, and 3 into one generic model in the model description of TR 38.753.

# Reference

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2504756, “Way Forward for [114bis][320] NR_SCM”, Nokia

- R4-2504762, “Simulation assumptions for SCM”, Huawei, HiSilicon, MediaTek

- R4-2505594, “Simulation results of spatial channel modelling”, MediaTek Inc

- R4-2505595, “TP to TR38.753: TDL approaches and related Annex”, MediaTek