# 3GPP TSG RAN WG4 Meeting#115	R4-2505600

# St. Julians, Malta, May 19 – May 23, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on channel modeling for NTN testing for NGSO

Agenda item:	7.10.5.1

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed issues related to the channel modeling for NTN testing for NGSO [1]. In this paper, we share our perspectives on the remaining open issues.

2. TE Emulated Channel Model

2.1 Consideration of Earth rotation during signal flight time

Consider the Earth’s rotation during signal flight time for more accurate estimations of propagation delay

- Proposals

  - Option 1

ΔtDL=RDLtTc1-1cvUEtT・RDLtTRDLtT

  - Option 2

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

  - Option 4

Δt=DelayERt=ρtRECEFc+vtRECI⋅ρtRECEFρtRECEF=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF

ρtR ECEF=rtRECEF-UEtRECEF

- agreement

  - Consider the earth rotation during signal flight time impact into TE-emulated channel modelling

    - Take the proposals from option 1 or option2/4 for DL propagation delay calculation

    - Companies are encouraged to cross check the equations to derive the total propagation delay and its error.

      - Note: The details of option 1, option 2 and option 4 can be referred as R4-2503710, R4-2503247 and R4-2504344, respectively

Not consider the earth rotation for doppler shift calculation

During the last meeting, RAN4 discussed the consideration of Earth's rotation during signal flight time. There are a few proposals under considerations with minor variations. The proposed way forward is outlined below.

In our view, the mathematical formulations of options 2 and 4 are identical, except that option 2 assumes the velocity in ECEF format and the definition of ρtR ECEF as follows, which results in a sign difference between options 2 and 4.

ρtR ECEF=UEtRECEF- rtRECEF

On the other hand, option 1 seems to have derived the expression in terms of transmit time tT, which, in our view, is not intuitive. Therefore, we suggest proceeding with option 4.

Proposal 1: Choose option 4 to model the Earth's rotation during signal flight time.

2.2 Accuracy requirements for the Doppler and Propagation Delay

Accuracy requirements for maximum delay errors to obtain the propagation delay and Doppler shift trajectory

- agreement

  - The number of iterations to solve Kepler’s equation based on Newton-Raphson method is up to TE implementation

  - The interpolation method for the Doppler shift and Propagation delay trajectory is up to TE implementation

    - Note: the earth rotation during signal flight time impact should be considered to calculate the propagation delay

  - To derive accuracy requirement,

    - Baseline assumption

      - number of iterations is fixed to 4 for solving Kepler’s equation based on Newton-Raphson method

      - Calculate the Propagation delay and Doppler shift every 1ms

    - For accuracy requirement

      - Apply linear interpolation to calculate the propagation delay and Doppler shift with 1ms granularity as baseline, no restrict the implementation on the TE side

      - Consider the propagation delay error is less than TBD (s) and Doppler shift error is less than TBD (Hz)

        - The time period to calculate the error is 1s

    - TE should meet the minimum accuracy requirement defined.

    - Companies are encouraged to provide the acceptable value with associated propagation delay error and Doppler shift error calculation method for accuracy requirement in the next meeting

      - on Newton-Raphson method

      - Calculate the Propagation delay and Doppler shift every 1ms

    - For accuracy requirement

      - Apply linear interpolation to calculate the propagation delay and Doppler shift with 1ms granularity as baseline, no restrict the implementation on the TE side

      - Consider the propagation delay error is less than TBD (s) and Doppler shift error is less than TBD (Hz)

        - The time period to calculate the error is 1s

    - TE should meet the minimum accuracy requirement defined.

    - Companies are encouraged to provide the acceptable value with associated propagation delay error and Doppler shift error calculation method for accuracy requirement in the next meeting

It was agreed in the previous meeting that RAN4 would introduce accuracy requirements for the Keplerian method for the estimation of propagation delay and Doppler shift. The way forward is outlined below.

Based on the agreements presented above, we present our interpolation results between the Keplerian estimate and linear interpolation at 1ms intervals using the L1 norm over a duration of 1 second.

Fig. 1 Propagation delay and Doppler shift error (L1 norm) between Keplerian estimate and linear interpolation at 1ms interval over a duration of 1 second.

Table 1: Propagation delay and Doppler shift error (L1-norm) between Keplerian estimate and linear interpolation at 1ms interval over a duration of 1 second

| Propagation delay error (L1-norm) | Doppler shift error (L1-norm) |
| --- | --- |
| 4.228860190854094e-06 sec | 1.434340210417577e+02 Hz |

The L1 error norm for the propagation delay estimation over a duration of 1 sec with 1ms estimate interval is calculated as follows.

Errordelay1= i=11000Delaykepi-Delaylineari

Where x1is the L1 error norm of x and i is estimation index at 1ms interval over a duration of 1 second. Similarly, the L1 error norm for the Doppler shift estimation over a duration of 1 sec with 1 ms estimate interval is calculated as follows.

ErrorDoppler1= i=11000Dopplerkepi-Dopplerlineari

Proposal 2: Utilize the L1 norm for calculating propagation delay and Doppler accuracy requirements. The L1 error norm for estimating propagation delay and Doppler shift over a duration of 1 second, with a 1ms estimation interval, is calculated as follows.

Errordelay1= i=11000Delaykepi-Delaylineari

ErrorDoppler1= i=11000Dopplerkepi-Dopplerlineari

Where x1is the L1 error norm x and i is estimation index at 1ms interval over a duration of 1 second.

Proposal 3: Consider the following errors for the accuracy requirements in calculating the propagation delay and Doppler shift using the Keplerian method.

Table 1: Propagation delay and Doppler shift error (L1 norm) between Keplerian estimate and linear interpolation at 1ms interval over a duration of 1 second

| Propagation delay error (L1-norm) | Doppler shift error (L1 norm) |
| --- | --- |
| 4.228860190854094e-06 sec | 1.434340210417577e+02 Hz |

3. Conclusions

In this paper, we present our perspectives on the performance requirements for NTN testing for UEs supporting NGSO scenarios. The following proposals have been made.

Proposal 1: Choose option 4 to model the Earth's rotation during signal flight time.

Proposal 2: Utilize the L1 norm for calculating propagation delay and Doppler accuracy requirements. The L1 error norm for estimating propagation delay and Doppler shift over a duration of 1 second, with a 1ms estimation interval, is calculated as follows.

Errordelay1= i=11000Delaykepi-Delaylineari

ErrorDoppler1= i=11000Dopplerkepi-Dopplerlineari

Where x1is the L1 error norm of x and i is estimation index at 1ms interval over a duration of 1 second.

Proposal 3: Consider the following errors for the accuracy requirements in calculating the propagation delay and Doppler shift using the Keplerian method.

Table 1: Propagation delay and Doppler shift error (L1 norm) between Keplerian estimate and linear interpolation at 1ms interval over a duration of 1 second

| Propagation delay error (L1-norm) | Doppler shift error (L1 norm) |
| --- | --- |
| 4.228860190854094e-06 sec | 1.434340210417577e+02 Hz |

4. References

[1] R4-2502314, “WF for NTN testing for NGSO channel model,” 3GPP TSG RAN WG4 Meeting #114, Wuhan, CN, Apr 7 – 11, 2025.