# 3GPP TSG RAN WG4 Meeting#115	R4-2505601

# St. Julians, Malta, May 19 – May 23, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for NTN testing for NGSO

Agenda item:	7.10.5.3

Document for:	Discussion

# 1. Introduction

RAN4 is nearing the completion of channel modeling for NTN testing for NGSO. Consequently, it is essential to start discussions on demodulation requirements. Several companies have proposed methods to address the demodulation requirements for NGSO testing, considering time-varying Doppler and propagation delay [1]. This paper presents our perspectives on how to approach the RAN4 demodulation requirements for this WI.

2. Demodulation Requirements

2.1 Impact on RAN4 demodulation requirements

Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models

- Proposals:

  - Option 1:

    - Keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

    - Keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

  - Option 2: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

  - Option 3:

    - RAN4 shall start to agree parameters for using the NGSO channel model in order to understand whether the margin produced is acceptable to define requirements with.

    - RAN4 shall use the new NGSO channel model for all performance requirements, in it’s current state.

  - Option 4: After concluding discussions on the time-varying Doppler shift and propagation delay, initiate simulation campaigns to analyze the impact on demodulation requirements.

- Recommended WF:

  - Further discussion

There were a few proposals submitted during RAN4#114-bis to initiate the discussions on the impact of the RAN4 demodulation requirements. The way forward is outlined below.

With the TE-emulated channel model nearing completion, RAN4 can commence simulation campaigns to evaluate the impact of the TE-emulated time-varying Doppler shift and propagation delay on demodulation requirements.

Proposal 1: Launch simulation campaigns to assess the impact on demodulation requirements from time-varying propagation delay and Doppler shift. Companies are encouraged to submit their simulation results during RAN#116.

3. Conclusions

In this paper, we present our perspectives on the demodulation performance requirements for NTN testing for UEs supporting NGSO scenarios. The following proposals have been made.

Proposal 1: Launch simulation campaigns to assess the impact on demodulation requirements from time-varying propagation delay and Doppler shift. Companies are encouraged to submit their simulation results during RAN#116.

4. References

[1] R4-2502314, “WF for NTN testing for NGSO channel model,” 3GPP TSG RAN WG4 Meeting #114, Wuhan, CN, Apr 7 – 11, 2025.