3GPP TSG-RAN WG4 Meeting #115	R4-2505602

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.21.2

Source:	CAICT

Title:	Discussion on general aspects of AI/ML for NR air interface

Document for:	Discussion

1	Introduction

In this contribution, we discuss following issues from last meeting:

- Issue 1-3-2: Post deployment options

- Issue 1-1-3: Delay requirements for functionality activation/switching/fallback

2	Discussion

Issue 1-3-2: Post-deployment options

In RAN4 #114, a Way Forward [1] is drafted for next step discussion, in which “further potential clarification” and “significant issues” parts were raised for each option. Accordingly, we provide our initial thinkings on the discussion points of option 2 (Post deployment management based on LCM):

| Further potential clarification | - Whether the metric used for performance testing and the metric used for monitoring can be aligned. - Whether monitoring used for post-deployment testing should be NW sided only. |
| --- | --- |
| Significant issues | - Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) - Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs. |

- Whether the metric used for performance testing and the metric used for monitoring can be aligned: First, the definition of “the metric used for monitoring” requires clarification. If it refers to the metrics for monitoring tests in RAN4, the issue shall be left pending for future discussion on LCM monitoring test, if any. However, if it refers to the specified monitoring metrics, this needs to be discussed per use case basis.

For beam management, RAN1 had agreed on the definition of performance metric of Top1 or Top K beam prediction accuracy for UE-assisted performance monitoring.

Agreement (RAN1 #120bis) For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling

RAN4 had agreed on the metrics/KPIs for beam ID prediction

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

In our view, the two metrics cannot be used interchangeably. The RAN1 metric focuses on ensuring that the predicted beam set of a single instance covers at least one of the M best-measured beams. In contrast, the RAN4 metric focuses on ensuring that the actual best beam within the predicted beam set of a single instance has an RSRP no lower than a certain threshold.

Regarding to CSI prediction, RAN4 agreed to take the ratio γ as test metric while RAN1 agreed to take intermediate KPI as monitoring metric. These two metrics are not aligned.

Observation 1: Based on current progress in two WGs, the metric used for performance testing in RAN4 and the metric used for monitoring specified in RAN1 are not aligned for beam management and CSI prediction.

- Whether monitoring used for post-deployment testing should be NW sided only: Our understanding is, it does not need to be limited to NW-side. For the case of CSI prediction, monitoring on UE side is more straightforward since UE owns the measured CSI information.

- Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.): In our understanding, performance monitoring is based on measurements. During measurement, UE would keep recording some statistics information on metrics, such as, channel quality, noise level, delay spread and doppler spread, along with performance monitoring. These statistics information can be used to identify the root cause of performance degradation once being discovered in performance monitoring. The details could be up to inner implementation and optimization.

- Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs: RAN4 needs to define requirements for LCM monitoring, at least including delay requirements of monitoring reporting and model switching/fallback.

Regarding to option 1 (Pre-activation functionality/model update testing), the concerns are two-fold. Firstly, conducting conformance tests/validation for all potential models or updates would induce significant testing workload, of which the feasibility and necessity are both questionable. If we differentiate so-called major and minor update, the criteria would require more discussion. Clear rules should be specified to help effectively planning and managing tests. Secondly, if pre-activation tests are conducted in lab environment, it is nearly the same as conformance test, unless field data could be introduced to verify the performance. If it means verifying the model through real operation in the field, the procedure is similar as performance monitoring and LCM.

Proposal 1: Prioritize option 2 (performance monitoring and LCM) for post-deployment handling.

Issue 1-1-3: Delay requirements for functionality activation / switching / fallback

In last RAN4 meeting, we had following agreements on this issue.

Delay requirements for activation/switching/fallback Agreement: - FFS on whether the deactivation requirements will be introduced - The starting point is when the UE receives the signalling to commence deactivation - FFS when the endpoint for deactivation is - FFS whether fallback delay is the same as deactivation delay or has some other components

Regarding the FFS in the main bullet, we think it is necessary to introduce deactivation requirement, particularly for fallback case. When a functionality becomes non-applicable on UE-side, UE would report this to network. Upon receiving the report, the network is expected to deactivate the functionality by sending deactivation command. Once the UE receives the deactivation command, it returns to non-AI/ML operation. In our view, the process should be completed as soon as possible. Hence, a deactivation delay requirement is necessary for this case.

As for the end point of deactivation, there has been discussion among companies about the need for clarity. In our understanding, the endpoint should explicitly indicate that UE has terminated precedent configuration and is proceeding with new configuration. There are two possible options that could be discussed:

- Option 1: “UE shall be able to perform L1-RSRP measurements of configured CSI-RS and/or SSB in set A”.

- Option 2: “UE shall send L1-RSRP reports for report configuration configured for set A”.

Proposal 2: Suggest to introduce delay requirements, for at least fallback case.

Conclusion

In this contribution, the issue of post-deployment and delay requirement are discussed with following proposals and observations:

Observation 1: Based on current progress in two WGs, the metric used for performance testing in RAN4 and the metric used for monitoring specified in RAN1 are not aligned for beam management and CSI prediction.

Proposal 1: Prioritize option 2 (performance monitoring and LCM) for post-deployment handling.

Proposal 2: Suggest to introduce delay requirements, for at least fallback.

Reference

[1] R4-2502953, WF on AI/ML air interface study, Moderator (Ericsson)