3GPP TSG-RAN WG4 Meeting #115	R4-2505605

# St Julian’s, Malta, 19th - 23rd, May 2025

Source: 	SK Telecom

Title: 	Discussion on HPUE inter-band UL CA and EN-DC

Agenda Item:	7.1.2.1.3

Document for:	Discussion

- Introduction

During Rel-19, 3Tx for handheld UE has been discussed over several meetings; however, how to capture 3Tx for handheld UE in the specification has not yet been addressed. Hence, this contribution presents our views on the remaining issues related to inter-band UL CA with 2Tx and/or 3Tx, focusing on the revised aspects of the specification required to support the 3Tx feature for handheld UE.

- 2. Discussion

The Work Item (WI) “UE RF enhancements for NR FR1/FR2 and EN-DC” includes 3Tx handheld UE, which should be clearly specified in the specification. However, in the current specification, there are certain expressions that limit 3Tx operation to Fixed Wireless Access (FWA), which should be modified or removed. Below are examples of such expressions:

Section 4.3 of TS 38.101-1 [1] For a terminal that supports inter-band Carrier Aggregation (CA) with UL MIMO or Tx diversity operation, the requirements are targeted for FWA form factor in current version of specification.

Section 4.2 of TS 38.101-3 [2] For a terminal that supports inter-band Dual-Connectivity (DC) with UL MIMO or Tx diversity operation, the requirements are targeted for FWA form factor in current specification.

Proposal 1: Expressions that limit 3Tx operation to FWA should be modified or removed throughout the specification.

In addition, during the RAN#113 meeting [3], RAN4 agreed to re-organize the 2Tx/3Tx notes, as redundancies have emerged in the descriptions with the evolution of the specification, as outlined below:

2.2 Re-organize 2Tx/3Tx Notes in clause 5 and clause 6 Agreement: - Companies are encouraged to consider the necessity and possibility to re-organize the Notes in future meetings

We agree with the motivation for re-organizing the 2Tx/3Tx notes; however, the WI scope, which includes 3Tx for handheld UEs, should be properly reflected. For example, removing Note 7 in Table 6.2H.1.3-1 and Table 6.2L.1.3-1 of TS 38.101-3 could be one way to address this.

Table 6.2H.1.3-1: Maximum output power for inter-band EN-DC with UL MIMO (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_3A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_7A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_8A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_20A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_28A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_40A_n78A | 26 | +2/-3 | 23 | +2/-3 |
| DC_41A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with UL MIMO in NR band NOTE 6: VoidNOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

Table 6.2L.1.3-1: Maximum output power for inter-band EN-DC with Tx Diversity (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_3A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_7A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_8A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_20A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_28A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_40A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_41A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with Tx Diversity in NR band NOTE 6: Void NOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

Proposal 2: 3Tx for handheld UE should be properly reflected in the note reorganization.

- 3. Conclusion

Proposal 1: Expressions that limit 3Tx operation to FWA should be modified or removed throughout the specification.

Proposal 2: 3Tx for handheld UE should be properly reflected in the note reorganization.

- 4. Reference

[1] 3GPP TS 38.101-1 V19.1.0, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[2] 3GPP TS 38.101-3 V19.1.0, User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

[3] R4-2420329, WF on HPUE RF requirements, Samsung, RAN4#113