# 3GPP TSG RAN WG4 Meeting#115	R4-2505606

# St. Julians, Malta, May 19 – May 23, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for NR-NTN with bandwidth less than 5MHz

Agenda item:	7.10.4.6

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed whether to introduce demodulation requirements for NR with bandwidth less than 5MHz to enable use cases in spectrum-constrained scenarios. It was decided that RAN4 will not introduce any requirements for PDSCH, PBCH, or CSI reporting, while the PDCCH requirements will be subject to further discussion [1]. In this paper, we present our additional perspectives on the scope of PDCCH demodulation requirements for this WI.

2. Scope of Demodulation Performance Requirements

#### PDCCH – Scope of the UE demodulation requirements

Way forward:

- Option 1: No new PDCCH requirements

- Option 2: Discuss the feasibility of introducing PDCCH requirements

During the last meeting, RAN4 discussed whether to introduce PDCCH requirements. The corresponding way forward is outlined below.

Examining the demodulation performance requirements for NR with bandwidth less than 5MHz, it is noted that requirements for 3MHz bandwidth were introduced. Consequently, both PBCH and PDCCH requirements were defined for NR operating with 3MHz bandwidth [2]. Additionally, it is recommended in the WID to develop this feature for NR-NTN based on the work done for NR support in dedicated spectrum less than 5MHz. The corresponding UE capability for this NR feature is provided below.

Based on the UE capability for this feature, PBCH will consist of 12 PRBs, while CORESET0 will consist of 15 PRBs.

Observation 1: RAN4 has introduced PBCH and PDCCH demodulation requirements for NR with bandwidth less than 5MHz in FR1 FDD.

Observation 2: According to the UE capability for this feature, CORESET0 will include 15 PRBs within a 3MHz 15 PRB BWP.

Proposal 1: Consider only 3MHz channel bandwidth with 15 PRB BWP for this WI.

Although NR has introduced PDCCH requirements, these requirements may not be sufficient to ensure PDCCH performance in the NR-NTN scenario due to significant differences in channel models between NR and NR-NTN. Therefore, it may be necessary to investigate the feasibility of introducing PDCCH requirements for NR-NTN with bandwidths less than 5MHz.

Proposal 2: Discuss the feasibility of introducing PDCCH requirements for NR-NTN in FR1 FDD with 15 PRBs for PDCCH CORESET0.

3. Conclusions

In this paper, we present our perspectives on the PDCCH demodulation performance requirements for NR-NTN with bandwidth less than 5MHz in FR1. The following proposals have been made.

Observation 1: RAN4 has introduced PBCH and PDCCH demodulation requirements for NR with bandwidth less than 5MHz in FR1 FDD.

Observation 2: According to the UE capability for this feature, CORESET0 will include 15 PRBs within a 3MHz 15 PRB BWP.

Proposal 1: Consider only 3MHz channel bandwidth with 15 PRB BWP for this WI.

Proposal 2: Discuss the feasibility of introducing PDCCH requirements for NR-NTN in FR1 FDD with 15 PRBs for PDCCH CORESET0.

4. References

[1] R4-2504753, “WF for NR IoT NTN less than 5MHz demod,” 3GPP TSG RAN WG4 Meeting #114-bis, Wuhan, China, April 7 – 11, 2025.

[2] TS 38.101-4, ‘User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)’, v18.6.0, Jan 2025.