# 3GPP TSG-RAN WG4 Meeting #115	R4-2505616

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.26.3.1

Source: 	CMCC

Title: 	Discussion on A-IoT BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements for Type 1-C Ambient-IOT BS.

- Discussion

2.1 A-IoT BS Tx requirement

The conclusion on BS Tx requirements in study item are captured in following table

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES |
| Output power dynamic | RE power control dynamic range | TBD in WI phase |
|  | Total power dynamic range | TBD in WI phase |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | TBD in WI phase |
|  | Transmitter transient period | TBD in WI phase |
| Transmit signal quality | Frequency error | YES |
|  | Some requirements like EVM (Modulation quality) | YES |
|  | TAE | NA |
| Unwanted emissions | Occupied bandwidth | YES |
|  | ACLR | YES |
|  | IBE | NO |
|  | Operating band unwanted emissions | YES |
|  | Transmitter spurious emissions | YES |
| Transmitter intermodulation |  | TBD in WI phase |

- MCL definition

Legacy medium range BS use 53dB MCL assumption with minimum 5m horizontal distance between BS and UE. But during the simulation, the minimum distance is assumed as 1m. this MCL will impact max input power level definition of device and is deployment related. It’s suggested to take care of the definition of this MCL for micro cell scenario and taken the device allowed max input power level and deployment scenario into consideration.

Observation 1: Take care of the definition of MCL for micro cell scenario if we finally use less than 5m horizontal distance for device max input power definition.

- OFF power requirements

Last meeting approve to define OFF power requirements. One motivation of OFF power is to avoid the interference from closed reader transmitter to reader receiver, especially when the received SNR is much lower. From coverage point of view, the R2D is bottleneck compared with D2R, so there is still margin at D2R receiver and the impact of OFF power interference could be negligible. Therefore, there is no need to define more strict requirements than legacy BS requirements.

Proposal 1: for OFF power, no need to define more strict requirements than legacy BS.

- Tx signal quality related requirement

For signal quality related requirements, gNB spec define EVM requirement which is used to evaluate phase distance. A-IOT reader only supports OOK-4 modulation. So current gNB quality related requirement is not applicable anymore. RFID[1] use three types ASK modulation types, i.e. DSB-ASK, SSB-ASK or PR-ASK and define RF envelop requirement including modulation depth, RF envelop ripple, RF pluswidth etc which can be referred for OOK modulation. Details illustration of RFID RF envelop requirement is listed as below for information.

In theory, The baseband signal for OOK-4 is much like that of the ASK signal. But now RAN1 has introduced the CP for OOK-4 to achieve orthogonal with NR sub-carrier which will impact the modulation signal quality. So relaxation is required for ripple requirements. Besides, we should consider the tradeoff between ripple and ACLR. So, it seems reasonable to allow certain relaxation based on RFID requirements.

Observation 2: certain relaxation should be allowed compared with RFID ripple requirements when considering following factors:

- the tradoff between ACLR and ripple

- CP at reader signal.

- Unwanted emission requirements

For unwanted emission requirements, legacy NB-IOT requirements are used for co-existence evaluation in SI. The conclusion of evaluation shows that legacy requirements can fulfil the co-existence when NR macro BS is outdoor and A-IOT reader is indoor. One scenario that causes severe interference is when legacy NR UE is indoor, which require some existing scheduling mechanism to avoid adjacent interference or enhanced mechanism to mitigate interference (as captured in TR 38.769 clause 6.6.5.10). Consider enhanced mechanism to mitigate interference is not within Rel-19 WI scope, then legacy NB-IOT requirements can be reused for A-IOT reader.

For NB-IoT, 100kHz offset is defined. OOK4 modulation scheme has worse out of band spectrum characteristics. In theory, reader will require larger offset for the same requirements. In last meeting, some companies propose to allow relaxation based on NB requirements with the same 100kHz offset. For future, reader FDM operation may be supported for other devices, e.g. 2a, 2b or 2c. for forward compatible, it’s still suggested to use the same NB-IoT requriements and wider offset is allowed.

Proposal 2: it’s suggested to reuse legacy NB-IoT ACLR unwanted emission requirements with wider offset to consider forward compatible.

- Tx intermodulation requirements

According to the assumption, there is no co-located indoor 900MHz BS, therefore, no need for Tx intermodulation requirements.

Proposal 3: no need for Tx intermodulation requirements.

2.2 A-IoT BS Rx requirement

The conclusion on BS Rx requirements in study item are captured in following table

| A-IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | YES |
| Dynamic range |  | TBD in WI phase |
| Adjacent Channel Selectivity |  | YES |
| In-band blocking | general blocking | YES |
|  | narrowband blocking | YES |
| Out-of-band blocking |  | YES |
| Receiver spurious emission |  | YES |
| Receiver intermodulation | General intermodulation | Wait for RAN1 conclusion. |
|  | Narrowband intermodulation | YES |
| In-channel selectivity |  | TBD in WI phase |

- Reference Sensitivity

Based on RAN1 link budget analysis as below, R2D link is bottleneck for D1T1-B.

Table D1T1-96bit-10%BLER-6kbps-BPSK-6dBi BS antenna gain

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D1T1-B | 1-4 | 67.1 | 84.54 | 67.10 | 36.59 |

But it’s noted, RAN1 use 33dBm as reader max output power, but based on RAN4 latest agreements, reader output power could be increased to 38dBm per antenna connector. If the assume 2Tx of reader, then the coverage margin between R2D and D2R link could be negligible (from 7.44 to -0.56dB). besides, if CW is distant from device, then the D2R link will be the coverage bottleneck. Therefore, REFSENSE is significant.

Following list our simulation results and related parameters

|  | BLER |  |
| --- | --- | --- |
| SNR | Variable SFO | Fixed SFO |
| -11 | 13% | 23% |
| -10.8 | 10% | 19% |
| -10.6 | 10% | 17% |
| -10.4 | 9% | 9% |
| -10.2 | 7% | 9% |
| -10 | 6% | 6% |

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 2 |
| Reference data rate | 1 kbps (M) |
| Message size | 96 bits |
| BLER target | 10% |
| Sampling frequency（SFO） | Randomly select a value from the range of [104 ppm~105 ppm] for device 1 |
| [OOK/BPSK chip rate] | 7.5 kcps for 2SB 15 kHz Transmission bandwidth |
| Receiver bandwidth | the same as transmission bandwidth. |
| Waveform (CW) | unmodulated single tone |
| Modulation | OOK |
| Line code | Manchester encoding |
| FEC | 1/3 Convolutional Code |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |
| Repetition | None |
| Preamble configuration | len131=[-1 -1 -1 -1 -1 -1 1 1 1 -1 -1 -1 -1 -1 -1 1 1 1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 1 1 1 -1 -1 1 1 1 -1 1 1 1 -1 -1 1 1 1 -1 -1 -1 1 1 1 1 -1 -1 1 1 1 -1 -1 -1 1 -1 -1 1 -1 -1 -1 1 1 1 -1 1 1 -1 1 1 -1 -1 1 -1 1 1 -1 1 1 -1 1 1 -1 -1 1 -1 -1 -1 1 -1 -1 1 1 -1 1 1 -1 1 -1 1 -1 1 -1 1 -1 1 1 -1 1 1 -1 1 -1 1 -1 -1 1 -1 -1 1 -1 1 -1 1]; |
| Midamble configuration | len131=[-1 -1 -1 -1 -1 -1 1 1 1 -1 -1 -1 -1 -1 -1 1 1 1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 1 1 1 -1 -1 1 1 1 -1 1 1 1 -1 -1 1 1 1 -1 -1 -1 1 1 1 1 -1 -1 1 1 1 -1 -1 -1 1 -1 -1 1 -1 -1 -1 1 1 1 -1 1 1 -1 1 1 -1 -1 1 -1 1 1 -1 1 1 -1 1 1 -1 -1 1 -1 -1 -1 1 -1 -1 1 1 -1 1 1 -1 1 -1 1 -1 1 -1 1 -1 1 1 -1 1 1 -1 1 -1 1 -1 -1 1 -1 -1 1 -1 1 -1 1]; |

Observation 3: the SNR that meet 10% BLER is -10.8 ~-10.6dB with corresponding assumptions as in above table. Note above 131 bit preamble is used and we will update final simulation results based on final RAN1 conclusion.

Now there is no RAN1 requirements about detailed preamble and midable configuration, we will update simulation results in next meeting based on final RAN1 conclusion.

- ACS and blocking related requirements

If there is no indoor UE, then the ACS and blocking seems not that significant. But it’s relatively easier for BS to achieve ACS requirements using digital filter. There is no need to define much relaxed requirements since BS could easily achieve it.

Proposal 4: For ACS requirements, reuse legacy NB-IOT requirements.

- Inter-modulation related requirements

Inter-modulation requirements is used to evaluate receiver non-linearity requirements. Besides, there is NB devices using band n8, it’s suggested to define the narrow-band inter-modulation requirements and reuse FR1 NB-IoT medium range BS Rx narrowband intermodulation requirement.

Proposal 5: it’s suggested to reuse FR1 NB-IoT medium range BS Rx narrowband intermodulation requirement.

# 3. Conclusions

In this contribution, RF requirements for gNB like reader are discussed with following observations and proposals.

Observation 1: Take care of the definition of MCL for micro cell scenario if we finally use less than 5m horizontal distance for device max input power definition.

Proposal 1: for OFF power, no need to define more strict requirements than legacy BS.

Observation 2: certain relaxation should be allowed compared with RFID ripple requirements when considering following factors:

- the tradoff between ACLR and ripple

- CP at reader signal.

Proposal 2: it’s suggested to reuse legacy NB-IoT ACLR unwanted emission requirements with wider offset to consider forward compatible.

Proposal 3: no need for Tx intermodulation requirements.

Observation 3: the SNR that meet 10% BLER is -10.8 ~-10.6dB with corresponding assumptions as in above table. Note above 131 bit preamble is used and we will update final simulation results based on final RAN1 conclusion.

Proposal 4: For ACS requirements, reuse legacy NB-IOT requirements.

Proposal 5: it’s suggested to reuse FR1 NB-IoT medium range BS Rx narrowband intermodulation requirement.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-77AH-IoT-0018, On BS emission requirement in standalone operation for NB-IoT, Huawei

- R4-160610, On BS emission mask in stand-alone operation for NB-IoT, Huawei

- R4-161289, Way forward on BS unwanted emission requirement for NB-IoT

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz - 960 MHz, Version 2.0.0 Ratified

# 2 Text Proposal

##### ---Start of changes---

## 5.3	BS channel bandwidth

### 5.3.1	General

The reader channel bandwidth supports a single reader RF carrier in R2D link at the reader. [The placement of the UE channel bandwidth is flexible but can only be partially within the reader channel bandwidth].

The relationship between the channel bandwidth, the guardband and the transmission bandwidth configuration is shown in figure 5.3.1-1.

.

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one reader channel

### 5.3.2	Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each reader channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 .

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 200 kHz | 400 kHz | 600 kHz | 800 kHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 15 | 1 | 2 | 3 | 4 |

NOTE:	All reader Tx and device Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.2-1.

### 5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each reader channel bandwidth and SCS is specified in table 5.3.3-1.

Table 5.3.3-1: Minimum guardband (kHz) (FR1)

| SCS (kHz) | 200 kHz | 400 kHz | 600 kHz | 800 kHz |
| --- | --- | --- | --- | --- |
| 15 | 10 | 20 | 30 | 40 |

The number of RBs configured in any reader channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.3-1: reader PRB utilization

### 5.3.4	RB alignment

For each reader channel bandwidth, BS transmission bandwidth configuration must fulfil the minimum guardband requirement specified in clause 5.3.3.

[For each numerology, its common resource blocks are specified in clause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology.]

### 5.3.5	BS channel bandwidth per operating band

The requirements in this specification apply to the combination of BS channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1. The transmission bandwidth configuration in table 5.3.2-1 shall be supported for each of the BS channel bandwidths within the BS capability. The BS channel bandwidths are specified for the Tx path.

Table 5.3.5-1: Reader channel bandwidths and SCS per operating band

| NR Band | SCS (kHz) | Reader channel bandwidth (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 200 | 400 | 600 | 800 |
| n8 | 15 | 200 | 400 | 600 | 800 |

##### ---End of changes---