3GPP TSG RAN WG4 Meeting #115	R4-2505657

Malta, 19 May – 23 May, 2025

Agenda item:			7.18.3.2

Source:	vivo

Title:	On remaining issues on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

- Discussion

- At RAN4 114bis meeting, the following agreements are available for this WI:

Issue 1-1: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

Agreement:

- the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios.

  - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC

  - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only

Issue 1-3-1: whether solution 3 includes NR-DC scenario or not?

Agreement in online session:

- Solution 3 includes NR-DC of FR1 and FR1 + FR2

Issue 1-3-3: detailed CSSF table design for solution 3

Agreement in online session:

- This requirement applies regardless of whether measurement gap is configured or not.

Issue 1-4-1: UE capability for CSSF enhancement

Agreement:

- UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR.

- UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

- In this contribution we discuss the following open issues:

Issue 1-2-1: revision on solution 1 (measure one serving CC per band)

- Proposal 1 (OPPO): For solution 1, remove the sub-case and update the agreements as follows:

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Proposal 2 (OPPO): RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update.

- Recommended WF

  - [Moderator]: check if proposal 1 is agreeable and proposal 2 can be dropped.

  - Discuss proposal 1 and proposal 2.

The potential issue with the sentence “if there is no SCCs on which UE is configured to report SSB based measurements in band” in the LS is whether there is any help if a MO is configured without reporting, if it is possible. Anyway if this happen, the configure MO without reporting also indicates the measurement on that MO is not important from NW perspective. From this point it is ok to remove this sentence from the LS.

Proposal 1: For revision on solution 1, ok with proposal 1.

Issue 1-2-2: Expand solution 1 to FR1 case

- Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band.

  - Option 1a (Nokia):

    - Expand the solution 1 to at least FR1 intra-band contiguous CA.

    - Details:

      - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

      - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA.

- Option 2 (Huawei): Not to expand solution 1 to FR1 band.

- Recommended WF

  - [Moderator]: check if following option can be a compromise.

    - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

Regarding whether to extend solution 1 to FR1 case, out initially understanding is solution 1 is more suitable for FR2 scenario. For FR1, for the intra-band contiguous CA case, the interference experienced by different SCC could be similar to some extent hence the recommended WF is ok for a compromise.

Proposal 2: Regarding whether to expand solution 1 to FR1 case, ok with recommended WF.

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC)

[Way forward]:

- FFS:

  - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

    - Note: 3-searchers will be used by UE if UE report this capability.

- FFS: Other restriction (triggered by Huawei):

  - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple)

  - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple)

  - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT)

    - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

Regarding the following FFS part:

- FFS:

  - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

    - Note: 3-searchers will be used by UE if UE report this capability.

The proposal in the FFS part is preferred.

Proposal 3: Suggest to confirm that “Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.”

Regarding the constraint, the reason where 3 searchers are supported by default is for some case, for example when a gap in one FR is overlapped with the SMTC of the other FR, based on legacy specs 3 searchers need be used as in the following figure in [RP-233209]. The constraint, to our understanding, is to avoid the same situation where when the specs are based on 3 searchers, for some cases 4 searcher should be used.

Figure 1 Illustration of the 3 searcher issue

For this intention the intention of option 3 is satisfy the intention of the constraint however the wording “legacy requirement (for example 2-searcher requirement)” is not clear. The limitation of option 1 is strong however can solve the issue.

Proposal 4: For the constraint on 3 searchers, support option 1.

- Summary

In this contribution, we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode. Based on above analysis, following observations and proposals are present.

Proposal 1: For revision on solution 1, ok with proposal 1.

Proposal 2: Regarding whether to expand solution 1 to FR1 case, ok with recommended WF.

Proposal 3: Suggest to confirm that “Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.”

Proposal 4: For the constraint on 3 searchers, support option 1.

- References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RANP 105

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN4 114bis, Apple