3GPP TSG RAN WG4 Meeting #115	   R4-2505658

Malta, 19 May – 23 May, 2025

Agenda item:			7.18.3.1

Source:	vivo

Title:	Remaining issues on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

- Discussion

At RAN4 114bis meeting, the following agreements are available:

Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

Agreement in RRM session (Monday):

    - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell.

    - Leave the decision to RAN2. Further discuss the details for the LS to RAN2, and the LS can only be approved if the content is agreeable.

      - Details of the LS can refer to R4-2504962.

Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Agreement in the ad-hoc:

- Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4.

Issue 1-2-3: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

Agreement in the ad-hoc:

  - For each scenario for PC3 UE, the described factors in the table should be replaced with capability value.

In this contribution we discuss the following open issues:

- Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

In the last meeting, following issues are captured in the WF

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”:

- Proposals for scenario 1/2/3/4

  - Option 1: UE autonomous quitting.

    - Option 1a (vivo, LGE, HW, Intel, Samsung, Ericsson):

      - UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results.

      - (vivo): If there is no consensus, it is preferred to only consider threshold conditions as the exit conditions.

    - Option 1b (LGE): RAN4 to leave it to UE implementation whether to keep or re-active FR2 FBS when serving cell’s signal quality is lower than the threshold configured by network and after quit conditions being discussed have been met.

    - Option 1c (HW):

      - UE is allowed to fall back to normal measurement when UE has performed fast measurement for at least T1.

      - After quitting fast measurement, UE shall re-activate fast measurement no longer than T2 from the quitting time.

    - Option 1d (Intel):

      - The reduced beam sweeping factor state continues to apply until the UE reports the measurement results to the network. And the UE makes another exam on the criteria that are configured for the serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the quality criteria are no longer met.

    - Option 1e (Samsung):

      - Define the maximum period of time of UE to continue BSF measuring after it has reported the qualified measurement result as a timer.

    - Option 1f (Ericsson):

      - UE may quit FBS if the UE has completed at least one measurement in the FBS way and the UE identifies that there isn’t any urgent task to continue FBS, e.g., the UE remains a stationary state or no qualified neighbour cells are available for switching.

  - Option 2: UE indication to the NW is used to deactivate FBS based L3 measurements.

    - Option 2a (OPPO): The detailed signaling can leave to RAN2 decision.

    - Option 2b (LGE): Reuse Rel-18 UAI ‘multiRx-PreferenceFR2’.

    - Option 2c (CMCC): Introduce UAI similar as “multiRx-PreferenceFR2” to inform network there is power consumption issue.

    - Option 2d (CT, Samsung, MTK): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction.

    - Option 2e (Apple): To consider UE power consumption or overheating, UAI mechanism should be used. Either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed.

      - To address the power consumption issues in the two cases (Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving; Case 2: UE is still in the cell centre and the triggering conditions are met due to block.), it is proposed to consider the following two options:

        - Option 1: No specific action is taken, assuming UE can use UAI to inform the network that it needs to fall back to Non-FBS mode (i.e., deactivate FBS operation) if power consumption/overheating becomes an issue.

        - Option 2: After the UE sends a qualified measurement report, the UE can start a timer, and when the timer expires, the UE exits from FBS operation.

  - Option 3: Timer-based solution to deactivate Rx BSF reduction.

    - Option 3a (CATT)

      - UE deactivates Rx BSF reduction feature based on timer-solution, i.e., when either of the conditions are met:

        - Serving cell quality threshold condition is still met, but timer expires.

        - While timer is still running, but serving cell quality threshold condition is no long met.

      - Introduce another timer, and UE re-evaluates the serving cell quality threshold condition after it expires. This timer starts to run when UE deactivates Rx BSF reduction feature.

    - Option 3b: (Ericsson)

      - Instead of TTT, RAN4 to introduce ‘time to deactivate’ parameter, which limits the minimal FBS measurement time after activated/enabled.

      - UE may quit FBS in certain case with or without other conditions, but at least after the minimal time period ‘time to deactivate’.

- Proposals for scenario 5/7

  - Option 4 (OPPO):

    - For scenario 5/7, UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection.

For the conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF, it has already been agreed that “UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4”, hence for scenario 1/2/3/4, the key point of the discussion is whether to have extra conditions to allow leave fast measurement mode for scenarios where threshold based condition alone cannot work efficiently. To our understanding the FBS will happen when the cell quality is not so good and the agreed exit method based on threshold will take care of most of scenarios hence the extra exit conditions discussed here is an further optimization.

Comparing option 1 - autonomous quitting and option 2 - UE indication to the NW, option 1 is preferred since even for threshold based exit method, the NW does not get a notification regarding whether the FBS is deactivated or not. In addition, the target of FBS is to provide quick feedback on measurement to NW hence a UE can autonomously leave FBS mode for scenarios not covered by threshold based exit method after reporting qualified measurement results. The logic is without reporting qualified measurement results, even the UE can leave FBS mode in time to avoid overheating or extra power consumption etc., the purpose of the FBS is not fulfilled without a qualified measurement reporting. Therefore option 1a is preferred.

- Proposal 1: For the conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF , for scenario 1/2/3/4, option 1a is supported.

- Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-2-4: Other aspects of UE capability design

- Proposals

  - Option 1: (QC)

    - RAN4 introduces per-band UE capability for FR2 RX beam sweeping factor optimization and L3 measurement delay reduction.

  - Option 2: (HW)

    - Define separate UE capability for intra-frequency and inter-frequency.

  - Option 3: (Ericsson)

    - We recommend initially limiting the consideration of FBS to a single FR2 serving cell and excluding the scenario of multiple FR2 serving cells from the BSF capability assessment at the first step.

For the UE capability, UE implement flexibility should be considered which could either be addressed by introducing more flexible UE capability such as per-band UE capability in option 1, or be addressed by limiting carriers/MOs to be measured hence either option 1 or option 2 can be considered.

- Proposal 2: For UE capability design, option 1 or 2 could be considered.

- Summary

In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. Based on above analysis, following proposals are present.

- Proposal 1: For the conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF , for scenario 1/2/3/4, option 1a is supported.

- Proposal 2: For UE capability design, option 1 or 2 could be considered.

- References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RANP 105

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, RAN4 114, CATT