3GPP TSG-RAN WG4 Meeting #115	R4-2505659

Malta, 19 May – 23 May, 2025

Source: 	vivo

Title: 	On remaining RRM requirements for UE-to-UE CLI handling

Agenda Item:			7.25.4.1

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD based on [2] and [3].

- Discussion

- Sub-topic 1-1: General issues

At previous meeting, the following agreements are achieved based on [2].

Issue 1-1-1: Rx beam

- Agreement

  - RAN4 to use a reference to RAN1 spec to reflect the Rx beam assumption for L1 UE-to-UE CLI measurements, which can be further discussed in CR drafting phase.

  - The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

Issue 1-1-2: Measurement reporting

- Agreement

  - Define reporting requirements for periodic reporting of L1-CLI-RSSI.

  - L1-RSRP measurement reporting requirement for periodic reporting as defined in clause 9.5.3.1 are re-used

Issue 1-1-3: Measurement capability

- Agreement

  - RAN4 not to discuss measurement capability requirements unless requested by other WGs.

Issue 1-1-4: Report mapping

- Agreement (online session on Monday)

  - Confirm that the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI, including both absolute and differential reporting, given by RAN1 LS R1-2501620, are reasonable.

  - Note: In RAN4 understanding, the “Infinite” in Table 10.1.22.1.2-1, TS38.133 is also reused for L1 SRS-RSRP.

  - RAN4 to define measurement report mapping for L1-SRS-RSRP and L1-CLI-RSSI, including both absolute reporting and differential reporting.

Issue 1-1-6: Spec structure

- Agreement

  - Close the issue, discuss the CR work split directly.

Based on the WF [2], the following agreements are available for discussion:

Issue 1-1-5: Requirement applicability

- Proposals

  - Proposal 1 (vivo, CATT, Nokia, LGE, Samsung, HW, CTC, QC):

    - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

      - when CLI-RSSI measurement resource is configured within active BWP

  - Proposal 2 (Samsung):

    - The L1 CLI measurement requirements are applicable provided that L1 CLI measurement resource are measurable

  - Proposal 3 (ZTE):

    - The following applicability rules are specified for UE-to-UE CLI measurement:

      - For SRS-RSRP resource, it should be within the UL active BWP, UE could measure the SRS-RSRP resource within the UL active BWP.

      - For CLI-RSSI resource

        - For CLI-RSSI measurement within UL SB, no restriction on resource configuration, but UE only measure the resource within UL active BWP.

        - For CLI-RSSI measurement within DL SB, no matter the CLI-RSSI resource cross two DL SBs or not, UE measures the CLI-RSSI resource within one or two DL SBs, no restriction on resource configuration.

      - No gap is needed for UE-to-UE CLI measurement

  - Proposal 4 (MTK):

    - L1 CLI measurement resources must be within the DL active BWP of the UE, where L1-SRS-RSRP confined within UL subband, while L1-CLI-RSSI based method #1 and #3 to be confined within DL and UL subband, respectively.

- Recommended WF

  - P1 and P3 are similar in that the L1 CLI measurement resources are limited to be within active BWP, while in P4 the L1 CLI measurement resources are limited to be within UL or DL subband. It is suggested to agree on P1 based on majority view.

  - P2 is for a separate issue, and it is suggested to agree on P2 as it is quite straightforward.

Proposal 1: For requirement applicability, support P1.

    - Sub-topic 1-2: L1-SRS-RSRP

At previous meeting, the following agreements are achieved based on [2].

Issue 1-2-1: Measurement period

- Agreement

  - The number of samples (M) for measurement period:

    - For periodic and semi-persistent SRS resource, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

    - For aperiodic SRS resources M=1

Issue 1-2-2: Scheduling restriction

- Agreement (online session on Monday with update)

  - For UL,

    - Scheduling restrictions always apply, for both FR1 and FR2.

    - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements.

    - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211.

  - For DL,

    - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies.

    - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements.

    - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

For the FFS part, since the L1-SRS-RSRP measurement is performed with at least Tc*NTA_offset offset with respect to the DL timing, after the measurement on SRS resource, the DL reception will happen on the DL subband. To our understanding the timing offset of the L1-SRS-RSRP could already cover any switch between UL subband and DL subband if this switch is needed.

Proposal 2: For scheduling restriction for L1-SRS-RSRP measurement, no restricted symbol is needed after SRS resource to consider the switching between UL subband and DL subband.

Issue 1-2-3: Measurement restriction

- Agreements

  - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

  - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options

    - Option 1: Prioritize L1-SRS-RSRP measurement

    - Option 2: Prioritize L1-CLI-RSSI measurement

    - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

Proposal 3: For measurement restriction for L1-SRS-RSRP measurement, OK with option 1 and 3.

- Sub-topic 1-3: L1-CLI-RSSI

At previous meeting, the following agreements are achieved based on [2].

Issue 1-3-1: Measurement period

- Agreements (online session on Monday)

  - For DUD case with method #1 and resource type#2

    - In RAN4 understanding, if UE can support simultaneous PDSCH reception on the two DL subbnads for DUD, it can support simultaneous RSSI measurement on the two DL subbands based on 1 sample.

    - RAN4 to decide the number of samples for measurement period based on RAN1 progress on UE capability.

  - For method #3, the measurement period is based on 1 sample.

Issue 1-3-2: Scheduling restriction

- Agreements (adhoc session on Tuesday with addition)

  - Method #1:

    - For UL,

      - Scheduling restrictions always apply, for both FR1 and FR2.

      - The restricted symbols include symbols for RSSI resource.

      - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2

    - For DL,

      - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource

  - Method #3:

    - For UL,

      - Scheduling restrictions always apply, for both FR1 and FR2.

      - The restricted symbols include symbols for RSSI resource.

      - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2

    - For DL,

      - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource.

        - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH.

      - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband.

  - Generic assumption for defining RRM requirements, not intend to add it in the core requirement.

    - For all Method #1, Method #2 and Method #3,

      - Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

For the DL scheduling restriction for L1-CLI-RSSI measurement, based on the generic assumption for the RF bandwidth, our understanding is there is no need to have any restricted symbol to account for UL and DL subband switching.

Proposal 4: For the DL scheduling restriction for L1-CLI-RSSI measurement, no need to have any restricted symbol to account for UL and DL subband switching.

Issue 1-3-3: Measurement restriction

- Agreements (adhoc session on Tuesday)

  - Collision between L1-CLI-RSSI measurements with different methods

    - Capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol.

  - Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements

    - For Method #1& Method 3:

      - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: For requirement applicability, support P1.

Proposal 2: For scheduling restriction for L1-SRS-RSRP measurement, no restricted symbol is needed after SRS resource to consider the switching between UL subband and DL subband.

Proposal 3: For measurement restriction for L1-SRS-RSRP measurement, OK with option 1 and 3.

Proposal 4: For the DL scheduling restriction for L1-CLI-RSSI measurement, no need to have any restricted symbol to account for UL and DL subband switching.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4 114bis

[3] R4-2503623,	Topic summary for [114][222] NR_duplex_evo, Huawei, RAN4 114bis