3GPP TSG-RAN WG4 Meeting #115	R4-2505660

Malta, 19 May – 23 May, 2025

Source: 	vivo

Title: 	On remaining RRM impacts for SBFD operation

Agenda Item:			7.25.4.2

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD.

- Discussion

This WI has been discussed at RAN4 112 and RAN4 112bis meeting. At previous meeting, based on the WF [2] and topic summary [3], the following agreements are available:

      - Issue 2-1-1a: CSI-RS L1 measurement – collision with UL

- Agreements (adhoc session on Thursday)

  - Dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement.

  - Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission following the R16 NR-U approach.

      - Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Background:

  - Different cases for DUD

    - Case 1: X1 >= X3 and X2 >= X3

    - Case 2: X1 >= X3 and X2 < X3

    - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

    - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

  - RAN1 agreements related to CSI processing time

Agreement [RAN1#118b] For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

- Agreements (adhoc session on Thursday with additions)

  - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD

  - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2

  - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements

    - Option 1: Add a lower bound of 8ms on top of existing delay requirements.

    - Option 2: other

  - For Case 3

    - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs

      - In the test case, not model the interfering UE.

For the open issue regarding to CSI processing delay, it is ok to introduce 8ms when UE performs measurement on 2 DLs.

Proposal 1: Regarding to CSI processing delay, it is ok to introduce 8ms when UE performs measurement on 2 DLs.

      - Issue 2-1-1c: CSI-RS L1 measurement – different symbol types

- Agreements

  - For L1-RSRP and L1-SINR

    - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network

    - For L1-RSRP and L1-SINR measurement period, further discuss the following options.

      - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach.

      - Option 2: other

  - For RLM/BFD/CBD, further discuss the following options

    - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type

    - Existing measurement period apply

    - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

For the L1-RSRP and L1-SINR measurement period, measurement period for CSI-RS based L1-RSRP/L1-SINR can be extended by using a scaling factor instead of counting the missing attempts.

Proposal 2: For the L1-RSRP and L1-SINR measurement period, measurement period for CSI-RS based L1-RSRP/L1-SINR can be extended by using a scaling factor.

For CSI-RS based RLM/BFD/CBD, it is ok to consider the first two bullets.

Proposal 3: For CSI-RS based RLM/BFD/CBD, support the following:

- RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type

- Existing measurement period apply

      - Issue 2-1-4: Requirements for RACH requirements

- Agreements

  - Discuss whether and how RACH requirements are to be updated for SBFD operation based on RAN2 agreement.

      - Issue 2-1-6: Requirements for MG and BWP switch

- Agreements

  - Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification, e.g. in MG and BWP switch requirements.

      - Issue 2-1-7: Requirements for UL resource muting

- Agreement

  - RAN4 to confirm there is no RRM impact due to UL resource muting

In addition we discuss the following open issue:

Issue 2-1-2: Requirements for CSI-RS based L3 measurement

- Proposals for requirements for DUD case

  - Proposal 1 (CATT, Nokia, E///, ZTE):

    - RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells.

      - UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell. (CATT, ZTE)

      - RAN4 can send LS to RAN2 to ask for supporting the configuration (E///)

      - RE-use the conclusion of L1-RSRP/SINR CSI-RS resource in different symbol types (ZTE)

  - Proposal 2 (Samsung, CMCC, HW):

    - For DUD case, use the same principle as agreed in last meeting for DU case

      - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS. (HW)

  - Proposal 3 (MTK):

    - RAN4 shall deprioritize CSI-RS based L3 measurements.

- Proposals for collision between CSI-RS resource and dynamic UL transmission

  - Proposal 1 (CATT):

    - Measurement is prioritized over UL transmission

  - Proposal 2 (ZTE):

    - For serving cell CSI-RS based L3 measurement, apply same principle as that for CSI-RS based L1 measurement.

    - For neighbour cell CSI-RS based L3 measurement, follow existing scheduling restriction to handle the collision.

- Recommended WF

  - Discuss the following options on principles for defining requirements for DUD case

    - Option 1: define requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells

    - Option 2: same as DU case, UE measurement requirements would be same as in legacy under certain conditions

For the CSI-RS based L3 measurement for DUD case, “option 2: same as DU case, UE measurement requirements would be same as in legacy under certain conditions” is preferred.

Proposal 4: For the CSI-RS based L3 measurement for DUD case, prefer option 2, i.e., same as DU case.

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: Regarding to CSI processing delay, it is ok to introduce 8ms when UE performs measurement on 2 DLs.

Proposal 2: For the L1-RSRP and L1-SINR measurement period, measurement period for CSI-RS based L1-RSRP/L1-SINR can be extended by using a scaling factor.

Proposal 3: For CSI-RS based RLM/BFD/CBD, support the following:

- RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type

- Existing measurement period apply

Proposal 4: For the CSI-RS based L3 measurement for DUD case, prefer option 2, i.e., same as DU case.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4 114bis

[3] R4-2503623,	Topic summary for [114][222] NR_duplex_evo, Huawei, RAN4 114bis