# 3GPP TSG-RAN WG4 Meeting #115  		R4-2505662

Malta, 19 May – 23 May, 2025

Source: 	vivo

Title: 	On remaining issues for on-demand SSB SCell operation in R19 NES

Agenda Item:	7.27.3.1

Document for:	Discussion

Introduction

In RAN4 #114bis, the WF [1] and [2] is agreed.

Based on all above information, we provide our views on RRM requirement impacts for on-demand SSB SCell operation in R19 NES.

Discussion

OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 3: OD-SSB in FMW collided with MG

Way forward

- FFS UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

For the scenario when OD-SSB measurement within FWM collides with MG occasion, MG should have higher priority.

Proposal 1: UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

Sub-issue 4: Multiple active OD-SSBs carriers

- Proposals

  - Option 1: Apple, OPPO

    - Not support in this release. RAN4 only defines requirements for single OD-SSB carrier under CA operation.

  - Option 2: Apple

    - Introduce CSSF in corresponding measurement requirement

  - Option 3: ZTE

    - CSSF only counts OD-SSB layers if multiple OD-SSB layers are non-overlapping.

  - Option 4: Huawei

    - For carriers whose time period with activated OD-SSB are non-overlapping, these carriers are considered as one carrier in CSSF calculation.

  - Option 5: Ericsson

    - The CSSF of OD-SSB based deactivated SCell shall only account the number of frequency layers with activated OD-SSBs within its fast measurement window.

      - The CSSF equals 1 if non-overlapping with other OD-SSBs transmission during the fast measurement window.

      - The CSSF equals N+1 if overlapping with N other layres’ OD-SSB fast measurement window

- Recommended WF

  - Further discussion

The following agreement on CSSF for single OD-SSB carrier is achieved at RAN4 114bis meeting.

Agreement

- When NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer

  - Extend the FMW window time by one OD-SSB periodicity

  - Longer measurement delay of other frequency layers with AO-SSBs are expected

For multiple active OD-SSBs carriers, it is ok if no requirements are defined in R19. Otherwise if this feature is supported in R19, following the logic of single OD-SSB, when 2 FMW window collides, the CSSF for these two deactivated SCell within FWM could be 2. However it is possible that OD-SSB on different carriers have different periodicity hence there are partially overlapping case. In additional it is not sure, using previous example, when two FMM collides and CSSF =2, the carrier which is measured secondly still have can finish corresponding measurement within the FMW window. In summary since it is for OD-SSB, it is not easy just increase CSSF value when multiple SSB of different carriers colldie as that of the legacy case.

Proposal 2: For Multiple active OD-SSBs carriers , prefer option 1.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 1: The start point of T2

Wayforward

The start point of T2 is

- Option 1:

  - The time point of OD-SSB deactivation

- Option 2:

  - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

- Option 2a:

  - The time point of OD-SSB activation

Regarding the starting point of T2, the main purpose of T2 is to ensure UE leaves fast measurement mode and have power saving. Hence when the length of FMW is defined, the starting point of T2 is not related to the L3 reporting time and even not related to the OD-SSB deactivated point. The starting point of T2 can be the beginning of the FMW, option 1 is also acceptable.

Proposal 3: The start point of T2 can be the beginning of the FMW (option 2a).

Sub-issue 2: The minimum interval(T2) between multiple FMWs

Wayforward

- Option 1: 5* measCycleSCell

- Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity)

- Option 1b: 5*max (measCycleSCell, DRX cycles)

- Option 2: Minimum measurement period defined in existing spec.

- Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1

For the duration of T2, in principle either option 1 or option 1b works, considering the range of measCycle is from 160ms to 1280ms, which is similar to that of DRX. Option 1 is preferred.

Proposal 4: The duration of T2 is 5* measCycle.

Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2

Wayforward

- Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

- Option 2: The UE is assumed to perform fast mode measurement without cell identification period.

- Option 3: No requirement for UE within T2.

Since the intention of FMW and T2 is to setup to boundary where fast measurement can be performed and where legacy measurement will be performed. It is preferred that the principle of FMW and T2 is preserved at any circumstance.

Proposal 5: For UE’s behaviour when OD-SSB further activation/reconfiguration within T2, either option 1 or option 3 is ok.

OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-2-2: Requirement for RAN1 Alt Time-C1

Agreement:

- RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario.

  - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification.

  - FFS for other procedures.

For the above agreements, we also have the following agreement:

Sub-issue 1: Lower bound for measurement in FMW

Agreement

Introduce UE capability on whether a lower bound is needed, i.e.,

  - The lower bound is not needed (default value), or,

  - A lower bound is needed and based on OD-SSB periodicity of [10] ms

    - Further confirm whether [10] is feasible, and only one value

Since a lower bound of [10] ms has been considered, when AO-SSB and OD-SSB periodicity is 10 ms or 5ms, the periodicity used within FMW should be 10ms.

Proposal 6: For requirement for RAN1 Alt Time-C1, when AO-SSB and OD-SSB periodicity is 10 ms or 5ms, the periodicity used within FMW should be 10ms.

Issue 1-2-3: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: Define the requirement based on OD-SSB periodicity.

- Option 2: Not to define requirement.

For this issue, option 2 is ok since it simply does not encourage NW to have this kind of configuration. Alternatively, the OD-SSB from NW must be sent for a purpose and when it was sent, it is reasonable for UE to measurement OD-SSB.

Proposal 7: For requirement for RAN1 Alt Time-C2, either option 1 or option 2 is ok.

Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

Agreement

RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015.

- Option 1: RAN4 to define requirement for different frequencies

- Option 2: RAN4 not to define requirements for different frequencies

Proposal 8: When OD-SSB and always-on SSB are within different frequencies, prefer RAN4 not to define requirements.

OD-SSB based SCell activation

OD-SSB general issues

Issue 1-4-1: Additional Processing time for OD-SSB reception – scenario

Issue 1-4-2: Additional Processing time for OD-SSB reception - value

- Background

In RAN4 #113 meeting, RAN4 achieved agreements as follow.

Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

- Information

RAN1 #120-bis Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. - If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ

Agreement

UE applies the additional processing timing only in deactivated SCell measurement

Note: The additional UE processing timing is not applied to SCell activation procedure.

The additional processing timing is T_processing, the value of T_processing:

- Introduce different values with UE capability: [2 or 3]ms and 5ms.

Proposal 9: For the additional UE processing timing T_processing, among [2 or 3]ms, 2ms is preferred.

Issue 1-4-3: Additional Processing time for OD-SSB deactivation

Wayforward

- FFS RAN4 to consider defining the same timeline for activating and deactivating OD-SSB

Proposal 10: For the additional UE processing timing for OD-SSB deactivation, no need to define it.

Issue 1-4-10: Cell identification

The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios.

- Proposals

  - Option 1-1: Ericsson

    - UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the subsequent OD-SSB is transmitted with the same spatial reception parameter.

  - Option 1-2: CMCC

    - The cell is the detected cell if

      - T2 is less than [5]s, and the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power, and on the same frequency carrier as the OD-SSB used for the valid measurement report,

      - provided the timing to this cell has not changed more than  3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected

    - The cell is the newly detectable cell if T2 is larger than [5]s.

  - Option 2: Xiaomi, Ericsson

    - Case 2: Always-on SSB on the cell

  - Option 3: Xiaomi, Ericsson

    - OD-SSB Scell being measured is contiguous to an active serving cell in the same band.

  - Option 4: vivo

    - Extend referenceCell to OD-SSB SCell to skip cell identification

  - Option 5: Samsung

    - No need to extend referenceCell to OD-SSB SCell to skip cell identification

- Recommended WF

  - Moderator suggests the group to check the following sub-issues.

    - The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios:

      - Case 1: No always-on SSB on the cell

        - Option 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

        - Option 2-1: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the OD-SSB is transmitted again with the same spatial reception parameter.

          - Other detail condition is FFS.

(Note: this option has already covered in issue 1-2-3)

        - Option 2-2: UE has sent a valid measurement report based on OD-SSB for the SCell being activated [5]s before, and then the OD-SSB is transmitted again with the same spatial reception parameter same transmission power, and on the same frequency carrier.

          - Other detail condition is FFS.

        - Option 3: Extend referenceCell to OD-SSB SCell

        - Other option is not precluded

      - Case 2: Always-on SSB on the cell

        - Option 1: Cell identification can be always skipped

          - Option 1a:  Cell identification can be always skipped in scenario 2

        - Option 2: follow SCell activation known condition

        - Other option is not precluded

Proposal 11: For OD-SSB based cell identification before deactivated SCell measurement can be skipped, prefer option 3 in case 1.

Issue 1-4-11: OD-SSB based L3 reporting

- Proposals

  - Option 1: Qualcomm, CMCC

    - RAN4 not introduce OD-SSB based L3 measurement reporting.

  - Option 2: Huawei

    - RAN4 to send LS to RAN2 to support L3 report triggered by OD-SSB activation.

- Recommended WF

  - Further discussion

Although it is not yet clear whether UE L3 reporting can be triggered by the MAC CE based OD-SSB activation, it is somehow beneficial for the network power saving, if UE report the L3 results immediately after the MAC CE based OD-SSB activation, since it would extend the possibility of the known scenario. On the other hand, we do not see any issue if RAN4 re-use legacy L3 measurement requirements.

Proposal 12: For OD-SSB based L3 reporting , slightly prefer option 2.

Issue on <On intra-band CA>

For intra-band co-located CA case, how much NW energy saving gain can be achieved for the OD-SSB case would be unclear, and such scenario significantly complicates the UE RRM requirements for SCell activation. As discussed in the SI phase, most NW energy comes from gNB TxRUs to support large antenna array, and with intra-band collocated active serving cell, such TxRUs shall be already activated. Since the power consumption is already high, periodic SSB transmission, if needed, on an intra-band collocated SCell, will not cause a significant power consumption issue. On the other hand, no matter SSB is transmitted or not for an intra-band collocated SCell, spec impacts would be quite large for both cases, at least in FR1. Therefore, RAN4 can consider deprioritize such scenario, since the necessity of such feature and the network energy saving gain are not clear, but big spec impact is foreseen.

For intra-band non-collocated CA in FR1, as discussed in R18, it shall be up to UE capability whether AGC can be adjusted for two intra-band CCs with non-collocated serving cells separately. For simplicity, it is possible to only consider the case UE have such capability.

Proposal 13: RAN4 to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1. For intra-band non-collocated CA in FR1, only consider UE with similar behaviour as supporting intraBandNR-CA-non-collocated-r18

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

Proposal 2: For Multiple active OD-SSBs carriers , prefer option 1.

Proposal 3: The start point of T2 can be the beginning of the FMW (option 2a).

Proposal 4: The duration of T2 is 5* measCycle.

Proposal 5: For UE’s behaviour when OD-SSB further activation/reconfiguration within T2, either option 1 or option 3 is ok.

Proposal 6: For requirement for RAN1 Alt Time-C1, when AO-SSB and OD-SSB periodicity is 10 ms or 5ms, the periodicity used within FMW should be 10ms.

Proposal 7: For requirement for RAN1 Alt Time-C2, either option 1 or option 2 is ok.

Proposal 8: When OD-SSB and always-on SSB are within different frequencies, prefer RAN4 not to define requirements.

Proposal 9: For the additional UE processing timing T_processing, among [2 or 3]ms, 2ms is preferred.

Proposal 10: For the additional UE processing timing for OD-SSB deactivation, no need to define it.

Proposal 11: For OD-SSB based cell identification before deactivated SCell measurement can be skipped, prefer option 3 in case 1.

Proposal 12: For OD-SSB based L3 reporting , slightly prefer option 2.

Proposal 13: RAN4 to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1. For intra-band non-collocated CA in FR1, only consider UE with similar behaviour as supporting intraBandNR-CA-non-collocated-r18

- References

[1] R4-2504907,	WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN4 114bis

[2] R4-2504906,	WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4 114bis