# 3GPP TSG-RAN WG4 Meeting #115  	                                 R4-2505663

Malta, 19 May – 23 May, 2025

Source: 	vivo

Title: 	On remaining issues for adaptation on common signal in R19 NES

Agenda Item:	7.27.3.2

Document for:	Discussion

Introduction

At RAN4 114bis, the WFs [1], [2] are approved.

Based on all above information, we provide our views on RRM requirements for common signal or channel adaptation in R19 NES.

At RAN1 120bis meeting the following agreements are available for SSB adaptation.

Agreement

For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- The DCI is scrambled a new RNTI,

  - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter,

- the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell.

Agreement

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

- Xmax=2

Agreement

Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported

- CB-PreamblesPerSSB

At RAN2 129bis meeting the following agreements related to SSB adaptation are available:

For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

The following agreements are achieved at RAN4 114bis meeting:

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

  - Further clarification on the scope of RRM impact for SSB adaption:

    - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

    - SSB adaptation in SCell for NCD-SSB.

    - Time domain positions of SSBs within a burst is same before and after SSB adaptation

  - Note: Above could be further updated based on further RAN1 progress if any.

Issue 1-2-1: L1 requirements at transition

Agreement

  - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline.

  - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs.

Issue 1-3-1: L3 requirements for SSB adaptation

Agreement:

- RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation.

    - For neighbour cell L3 measurement, legacy requirements apply.

Open issue for further discuss:

- For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects:

  - Whether to define L3 measurement requirement at transition.

  - Any other RRM requirements impacts due to additional SMTC configurations.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

Agreement:

    - Normal SCell activation

    - Direct SCell activation

    - PUCCH SCell activation

    - FFS Rel-18 SCell activation with L3 reporting

    - FFS Multiple SCells activation

Discussion

In this contribution we discuss the following open topics:

- Sub-topic 1-1 SSB adaptation – Scenario

- Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-2: L1 requirements after transition  - DRX

- Proposals

  - Option 1: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode. (CTC, Samsung)

  - Option 2: keep (TDRX,TSSB) when DRX cycle≤320 ms in the L1-RSRP for serving cell. (Vivo)

- Recommended WF:

  - Discuss above options and whether there is RRM requirements impacts in terms of DRX configurations.

Requirements will be defined for L1-RSRP for serving cell, L1-SINR and CBD and their corresponding legacy requirements are copied below:

Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3  P  PCBD)  TSSB) |
| DRX cycle > 320ms | Ceil(3  P  PCBD)  TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Table 9.8.4.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. |  |

We prefer the requirement using the same principle as that of legacy, i.e., keep (TDRX,TSSB) when DRX cycle≤320 ms.

Proposal 1: For SSB adaptation issue 1-2-2, prefer keep (TDRX,TSSB) when DRX cycle≤320 ms in the L1-RSRP for serving cell.

Issue 1-2-3: L1 requirements - whether to release the measurement results from resources before transition

- Proposals

  - Option 1: UE can to perform measurement based on the SSB resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation. (Samsung)

- Recommended WF:

  - Agree on option 1 and no spec impacts.

Proposal 2: For SSB adaptation issue 1-2-3, Ok with the recommended WF.

Issue 1-2-4: L1 requirements – Measurement restriction impact

- Proposals

  - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei)

- Recommended WF:

  - Discuss above option.

9.5.5.1	Measurement restriction for SSB based L1-RSRP

For FR1, if UE support rxTimingDiff-r18, when [the SSB for L1-RSRP measurement and CSI-RS for RLM, BFD, CBD or L1-RSRP measurement are overlapping or partially overlapping in OFDM symbols],

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

Based on the current spec a UE is required to measure one of but not both SSB for L1-RSRP measurement when SSB and CSI-RS for L1 measurement collide. For the SSB adaptation, the periodicity of SSB is changed after adaptation whereas other properties of SSB is the same as before. Therefore the legacy method could be reused, i.e., a UE can choose to measure one of them.

Proposal 3: For SSB adaptation, measurement restriction for SSB based L1-RSRP can be the same as that of legacy when SSB collides with CSI-RS.

- Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation

Agreement:

- RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation.

    - For neighbour cell L3 measurement, legacy requirements apply.

Open issue for further discuss:

- For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects:

  - Whether to define L3 measurement requirement at transition.

  - Any other RRM requirements impacts due to additional SMTC configurations.

Regarding other RRM requirement impact due to additional SMTC configuration, it is foreseen no impact on RRM requirement since multiple (2) SMTC configuration has already existed in the current specs. For the requirement during transition period, it is suggested that the requirements follow the largest SMTC between old SMTC configuration and new SMTC configuration.

Proposal 4: No impact on RRM requirement due to additional SMTC configuration. For the requirement during transition period, it is suggested that the requirements follow the largest SMTC periodicity between old SMTC configuration and new SMTC configuration.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: For SSB adaptation issue 1-2-2, prefer keep (TDRX,TSSB) when DRX cycle≤320 ms in the L1-RSRP for serving cell.

Proposal 2: For SSB adaptation issue 1-2-3, Ok with the recommended WF.

Proposal 3: For SSB adaptation, measurement restriction for SSB based L1-RSRP can be the same as that of legacy when SSB collides with CSI-RS.

Proposal 4: No impact on RRM requirement due to additional SMTC configuration. For the requirement during transition period, it is suggested that the requirements follow the largest SMTC periodicity between old SMTC configuration and new SMTC configuration.

- References

[1] R4-2504907,	WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN4 114bis

[2] R4-2504906,	WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4 114bis