# 3GPP TSG-RAN WG4 Meeting #115  		R4-2505664

Malta, 19 May – 23 May, 2025

Source: 	vivo

Title: 	On remaining issues for on-demand SIB1 in R19 NES

Agenda Item:	7.27.3.3

Document for:	Discussion

Introduction

In [1], the following objective is listed.

Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized Specify the corresponding core requirements, for the above features [RAN4].

Based on this, and latest RAN1/2 progress on this, we provide our views on the RRM requirement impacts from on-demand SIB1 in R19 NES.

Discussion

In the agreed WF in RAN4 #114bis [2], for OD-SIB1, the following agreements are available.

Issue 2-1-2: RRM requirements impacts for OD-SIB1 – timing Agreement - Clarify in transmit timing requirements to include UL WUS transmission (PRACH) for OD-SIB1: - reference cell is the NES cell - NTA for UL-WUS is defined as 0 - NTA offset is the [n-TimingAdvanceOffset] from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133 - Note: The IE can be updated based on other WG’s progress.

And for issue 2-1-1, the following way forward are available:

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Way forward:

    - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo)

    - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE)

    - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time. (E///, LGE, MTK)

The procedure of OD-SIB1 is illustrated in the following figure.

Figure 1  Flow chart of OD-SIB1 procedure

The detail of the procedure is:

    - UE receives UL WUS configuration from cell A, via SIB. The UL WUS is for wake-up of NES cell. Triggering condition of the UL WUS can be configured to the UE in the UL WUS configuration.

    - After UE meets the WUS triggering conditions, UE shall send UL WUS to the NES cell. The UL WUS uses PRACH formats, but sending UL WUS does not trigger an initial access to any cell.

    - After UE transmits the PRACH, UE waits for msg2. Potential retransmission and power ramping are possible and the details are still being discussed in RAN1/2.

    - A successful decoding of msg2 means UE is going to try to obtain the SIB1 from NES cell within a time window. SIB1 repetition is performed within this window.

    - UE obtains the SIB1 from NES cell and evaluates whether UE shall camp on the NES Cell.

Note that in the above procedure, UE does not enter RRC connected mode. Only idle/inactive state needs to be considered. Moreover, according to RAN2 agreements in the annex, NES-capable UE actually performs cell re-selection to NES cell while non-NES-capable UE are prevented form reselecting to NES cell via Re-selection priority rules or ExcludedCellList.

Observation 1  OD-SIB1 related procedure only impacts UE behaviour in RRC idle/inactive states.

Observation 2  UE performs cell re-selection to NES cell with OD-SIB1 enabled, when UL WUS triggering condition is met, i.e. UL WUS triggering condition is the same as cell re-selection condition and OD-SIB1 acquisition shall be regarded as a special case of cell re-selection.

In legacy cell re-selection requirements, the related UE behaviour is classified into two parts:

1)  Performing re-selection evaluation based on RRM measurements, while paging reception is not interrupted.

2)  Performing target cell relevant SIBs reception, while paging is interrupted.

Figure 2 is an illustration of UE behaviour on above two parts. The overall NR cell re-selection delay requirements, which are tested since R15, is the summation of these two parts. In legacy cell re-selection, SIB1 reception and other SIBs reception are considered without UL WUS requests. SIB reception with UL WUS configured are not considered in legacy requirements. Therefore, it is RAN4’s responsibility to check the impacts on cell re-selection delay from R19 UL WUS procedure, especially for the 2nd part above, i.e. the impacts on paging interruption.

Figure 2  UE behaviour in idle/in-active state, which comprise 1) neighbour cell measurements when camping on a cell; 2) SIBs reception during cell re-selection procedure.

Note that in existing TS 38.133, as stated in Clause 4.2.2.6, the paging interruption requirements are specified without providing a specific value on TSI_NR, as cited below.

TS 38.133 v18.8.0 Clause 4.2.2.6 - 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

In TS 38.133 Clause A.6.1.1 test cases, TSI_NR is assumed as 1280ms for 1.28s DRX cycle, 0.64s DRX cycle and 0.32s DRX cycle. Checking Table 4.4.3.1.3-1, Table 4.4.3.1.3-2, Table 4.4.3.1.3-3 in TS 38.508-1, the max SIB periodicity assumed for cell re-selection test cases is 640ms. Therefore, our understanding is that, considering 0.64s DRX cycle length, 2 DRX cycles would be needed for all relevant SIBs reception, as shown in Figure 2.

TS 38.508-1 v18.4.0 - 4.4.3.1.2 Combinations of system information blocks for NR standalone and NE-DC … Combination NR-2 applies to the following test case scenarios: - NR FDD intra-frequency multi cell scenario - NR TDD intra-frequency multi cell scenario - NR FDD and NR TDD dual mode multi cell roaming scenario - NR FDD single cell RRM test case scenario - NR TDD single cell RRM test case scenario Combination NR-3 applies to the following test case scenarios: - NR FDD intra-frequency multi cell scenario with neighbouring cell related information - NR TDD intra-frequency multi cell scenario with neighbouring cell related information Combination NR-4 applies to the following test case scenarios: - NR FDD inter-frequency multi cell scenario - NR TDD inter-frequency multi cell scenario - NR FDD inter-band multi cell scenario - NR TDD inter-band multi cell scenario - NR FDD and NR TDD dual mode multi cell non-roaming scenario - NR FDD intra-band carrier aggregation component carriers cell scenario - NR FDD inter-band carrier aggregation component carriers cell scenario - NR TDD intra-band carrier aggregation component carriers cell scenario - NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario … ##### 4.4.3.1.3 Scheduling of system information blocks The scheduling configurations for combinations of system information blocks are defined in the following tables. There is no scheduling information for combination NR-1. Table 4.4.3.1.3-1: Scheduling for combination NR-2 Scheduling Information No.; Periodicity [radio frames]; Mapping of system information blocks. 1; 32; SIB2. Table 4.4.3.1.3-2: Scheduling for combination NR-3 Scheduling Information No.; Periodicity [radio frames]; Mapping of system information blocks. 1; 32; SIB2. 2; 64; SIB3. Table 4.4.3.1.3-3: Scheduling for combination NR-4 Scheduling Information No.; Periodicity [radio frames]; Mapping of system information blocks. 1; 32; SIB2. 2; 64; SIB4.

We are not sure whether 1280ms is still an adequate estimation for OD-SIB1 case. We would like to encourage RAN4 for further discussion.

Note that compared to Other SI, SIB1 is essential for cell re-selection. Therefore, although no RRM requirements were specified for OD-OSI, which is an R15 feature, in our view it would be important to specify RRM requirements for OD-SIB1.

Proposal 1  RAN4 further discuss the potential RRM requirement impacts on paging interruption for R19 OD-SIB1 related procedure.

Another issue is that, in legacy, when UE performs RACH, UE will either setup or resume RRC connection. In other word, RACH procedure happens at the start or during RRC connected mode, where the UE power consumption for RACH power ramping is not a significant issue compared to traffic data latency. However, UL WUS for OD-SIB1 request is RACH procedure in idle mode, where UE has no traffic data but only paging or SIB reception to handle. According to TS 38.321, the RACH triggered for SI request shall be performed as follows.

TS 38.321 v18.4.0 Clause 5.1.2 1> if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and 1> if ra-AssociationPeriodIndex and si-RequestPeriod are configured: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB). 1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: 3> determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB).

Since UE only performs SSB measurement during its wake-up time in the idle/inactive mode, the selected RO shall also be during UE wake-up time in the idle/inactive mode. Hence, in our understanding, WUS transmission shall be only done when UE wakes up. Therefore, there are two potential possibilities for UE performing OD-SIB1 request, as shown in Figure 3, considering potential msg1 retransmission as RAN1/2 agreed.

Figure 3  Two options for OD-SIB1 request procedure. Option 1 assumes UE performs all WUS msg1 retransmission in one paging cycle. Option 2 assumes UE performs WUS msg1 retransmission in multiple paging cycles.

In Figure 3, Option 1 assumes UE shall perform all WUS msg1 retransmission in one paging cycle, if possible. Option 2 assumes UE may perform WUS msg1 retransmission in multiple paging cycles. Two different options may have different impacts on paging interruption requirements, including test cases. We are not sure following legacy principles, existing 1280ms in the test cases is still enough of OD-SIB1 related cell re-selection. Therefore, we would prefer to clarify in clause 4.2.2.6 of TS 38.133 that for OD-SIB1 relate cell re-selection, UL WUS transmission and msg2 reception shall be counted as necessary terms of paging interruptions.

Proposal 2: Clarify in TS 38.133 4.2.2.6 that, time for UL WUS msg1 transmission and time for msg2 reception are components of paging interruption for OD-SIB1 cell re-selection requirements.

In legacy paging interruption requirements, RAN4 assumes sufficient radio conditions, so that decoding of system information can be made without errors. This is because legacy cell re-selection is only related to downlink radio link quality judgements. In other word, UE may judge target cell downlink radio link quality before determining to perform cell re-selection to the target cell, and prevent such error case so as to ensure performance itself.

However, if UL WUS request for OD-SIB1 is considered, uplink radio link quality information, which is not available at UE side before any UL WUS transmission, may also impact the overall latency of cell re-selection. From UE power saving perspective, option 1 above shall be prevented if there is msg1 retransmission, especially if cell-reselection happens in consecutive paging cycles, e.g. when there are multiple NES cells, and UE bars NES cells one after another due to RACH failure. Therefore, it is important to allow UE to have more flexibility in determine how many RACH attempts would be required during each paging cycle. As shown in option 2 above, this would need relaxation on paging interruption requirements. It would be necessary to specify RRM requirements for the minimum msg1 attempts during each paging cycle, so as to balance between UE power saving and paging performance.

Proposal 3 RAN4 further discuss the RRM requirements on minimum WUS msg1 attempts during each paging cycle, as part of paging interruption requirements.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  OD-SIB1 related procedure only impacts UE behaviour in RRC idle/inactive states.

Observation 2  UE performs cell re-selection to NES cell with OD-SIB1 enabled, when UL WUS triggering condition is met, i.e. UL WUS triggering condition is the same as cell re-selection condition and OD-SIB1 acquisition shall be regarded as a special case of cell re-selection.

Proposal 1: RAN4 to further discuss the potential RRM requirement impacts on paging interruption for R19 OD-SIB1 related procedure.

Proposal 2: Clarify in TS 38.133 4.2.2.6 that, time for UL WUS msg1 transmission and time for msg2 reception are components of paging interruption for OD-SIB1 cell re-selection requirements.

Proposal 3: RAN4 to further discuss the RRM requirements on minimum WUS msg1 attempts during each paging cycle, as part of paging interruption requirements.

- References

[1] RP-242354,   Revised WID: Enhancements of network energy savings for NR, Ericsson,  RAN #105

[2] R4-2504907,	WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN4 114bis

[3] R4-2504906,	WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4 114bis