3GPP TSG-RAN WG4 Meeting #115	R4-2505674

Malta, Malta, 19th – 23rd May 2025

Title: 	UE assistance for SRS imbalance solution considerations

Agenda Item:	7.1.2.3.4

Source: 	Nokia

Document for:	Approval

# Introduction

In previous RAN4 meeting WF of 6Rx UE requirements was approved. This contribution discusses about of SRS imbalance addressing solutions. Following issues and way forward was recorded [1].

### Sub-topic 4-1: General considerations for SRS IL imbalance issue

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way forward:

- No further discussion on the issue in future meetings and focus on identification of possible SRS IL imbalance issue solutions.

  - NOTE: This does not mean that any solution for SRS IL imbalance issue is specified in Rel-19

Issue 4-1-2: SRS IL solution applicability to 2Rx/4Rx/6Rx/8Rx

Agreement

- FFS on the potential SRS IL solution applicability to 2Rx/4Rx/8Rx UEs in addition to 6Rx UEs

Issue 4-1-3: SRS IL imbalance solution as optional feature

Agreement

- If any new requirements or procedures for SRS IL handling are defined in Rel-19, they will be defined as optional UE features

### Sub-topic 4-2: SRS IL imbalance issue solutions

Way forward

- RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2.

- Companies are encouraged to provide views on additional aspects including

  - SRS output power testing

  - Enhancements to PCmax equations to improve SRS IL compensation

Issue 4-2-1: SRS IL imbalance UE self-compensation

Agreement:

- RAN4 assumptions on UE behavior for Rel-19 SRS IL handling:

  - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power

    - FFS if any additional clarifications on behavior are needed

    - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications)

Issue 4-2-2: SRS IL imbalance UE assistance and reporting

Agreement:

- The following two possible general solutions may be considered for further consideration

  - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation

    - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed

  - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility

    - Further check whether solution 2 is in the scope or not

  - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

# Discussion

There have been extensive discussions regarding the imbalance of SRS related to UE hardware implementation, antenna interface, and its impact on network performance, which commenced with Release 18 in both RAN4 and RAN1, as referenced in [2]. To date, there is no agreement made to address the imbalance issue. The methods proposed in previous meeting exhibit drawbacks, either in their ability to resolve the issue, added signalling overhead or changes to RAN1 and RAN2 specifications.

In this document we provide views to utilize existing PCMAX,f,c and PHR framework for SRS imbalance assistance as recorded in WF [1].

## Introduction

SRS imbalance issues become more pronounced as the number of antennas increases. To address this issue, solutions have been proposed since Release 18, focusing on UE self-compensation of SRS imbalance, signalling the imbalance to the network, or both. The former approach is viable only when the power amplifier (PA) has sufficient maximum linear power headroom on the main antenna while providing SRS resources to other antennas. If the value of SRS insertion loss (IL) is high, compensation may be partial; however, this remains a feasible, UE-based solution to address the imbalance issue. Even though the compensation is partial, any amount of compensation would improve the signal-to-noise ratio (SNR) in the gNodeB.

The UE is maximum output power (MOP) limited in cell edge scenarios; however, this is not likely a relevant scenario for MIMO downlink reception. Conversely, in cell edge scenarios, there are alternative methods to SRS-based sounding for estimating channel state information (CSI), which have been discussed in other papers.

Figure 1 represents a simplified solution of an SRS antenna switching (AS) solution for a single band. Only a 1T6R configuration is considered here, as the methodology described can be applied to any SRS AS solutions. After the power amplifier (PA), there is a switch arrangement for allocating SRS resources to the required antenna. The same arrangement can be used for transmit antenna switching (TAS) to optimize over-the-air (OTA) performance, which might be compromised by user interaction with the transmit antenna. The dashed line between the switches and antenna ports represents a reference plane in which transmission (TX) and reception (RX) are calibrated in the factory. At the same time, it provides a reference plane for channel sounding.

Figure 1 Switching arrangement for SRS resource routing

The idea of channel sounding is to provide gNodeB information about radio channel condition by utilizing UL/DL reciprocity and specific reference signal (SRS resource) transmitted to each of the DL antennas for the gNodeB to acquire DL CSI. UE side antennas are to be included into this estimation.

SRS imbalance occurs due to different losses from PA to each antenna. Different power level of SRS symbols, due to UE implementation, arriving at the gNodeB provide erroneous indication of channel conditions which are considered to have non-negligible impact on the DL throughput.

## SRS resource power control

UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) of each SRS resource in SRS transmission occasion i as in TS 38.213; 7.31 by,

UE expected behaviour for 6RX SRS power, is according to 38.213; 7.3:

“else, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the SRS ports of each SRS resource of an SRS resource set in a symbol for SRS transmission.

where,

-	PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS 38.101-2] and [TS 38.101-3] for carrier f of serving cell c in SRS transmission occasion i

-	PO_SRS,b,f,c(qs) is provided by p0 for active UL BWP b of carrier f of serving cell c and SRS resource set qs provided by SRS-ResourceSet and SRS-ResourceSetId

-	MSRS,b,f,c(i) is a SRS bandwidth expressed in number of resource blocks for SRS transmission occasion i on active UL BWP b of carrier f of serving cell c and μ is a SCS configuration defined in [4, TS 38.211]

-	αSRS,b,f,c(qs) is provided by alpha for active UL BWP b of carrier f of serving cell c and SRS resource set qs

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using RS resource index qd as described in clause 7.1.1 for the active DL BWP of serving cell c and SRS resource set qs [6, TS 38.214]. The RS resource index qd is provided by pathlossReferenceRS associated with the SRS resource set qs and is either an ssb-Index providing a SS/PBCH block index or a csi-RS-Index providing a CSI-RS resource index. If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a MAC CE [11, TS 38.321] can provide by SRS-PathlossReferenceRS-Id a corresponding RS resource index qd for aperiodic or semi-persistent SRS resource set qs.

-    hb,f,ci,l=fb,f,ci,l, where fb,f,ci,l is the current PUSCH power control adjustment state as described in clause 7.1.1, if srs-PowerControlAdjustmentStates indicates a same power control adjustment state for SRS transmissions and PUSCH transmissions

Further according to 38.213,

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L.

PCMAX,f,c is defined in TS 38.101-1 as,

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c

PCMAX_L,f,c=MIN{PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass)– MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c+∆TC,c + ∆TRxSRS,

P-MPRc)}

PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

And,

TREF and Teval are specified in Table 6.2.4-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given   by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

Following observations can be made.

Observation 1: PSRS,b,f,c (i,qs,l) is the transmission power for one SRS resource

Observation 2: For each TREF, PCMAX,f,c is set by a minimum PCMAX(i) for a physical channel in a slot(s).

Observation 3: If there is more than one SRS resources in a slot SRS transmission occasion and ∆TRxSRS may set the lower limit of PCMAX,f,c for PUSCH/PUCCH transmission occasions.

## Power headroom reporting

There are two types of power headroom reports specified in in 38.213: 7.7. Type 1 is for PUSCH transmission occasions i on active UL BWP of carrier  of serving cell . Type 3 PH reporting is supported only for a serving cell which PUSCH transmission is not configured.

PHtype1,b,f,c,ki,j,qd,l= PCMAX,f,c,k(i)-PO_PUSCH,b,f,cj+10log102μ⋅MRB,b,f,cPUSCH(i)+αb,f,cj⋅PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dB]

≈ PCMAX,f,c,ki- PPUSCH,b,f,ci,j,qd,l

[dB]

For a non-codebook SRS transmission UE relies on PH1 type report and PUSCH but PCMAX,f,c,ki may get limited by SRS transmission if more than one resources are sent in a slot.

## SRS imbalance assistance

SRS imbalance assistance may provide support for networks under UE operating conditions where SRS transmission occasions become power limited. In scenarios where UE SRS power is lower than PCMAX,f,c it is anticipated that the UE will compensate the SRS power at the antenna connectors to minimize the imbalance between SRS antenna ports.

Considering a UE equipped with single PA thus capable of only 1TxR SRS switching patterns. In a static UL antenna allocation scenario PUSCH transmission and PH reporting occur on same antenna. A gNB can derive PH for first SRS transmission occasion from this report, as the power control parameters for both PUSCH and SRS are known to the gNB. For the gNB ascertain the PH of all other SRS transmission occasions (across different antennas) it must informed of the SRS resource specific PCMAX_SRS,f,c(i). Ideally the difference PCMAX_SRS,f,c0- PCMAX_SRS,f,c(i) represents the SRS IL imbalance and for a UE that doesn’t compensate SRS imbalance, or does so only partially, this imbalance manifests as varying values for PCMAX_SRS,f,c(i), which UE can derive independently.

For a UE utilizing dynamic UL antenna allocation (TAS) PH reporting should follow the UL antenna. Consequently, the RX logical port mapping must be adjusted if the UL antenna assignment changes.

Measurements have been reported [3] indicating that UL and DL insertion losses are not reciprocal thereby reporting both SRS resource specific PCMAX_SRS,f,c(i) and PHSRSi may be justified. This non-reciprocity can be understood by considering the relative positions of the antenna ports with respect to the PA output and RX input ports, which, depending on the implementation, may result in significantly different insertion losses for RX and TX. Reciprocity can be assumed only for UE implementations where the post-PA loss for a SRS resource and the pre LNA loss for a corresponding logical RX port are same.

Conversely, when a UE Rx is operating under favourable SNR conditions where it is not constraint by thermal noise, it can accurately assess the received signal strength at its antenna connectors provided that factory calibrations are correctly done. There should not be significant difference in received signal strength observed between antenna ports.

Currently there is no mechanism to report SRS resource specific PCMAX_SRS,f,c(i) or resource specific PHSRSi. Modifications are required in the RAN1 and RAN2 specifications. If RAN4 decides to proceed with SRS imbalance assistance by using PCMAX_SRS,f,c(i) and PHSRSi it is our perspective that additional network signalling should be minimized, thereby recommending the reporting of only PCMAX_SRS,f,ci to the network.

Observation 4: For a UE using dynamic UL antenna allocation, PH reporting should follow the UL antenna, the RX logical port mapping shall be adjusted if the UL antenna assignment changes.

Observation 5: Non-reciprocity of insertion loss between UL paths for SRS and DL paths of a UE may exist due to different path losses to antennas.

Observation 6: If factory calibration of the UE is conducted accurately there should not be significant difference observed in the received signal strength at UE antenna ports.

Proposal 1: For UE SRS imbalance assistance, UE should report network only SRS resource specific PCMAX_SRS,f,c(i)

## Testing Aspects

Establishing testing procedures to verify the proper allocation of SRS resource power across each port can enhance the consistency of SRS output power within the UE population in a network, particularly when the SRS power is not constrained byPCMAX_SRS,f,c.  Furthermore, if the UE is tasked with reporting network SRS imbalance using  PCMAX_SRS,f,c(i) and PHSRSi there should be also testing added to ensure these parameters are correctly reported.

Proposal 2: UL RMCs shall be added to TS 38.101-1 to verify SRS resource power at each applicable antenna when SRS power is not limited by PCMAX_SRS,f,c.

Proposal 3: UL RMCs shall be added to TS 38.101-1 to verify PCMAX_SRS,f,c(i) and PHSRSi at each applicable antenna are correctly reported.

# Conclusion

In this contribution, we have made number of observations. Summarizing all the observations we can make following conclusions.

Proposal 1: For UE SRS imbalance assistance, UE should report network only SRS resource specific PCMAX_SRS,f,c(i)

Proposal 2: UL RMCs shall be added to TS 38.101-1 to verify SRS resource power at each applicable antenna when SRS power is not limited by PCMAX_SRS,f,c.

Proposal 3: UL RMCs shall be added to TS 38.101-1 to verify PCMAX_SRS,f,c(i) and PHSRSi at each applicable antenna are correctly reported.

# References

[1] R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, RAN4#114bis

[2] R4-2404935, Views on 6Rx for handheld and FWA UE, RAN4#110bis, Samsung

[3] R4-2504580, On the SRS IL imbalance reporting for the SRS-AS, RAN4#114-bis, Ericsson