# 3GPP TSG-RAN WG4 Meeting #115	R4-2505690

# Malta, MT, 19th – 23rd May, 2025

Title: 	On optimizing CSSF for FR2-1 L3 measurement delay

Source: 	OPPO

Agenda item:	7.18.3.2

Document for:	Discussion

Introduction

In last meeting, a WF [1] was approved on FR2-1 SSB based L3 measurement delay reduction.

In this contribution, we will provide discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSFoutside_gap.

Discussion

In previous meetings, RAN4 agreed on solution 1 and solution 3 for CSSF reduction, and send LS to RAN2 to inform them of the potential signaling impact.

Issue 1-2-1: revision on solution 1 Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503618. - Proposal 1 (OPPO, QC, Apple): For solution 1, remove the sub-case and update the agreements as follows: - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Not agree with P1: ZTE, Nokia, Intel, CMCC, E/// - Proposal 2 (OPPO): RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update.

Firstly, usually the measurement configuration can already reflect the measurement purposes and cared carriers from network side. If some SCC is configured in one MO, it should be taken care of by UE during the measurement and report, and the network can reconfigure and adapt the measurement configuration according the measurement reports and real-time demand. In this case, the other SCCs not selected can be taken as low priority or even released from MO list. The benefit of network indication to enable CSSF enhancement seems quite limited.

Secondly, about the additional network indication to measure a SCC, some sub-case is also not reasonable to RAN2. In RAN2’s assumption, if UE is configured with the measurement either in servingcellMO or MOs for neighbor cell measurements, the related reports are expected to be configured together, either for serving cell or neighbor cell measurements. It may happen that one UE is performing measurements but does not fulfill the condition of reporting within some time duration. But it seems not correct configuration for only measurement or report on one CC.

Based on what RAN4 agreed in last meeting, IF there is no SCCs on which UE is configured to report SSB based measurements in band (including both serving cell and neighbor cell measurement), then it means there is no measurement requirement for these SCC in band. Then it is quite strange that why network need further to indicate to measure such SCC and UE shall follow the indication.

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

We can understand the intention to reduce the measurements on both serving and neighbour cells. But the discussed case is redundant to us. So we suggest to remove the sub-case and update the agreements as follows:

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

Observation 1: The MO (e.g., either for serving cell or neighbor cell) is assumed to configured with report configuration based on RAN2’s spec.

Proposal 1: For solution 1, remove the sub-case and update the agreements as follows:

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

If RAN2 sent to LS reply and pointed out the issue of redundant configuration, we are not sure whether RAN4 needs to open the discussion again. For the potential revision of RAN4 agreements or negative reply, RAN4 can take them into account.

We think the SCC decided by UE can be more accurate than that indicated by the network, and such carrier can represent the quality of carriers in the same band. From UE side, UE can select one suitable SCC based on both network configuration and real-time channel conditions, which leave some flexibility to UE implementation.

Furthermore, even without the new signaling indication, we think the requirements under the solution 1 of the selected SCC are not broken. At this stage, RAN4 may focus on the CSSF itself and related RAN4’s requirements, e.g., mainly the tables of CSSF outside gap in clause 9.1.5 of TS 38.133.

Observation 2: Even without the new signaling indication of a specific SCC per-band for measurement, RAN4’s RRM requirements under the solution 1 are not broken.

Proposal 2: RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update.

Issue 1-2-2: Expand solution 1 to FR1 case - FFS: - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band. - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

RAN4 agreed to discuss whether to extend solution 1 to FR1 after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded. However, compared to FR2’s antenna panels and beam design, FR1’s RF and antenna design are quite different, e.g., FR1 omnidirectional antenna, LB, MB and HB RF architecture. We share the similar view as option 2. Even within the same band, different CCs may experience varying levels of interference in FR1, regardless of contiguous or non-contiguous CCs in the FR1 band. Therefore, one CC in FR1 band cannot accurately represent the measurement results for the entire band.

Proposal 3: Not to expand solution 1 to FR1 band.

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC) [Way forward]: - FFS: - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF. - Note: 3-searchers will be used by UE if UE report this capability. - FFS: Other restriction (triggered by Huawei): - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple) - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple) - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT) - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

We support to follow the previous agreement on solution 3 and keep Option 3 in last meeting (WF R4-2502592). RAN4 should Not to extend solution 3 by introducing network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF, which would restrict both UE and network’s implementation.

Proposal 4: For solution 3, not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

About the applicability of solution 3, the new 3-searcher requirements are not applicable when UE supports per-FR measurement gaps and solution 3 with new 3-searceher capability simultaneously. In particular, UE will follow legacy requirements when all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR.

Proposal 5: The new requirements of solution 3 with 3rd searcher are not applicable when UE supports per-FR measurement gaps and solution 3 with new 3-searceher capability simultaneously.

Conclusions

In this paper we provided our views on CSSF optimization.

Observation 1: The MO (e.g., either for serving cell or neighbor cell) is assumed to configured with report configuration based on RAN2’s spec.

Proposal 1: For solution 1, remove the sub-case and update the agreements as follows:

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

Observation 2: Even without the new signaling indication of a specific SCC per-band for measurement, RAN4’s RRM requirements under the solution 1 are not broken.

Proposal 2: RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update.

Proposal 3: Not to expand solution 1 to FR1 band.

Proposal 4: For solution 3, not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

Proposal 5: The new requirements of solution 3 with 3rd searcher are not applicable when UE supports per-FR measurement gaps and solution 3 with new 3-searceher capability simultaneously.

Reference

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1

- R4-2502662, LS on CSSF optimization for NR RRM Phase 5