# 3GPP TSG-RAN WG4 Meeting #115	R4-2505695

# Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.23.2

Source: 	OPPO, Nokia

Title: 	TP on RAN4 aspects for TR 38.744

Document for:	Approval

- Introduction

In this contribution, we provide the text proposal on the framework of RAN4 part in TR 38.744 to capture the RAN4 agreements.

- Text proposal

< Start of text proposal >

6.2	Interoperability, testability, and RRM requirements

6.2.1 RRM requirements for measurement prediction

6.2.1.1 General

RAN4 has studied RRM requirements of L3 cell-level RSRP (i.e. Point C as defined in Figure 9.2.4-1 in TS 38.300) for RRM measurement prediction, for which scenarios 2,3,4 as defined in Table 5.2.1-1 and corresponding RRM sub-use cases 1,2,3 as defined in section 4.2 are covered. The requirements of L1 beam-level (Point A1 as defined in Figure 9.2.4-1 in TS 38.300) measurement prediction are not considered.

6.2.1.2 Potential RRM requirements

Both absolute RSRP accuracy and relative RSRP accuracy are considered as the candidate metrics for RRM measurement prediction use cases for intra-frequency scenario and inter-frequency scenarios.

The absolute accuracy of predicted L3-RSRP is defined as:

- Absolute Predicted L3-RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP

The relative accuracy of predicted L3-RSRP for intra-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2), where cell 1 and cell 2 are on the same frequency.

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted L3-RSRP of beam 2) – (ground truth of L3-RSRP of beam 1 – ground truth of L3-RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

  - Editor Note: RAN4 may further decide whether this definition for beam-level RSRP accuracy will be needed

The accuracy of predicted relative L3-RSRP for inter-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2. Both cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2. [cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]

  - Editor Note: Depending upon the agreement on ground truth definition, RAN4 may revisit relative RSRP accuracy definition.

  - Editor Note: RAN4 will further discuss whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. Any further agreements will be further captured in this clause.

As baseline, the ground truth of L3-RSRP in FR2 is defined as the reported L3 RSRP measurement result under sufficient high SNR.

The ground truth of L3-RSRP in FR1 is based on:

- Alt1: The transmitted or reception power

- Alt 2: The reported measurement value under certain conditions.

Editor Note: Further down selection between Alt1 and Alt2 is needed.

For intra-frequency temporal domain prediction, the impact on prediction accuracy by UE speed, MRRT and OW/PW length is considered. The number of measurements performed in OW could be also considered.

For inter-frequency prediction scenario, the impact on prediction accuracy by following elements is considered:

- Side condition of frequency prediction (e.g., EPRE difference)

- Correlation coefficient between measured and predicted frequency layers

- Cluster approach e.g.

  - when measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

  - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

For both intra-frequency and inter-frequency prediction, RAN4 thinks RF and baseband errors on the measurements would impact the prediction accuracy.

- Editor Note: RAN4 may continue discussion on the measurement error model in next meetings. Any further agreements will be further captured in this clause

6.2.2 RRM requirements for measurement event prediction

6.2.2.1 General

RAN4 has studied the RRM requirements for both direct and indirect measurement event prediction as defined in section 5.3.

6.2.2.2 Potential RRM requirements

For indirect measurement event prediction, the requirements for the predicted event triggered reporting are considered including [delay and accuracy] requirements and the metric for the event prediction. The requirement for the predicted RSRPs [corresponding to the predicted event occurrence time] is also considered.

Editor Note: For both indirect and direct measurement event prediction, RAN4 may continue discussion on performance metric. Any further agreements will be further captured in this clause.

6.2.3 Testability and Interoperability

6.2.3.1 Testing goal

As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

6.2.3.2 Prediction consistency in time domain

To ensure prediction consistency in time domain, it is considered how to model different time-varying characteristics per cell/site due to moving UE trajectories, e.g., how to incorporate controlled randomness and the extent of time-domain variation and correlation. FR1 is prioritized during the discussion.

6.2.3.3 Testing setup

For the number of cells configured in the testing, two cells are considered including serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction.

Editor Note: RAN4 will further decide if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

<End of text proposal>