3GPP TSG-RAN WG4 Meeting #115	                                                                                       R4-2505696

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.23.3.1

Source: 	OPPO

Title: 	Study of impacts on RAN4 requirements for RRM measurement prediction

Document for:	Approval

- Introduction

In RAN4#114 meeting, the RRM requirements for AI mobility were discussed. The agreements and open issues were captured in the WF [1] and topic summary [2] respectively.

In this contribution, we will further evaluate the impacts on RRM requirements for measurement prediction based on RAN2 progress and current RAN4 discussions, and some background information derived from RAN2 SI is provided in Annex.

- Discussion

Performance Metrics/KPIs for Measurement Prediction use case

Regarding the definition of relative prediction accuracy, the following agreements were reached in RAN4#114bis meeting:

Issue 2-1-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

Based on the agreements in last meeting, the similar definition as legacy is reused for AI based mobility, i.e., the relative accuracy can be defined as the accuracy of RSRP difference between different cells. But we still need to clarify whether to consider the difference between two beams from the same cell and whether to consider the difference between the reported predicted results and reported measured results. Considering we have agreed not to consider L1 beam level measurement in this release, and we prefer not to combine the measurement prediction on time domain and spatial domain, we think we only need to define requirements for L3 cell level measurements. So for the relative accuracy, only the RSRP difference between two cells needs to be considered for both intra-frequency and inter-frequency measurement prediction. And the accuracy of RSRP difference between different beams of the same cell is not needed.

Regarding the difference between predicted results and measured results, since we are discussing prediction accuracy, we prefer not to introduce the impact of legacy measurement, i.e., not to include the measured results in the relative accuracy. If the measured results are considered in the definition and the ground truth is defined as the reported measurement results, the relative accuracy would be totally same as absolute accuracy. And when we say [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2], it may cause misleading that we are comparing the results between AI model input and output which is not reasonable. So we suggest not to define relative accuracy between predicted results and measured results.

In summary, the relative accuracy is defined as below, where the definition of ground truth can refer to the discussion for absolute accuracy.

- Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1– reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1– ground truth of RSRP of cell 2)

Proposal 1: For the definition of relative accuracy for intra-frequency measurement, not to consider RSRP difference between different beams from the same cell.

Proposal 2: For the definition of relative accuracy for inter-frequency measurement, not to consider RSRP difference between predicted results and measured results, i.e., only consider the difference between two predicted results.

Regarding the definition of absolute prediction accuracy and ground truth, the following agreements were reached in previous meeting:

RAN4#113 meeting: Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2 RAN4#114 meeting: Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2. Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list RAN4#114 meeting: Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Based on the discussions in previous meeting, there are two approaches to define the ground truth, where alt1 is to use the expected measurement results calculated based on transmission power and channel modelling and alt2 is to use the reported measurement results under certain conditions.

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

For alt1, the advantage is that it is easily calculated and TE side can clearly know the expected results at each timing point, but the issue is that TE does not know when the measurement is performed or predicted. Therefore, TE doesn’t know which one to be compared which may request additional report from UE on the time stamp. The other issue is that the performance requirements may be used in model monitoring in LCM procedure, we understand it may cause problem since UE cannot acquire the expected measurement results in real LCM implementation. In addition, we understand the model input and training data is based on the actual measurement results, thus the output is usually close to the UE measurement.

For alt2, according to the existing measurement model, from inputs at point A, UE needs to experience L1 filter, beam consolidation/selection and L3 filter to get L3 measurement results at point C in which the latter two parts are RRC configured. Based on the legacy procedure, when the SINR is high enough, the UE measurement results are approximate to the ideal values. The advantage is that this can accommodate different UE implementation e.g., L1 filtering, and can be aligned with model training and inference procedure. To define the prediction accuracy and testing, certain channel modelling, beam consolidation configuration and L3 filter configuration can be considered as possible side conditions. The issue for alt2 is that UE may not perform actual measurement on certain occasions in some of the prediction cases, but we think this can be resolved by setting additional procedure in the testing. The other concern is that both ground truth and report are from UE which makes UE possible to cheat in testing. But we understand the actual measurement used as ground truth still needs to meet the legacy requirements, and cheating in the testing is not benefit for UE since it may lead bad performance in real practice. So we do not need to assume that UE will cheat in the testing.

Based on the comparison of the two approaches above, the advantages and disadvantages are summarized as below and we prefer to use alt2 as baseline.

|  | Advantages | Disadvantages | Possible solutions |
| --- | --- | --- | --- |
| Alt 1 | - Easily to acquire and to be known by TE side | - TE does not know when the measurement is performed or predicted - May not be used in model monitoring in LCM procedure - May not be aligned with model training - Not consider the UE implementation inaccuracy | - UE report on time stamp |
| Alt 2 | - The reported results are approximate to the ideal values under high SINR - Can be used in model monitoring in LCM procedure - Aligned with model training - Accommodate the UE implementation inaccuracy | - Condition setting (e.g., continuous or time varying high SINR) - Acquire UE actual measurement for reduction case - Avoid UE cheating | - Control transmit power to adapt to the channel modeling - Additional procedure under same testing environment to acquire actual measurement |

Proposal 3: Use Alt2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1.

Proposal 4: RAN4 to consider certain channel model, beam consolidation configuration and L3 filter configuration as possible side conditions when defining the accuracy requirements and testing.

Impacts of measurement errors on the prediction accuracy

Issue 2-1-7: Impacts of measurement error on prediction accuracy WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

In last meeting, the impact of measurement error on prediction accuracy was discussed and companies are encouraged to bring proposals on how to consider the error models. According to the discussion in AI/ML BM and the simulation assumption in [3], the following assumption is used for RF error and baseband error:

- Simulation assumptions for RF error The impact of RF errors should be considered. Following assumptions can be used to generate RF errors. - Truncated Gaussian distribution under ±4.5 dB RF error (μ=0, σ2=4.5) is used - Each TX beam are Independent - Use the LLS simulation assumptions defined in Table 1-5 to generate L1-RSRP difference as the baseband errors.

For AI mobility, we think the similar approaches can be reused for RF error i.e., use the truncated Gaussian distribution. For baseband error, we understand the following two alternatives can be considered and can be decided in WI stage according to how to perform the simulation for prediction accuracy.

  - Alt 1: Use the link level simulation assumptions to generate L3-RSRP difference as baseband error.

  - Alt 2: Use the truncated Gaussian distribution under ±X dB as baseband error.

On the other hand, for L3 measurement prediction, maybe the RF and baseband error can be considered together for simplification, e.g., use a single model as the sum of RF and baseband error.

Proposal 5: Consider the following options to model the measurement error:

  - Option 1: For RF error, use the truncated Gaussian distribution under ±X dB; for baseband error, use the link level simulation assumptions to generate L3-RSRP difference as baseband error.

  - Option 2: For RF and baseband error, use the truncated Gaussian distribution under ±X dB and ±Y dB, respectively.

  - Option 3: For the sum of RF and baseband error, use the truncated Gaussian distribution under ±X dB.

Impacts on Core requirements

In RAN4#112bis meeting, RAN4 agreed to start the discussion for scenario 2, 3, 4 as listed below:

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Based on the conclusions in RAN2, UE measurement behaviour can be different from legacy, e.g., UE can skip some measurement based on different MRRT values or OW/PW values for intra-frequency measurement prediction or UE can skip some measurement on certain frequency for inter-frequency measurement prediction.

From RAN4 perspective, the measurement period requirements can be different according to the skipped measurements, e.g., the skipped SMTC occasions may not cause scheduling restrictions and the skipped frequencies may not need to be counted in the CSSF calculation.  The impact of skipped measurement instances and the corresponding measurement period requirements can be discussed in WI stage.

Proposal 6: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

Proposal 7: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

Proposal 8: RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

Proposal 9: RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction.

LCM related requirements

In RAN2#129bis meeting, the following agreements are reached:

General LCM: Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM).

Based on the agreements, the functionality-based LCM is considered for AI mobility and the general LCM framework for beam management can be the baseline. Considering the LCM related requirements for AI BM is still under discussion and the detailed LCM procedure for AI mobility is not clear, we think we can wait for more progress from RAN1/2 AI mobility and RAN4 AI BM discussion to decide the LCM related requirements.

Proposal 10: Wait for more progress from RAN1/2 AI mobility and RAN4 AI BM discussion for LCM related requirements.

- Conclusions

In this contribution, we discussed the possible RAN4 requirements for RRM measurement prediction and got the following observations and proposals:

Proposal 1: For the definition of relative accuracy for intra-frequency measurement, not to consider RSRP difference between different beams from the same cell.

Proposal 2: For the definition of relative accuracy for inter-frequency measurement, not to consider RSRP difference between predicted results and measured results, i.e., only consider the difference between two predicted results.

Proposal 3: Use Alt2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1.

Proposal 4: RAN4 to consider certain channel model, beam consolidation configuration and L3 filter configuration as possible side conditions when defining the accuracy requirements and testing.

Proposal 5: Consider the following options to model the measurement error:

  - Option 1: For RF error, use the truncated Gaussian distribution under ±X dB; for baseband error, use the link level simulation assumptions to generate L3-RSRP difference as baseband error.

  - Option 2: For RF and baseband error, use the truncated Gaussian distribution under ±X dB and ±Y dB, respectively.

  - Option 3: For the sum of RF and baseband error, use the truncated Gaussian distribution under ±X dB.

Proposal 6: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

Proposal 7: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

Proposal 8: RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

Proposal 9: RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction.

Proposal 10: Wait for more progress from RAN1/2 AI mobility and RAN4 AI BM discussion for LCM related requirements.

- References

R4-2504901 WF on RRM requirements for FS_NR_AIML_Mob

R4-2503620 Topic summary for [114bis][210] FS_NR_AIML_Mob

R4-2505152 Updated simulation assumptions for AI/ML BM

RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RP-240082 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

Annex: Background information

In RAN#102 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR is approved. In RAN#105 meeting, the related work plan in RAN4 has been further checked and updated as follows:

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

Some background information derived from RAN2 SI is as follows:

To evaluate potential benefits and gains of AI/ML aided mobility, following aspects are agreed in SID[1] for study:

- Measurement prediction (including UE sided and NW sided model)

- HO failure/RLF prediction (only UE sided model)

- Measurement events prediction (only UE sided model)

And as clarified in RAN2 on TR 38.744, the study of the use cases is driven mainly by two study goals.

- The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.

- The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

Regarding the RRM measurement prediction

- 3 sub-use cases are considered for cell-level RRM measurement prediction

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Temporal/frequency/spatial domain measurement prediction

- For temporal domain measurement prediction,

In RAN2#125bis meeting, it has been agreed as:

- For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predication.

In addition, based on the post email discussion after RAN2#127 meeting, temporal domain case A and case B have been further clarified as:

Intra-frequency temporal domain case A: continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding.

Figure 1

Intra-frequency temporal domain case B: measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding

Figure 2-1 temporal domain case example 1

Figure 2-2 temporal domain case example 2

- For frequency domain measurement prediction, typical feature is to use the measurement result from one frequency to predict the measurement result at another frequency, which can be applicable to FR1 to FR1 cases in R19 evaluation.

- For spatial domain measurement prediction, typical feature is to predict cell-level measurements through measurements of partial beams, which can be applicable to sub-use case 1 and 3 in R19.