# 3GPP TSG-RAN WG4 Meeting #115	R4-2505700

# Malta, MT, 19th – 23rd May, 2025

Title: 	On RRM impacts for SBFD operation

Source: 	OPPO

Agenda item:	7.25.4.2

Document for:	Discussion

Introduction

In last meeting, a WF on RRM requirements for NR_duplex_evo was approved [1].

In this contribution, we will provide our views on RRM impacts for SBFD operation.

Discussion

- Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements (adhoc session on Thursday with additions) - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE.

For CSI-RS L1 measurement with puncturing in frequency domain, according to the agreements in last meeting, for L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs.

RAN4 agreed to consider the degradation in the accuracy in case that the CSI-RS resource is not contiguous in frequency domain and the number of CSI-RS RBs in each DL subband is less than 48. And RAN4 may continuous to discuss the degraded accuracy based on 24 PRBs in this meeting. Expect defining new accuracy based on evaluation, we are also ok to reuse the logic of CSI-RS based L3 measurement that UE reports with fulfilling the legacy measurement reporting delay requirements but the accuracy requirements are not expected to be guaranteed.

Proposal 1: For Case 3 of CSI-RS L1 measurement with puncturing in frequency domain, the UE is not required to meet the CSI-RS based L1 measurement accuracy requirements for CSI-RSRP and CSI-SINR if the UE reports CSI-RS based L1 measurements with fulfilling the existing delay requirements.

Another open issue is to discuss whether and how to account for the UE capability on CSI processing time. If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements. We are ok to Option 1 by adding a lower bound on top of existing delay requirements in order to guarantee the minimum requirement can be met by UE. Looking into current test cases, the common value is TCSI_Reporting = 10ms which is for UE processing time for CSI reporting. If needed, we suggest to add a lower bound of 10ms on top of existing delay requirements.

Proposal 2: If the UE capability on CSI processing time was agreed and CSI processing time was indicated to be scaled by 2, ok to add a lower bound of 10 ms.

- Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - Agreements - For L1-RSRP and L1-SINR - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network - For L1-RSRP and L1-SINR measurement period, further discuss the following options. - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. - Option 2: other - For RLM/BFD/CBD, further discuss the following options - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type - Existing measurement period apply - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

According to RAN1 agreements, CSI-RS based L1-RSRP/L1-SINR measurement will differ resource occasions on different symbol types, i.e., resource occasions on both SBFD and non-SBFD symbols. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation, and the valid symbol type will be configured in CSI-ReportConfig for each report configuration.

Under SBFD operation, the measurement period should only count resource occasions on the valid symbol type, or in other words, the measurement period should be extended by excluding the resource occasions on the invalid symbol type. We are ok to follow the R16 NR-U approach to extend the measurement period due to the CSI-RS resource occasions on invalid symbol type.

Proposal 3: For L1-RSRP and L1-SINR measurement on different symbol types, RAN4 to follow the R16 NR-U approach to extend the measurement period considering the CSI-RS resource occasions on invalid symbol type.

Conclusions

In this paper we provided our views on RRM impact for SBFD operation.

Proposal 1: For Case 3 of CSI-RS L1 measurement with puncturing in frequency domain, the UE is not required to meet the CSI-RS based L1 measurement accuracy requirements for CSI-RSRP and CSI-SINR if the UE reports CSI-RS based L1 measurements with fulfilling the existing delay requirements.

Proposal 2: If the UE capability on CSI processing time was agreed and CSI processing time was indicated to be scaled by 2, ok to add a lower bound of 10 ms.

Proposal 3: For L1-RSRP and L1-SINR measurement on different symbol types, RAN4 to follow the R16 NR-U approach to extend the measurement period considering the CSI-RS resource occasions on invalid symbol type.

Reference

- R4-2504904, WF on RRM requirements for NR_duplex_evo