# 3GPP TSG-RAN WG4 Meeting #115	R4-2505712

# Malta, MT, 19th – 23rd May, 2025

Title: 	On CSI-RS based L1 measurements

Source: 	OPPO

Agenda item:	7.29.3.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on remaining issues of CSI-RS based L1 measurements.

Discussion

- Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 Agreement: - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting. - Further discuss whether step 2 can be skipped for serving cell or not. Agreement: - For a UE not supporting the capability, Step 2 is necessary. - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2. - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped: - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1. - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

In the regular beam management procedure, 3 steps are needed for measurement on serving cell as follows.

- P-1 (wide gNB Tx & wide UE Rx): SSB based BM (implicit/explicit SSB-ID report to gNB)

- P-2 (narrow gNB Tx & wide UE Rx): On CSI-RS resources with ‘repetition = off’, UE applies a fixed Rx beam obtained from P1 and measures/reports CRI and L1-RSRP.

- P-3(narrow gNB Tx & narrow UE Rx): On CSI-RS resources with ‘repetition = on’, UE sweeps its Rx beam with a finer beamwidth, desirably confined within the P1/P2 Rx beamwidth. No report is required.

RAN1 already agreed not to support Repetition=on for candidate cell CSI-RS in Rel-19. Since CSI-RS Repetition ON is extremely rare to be configured due to the large overhead burden in practical network, it is typically assumed CSI-RS resources repetition set to OFF. Thus, P-3 is typically NOT executed in beam management procedure where CSI-RS resources is transmitted repetition ON.

RAN1 Agreement - Repetition=off is supported for candidate cell CSI-RS in Rel-19. - Repetition=on is not supported for candidate cell CSI-RS in Rel-19. Conclusion RAN1 assumes the SSB QCLed with CSI-RS is used for timing reference of candidate cell CSI-RS measurement in LTM.

However, RAN4 also agreed to introduce new UE capability on whether step 2 can be skipped for neighbor cell or not, but still needs to decide whether step 2 can be skipped for serving cell or not. If the candidate LTM cell is a serving cell, the UE capability of skipping SSB based L1 measurement in step 2 can also apply.  If UE supports the capability, CSI-RS-based L1-RSRP measurement in step 3 can be conducted without an intermediate SSB-based L1-RSRP measurement in step 2. Otherwise, for a UE not supporting the capability, step 2 is necessary. For gNB-UE beam management on a serving cell, the UE could still use SSB based L1-RSRP measurement for UE Rx beam refinement for the serving cell.

Proposal 1: Whether step 2 is skipped for serving cell or not can also be based on the same UE capability.

- Issue 4-4: CSI-RS based intra-frequency/inter-frequency measurement definition Candidate options: - Option 1: Apple, CMCC A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: MediaTek, ZTE, Ericsson, Nokia, Qualcomm A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP. - Option 3: Huawei If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference. If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred. - Option 4: vivo - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource are confined within the active DL BWP. - Issue 4-10: Definition of frequency layer Agreements: - No need to introduce definition of frequency layer.

Since RAN4 already agreed not to introduce definition of frequency layer, we propose not to further discuss the definition of CSI-RS based intra-frequency/inter-frequency measurement. Intra-frequency or inter-frequency layer are not needed since RAN4 will only define requirements for CSI-RS based L1 measurement without gap, but RAN4 defines the applicability rule or conditions for the corresponding requirements.

We suggest Option 2 can be baseline that

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

      - It is applied for SCS = 60KHz

    - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Proposal 2: No need to define CSI-RS based intra-frequency/inter-frequency layer and measurement and close this issue.

Proposal 3: Define the applicability and conditions for requirements of CSI-RS based L1 measurement without gap:

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

      - It is applied for SCS = 60KHz

    - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

- Issue 4-12: Early CSI acquisition on a candidate cell Candidate options: - Option 1: Qualcomm - For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements. - For the optional CSI acquisition (DL CSI acquisition before LTM CSC MAC-CE reception), RAN4 to discuss whether and what requirements to define after the following aspects for CSI-RS based L1-RSRP measurement from the candidate cell are concluded: - Pre-requisite conditions, maximum RTD value, gap-less vs. gap-based, etc. - Option 2: Nokia - Discuss what is the point in time during the cell switch delay UE starts CSI-acquisition.

Per RAN1 agreements, after the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability. But RAN1 haven’t discuss how to select CSI-RS resource to measure and when to measure. On the other hand, for CSI acquisition after LTM CSC MAC-CE reception, UE determines the CSI report configuration based on the CSC, and measure CSI-RS resource(s) associated with determined CSI report configuration. RAN1 also agreed that the report is sent to the target cell by MAC CE.

RAN4 needs to discuss whether and what requirements to define for DL CSI acquisition. The CSI-acquisition may start before or after the cell switch, depending on the time the UE receives the CSI-RS associated with CSI report configuration. Anyway, the CSI will be reported to the target cell after the LTM cell switch execution.

For CSI acquisition after LTM CSC MAC-CE reception, we also see no need to define new requirements for CSI acquisition. If RAN1 agreed single CSI report configuration is configured for a candidate cell or multiple CSI-RS resources for CMR can be associated with the CSI report configuration and the configuration of CSI-RS for candidate cells can be known by UE before cell switch command, UE may start the early CSI-acquisition. For CSI acquisition before LTM CSC MAC-CE reception, RAN4 can defined the start of early CSI-acquisition and corresponding pre-requisite conditions. The legacy CSI-acquisition delay from UE side can be kept unchanged.

Proposal 4: For early CSI acquisition before LTM cell switch command MAC-CE, RAN4 considers to define the potential requirements including the start time and corresponding pre-requisite conditions.

Agreement: RAN1#120bis [FL proposal 5-3-1-v3] Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability Agreement Reply LS on number of beam measurements in the measurement report MAC CE is agreed. Agreement: RAN1#120 For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed. Working Assumption: RAN1#119 As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

Conclusion

In the contribution we provide the following proposals:

Proposal 1: Whether step 2 is skipped for serving cell or not can also be based on the same UE capability.

Proposal 2: No need to define CSI-RS based intra-frequency/inter-frequency layer and measurement and close this issue.

Proposal 3: Define the applicability and conditions for requirements of CSI-RS based L1 measurement without gap:

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

      - It is applied for SCS = 60KHz

    - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Proposal 4: For early CSI acquisition before LTM cell switch command MAC-CE, RAN4 considers to define the potential requirements including the start time and corresponding pre-requisite conditions.

Reference

- R4-2504965, WF on NR mobility enhancements Phase 4_part1