3GPP TSG-RAN WG4 Meeting #115							R4-2505718

Malta, MT, 19th – 23rd May 2025

Agenda Item:		7.11.3

Source: 			Intelsat

Title: 			Ku Band Co-existence: Performance of 45x45 and 78x78 Element

Phase Array Antennas

Document for:		Discussion and Decision

# Introduction

Following from previous RAN4 meetings #114-bis, held in Wuhan; to progress the work on the Ku band co-existence analysis, several agreements were reached on simulation scenarios, phased array VSAT and satellite parameters. These agreements reached can be found in [1]

The normative work on the Ku band, in keeping with modern antenna deployments and practical use case scenarios, incorporates electrically steered phase array antennas as well as parabolic antenna.

For the co-existence simulations and analysis, it was agreed to use two types of phased array antennas that would provide connectivity to LEO and GEO satellites. These antennas are:

- A 45x45 element phase array antenna

- A 78x78 element phase array antennas

And the connectivity to satellites as per agreement in [1] are shown in the following Table 1

Table 1 Phase Array VSATs Connectivity to LEO and GEO Satellites [1]

| VSAT model SAN | 45 x 45 Phased Array (1st step for coexistence) | 60 cm Parabolic (1st step for coexistence) | 78 x 78 Phased Array |
| --- | --- | --- | --- |
| GEO | X | X | X |
| LEO 600km | X | X |  |

The rationale behind selecting two types of antennas were that both types reflect practical and modern implementation scenarios that can be found in the field.

Furthermore, the characterisation of the phase array antennas is the first basic step towards building the co-existence simulation environment, where the performance of the phase array antennas is studied as an aggressor (interferer) or as the victim in presence of mobile UE and base station. From these interactions the ACLR and ACS of the Ku Band system can be extracted.

Moreover, to simulate two extreme cases, it was decided to simulate the antenna performance at 90 degree and at 35-degree elevations above the horizon: in the former case the GEO satellite is directly above the phase array and in the latter case the phase array is electrically steered to 55 degrees to point to a GEO satellite at 35-degree elevation.  See following Figure 1 and Figure 2

Figure 1 Phase Array VSAT Antenna Pointing to GEO Satellite at 90 Degrees Elevation

Figure 2 Phased Array Antenna, Electrically Steered to Point to a GEO Satellite at 35 Degree Elevation

During the antenna simulation work it became apparent that the choice of radiation mask, either FCC or CEPT, plays an important role in defining the performance of the phase array antennas. As illustrated in the proceeding sections, the output power or the transmitted bandwidth is adjusted to comply with either FCC or CEPT masks.

The FCC and CEPT masks are defined in the Technical Annex (see section 5). For GEO satellite, it is essential that the phase array antenna’s EIRP is below the mask. We will use these masks to determine the maximum power allowed at different scan angles assuming the lowest carrier bandwidth of 10 MHz.

# Phase Array Antenna: 45x45 Element

The 45x45 element phased array antenna key parameters are summarized below:

- Element gain: 3.5 dBi

- Array gain: 33.1 dB

- Antenna gain: 36.6 dBi

- Power: 10W

- Ohmic loss: 2 dB

- EIRP: 74.6 dBm (includes ohmic loss)

The same peak normalized element radiation equation used for the TN base station is also used here

Aθ,φ=-min--min12φφ3dB2,Am-min12θθ3dB2,Am,Am

Equation 1

where the various parameters are defined in [2]. At boresight, the phase array antenna is pointing straight up at the sky.  The 3D directivity pattern of the element gain is shown in Figure 3 with 3.5 dBi element gain.

Figure 3: 3D Directivity Pattern with 3.5 dBi element gain.

The 45x45 element array is shown in Figure 4 assuming centre-to-centre half wavelength at 14.5 GHz.

Figure 4: 45x45 Element Phase Array Antenna.

Following Figure 5 illustrates the 45x45 element phased array antenna gain and EIRP density at 0 deg scan (90 deg elevation angle). The antenna gain is 36.3 dBi at 14 GHz. With 10 MHz bandwidth and 10W power, the phased array antenna EIRP density is below the FCC and CEPT mask.

Figure 5: 45x45 element @ 0 deg scan: Antenna gain and EIRP density.

Following Figure 6 illustrates the 45x45 element phased array antenna gain and EIRP density at 55 deg scan (35 deg elevation angle). The antenna gain is 33 dBi at 14 GHz. With 10 MHz bandwidth and 10W power, the EIRP density is below the FCC and CEPT mask.

Figure 6: 45x45 element @ 55 deg scan: Antenna gain and EIRP density.

Following Table 2 provides the uplink EIRP for 45x45 element phased array antenna with different masks and scan angles.

Table 2: 45x45 uplink EIRP @ 14 GHz for 10 MHz Bandwidth.

|  |  | EIRP (dBm) |  |
| --- | --- | --- | --- |
| Scan Angle off-boresight (deg) | Elevation Angle (deg) | FCC mask | CEPT mask |
| 0 | 90 | 74.3 | 74.3 |
| 55 | 35 | 71 | 71 |

Following Table 3 provides the antenna gain for 45x45 element phased array antenna at different operating frequencies and scan angles.

Table 3: 45x45 Element Antenna Gain @ 11 GHz and 14 GHz.

|  |  | Antenna Gain (dBi) |  |
| --- | --- | --- | --- |
| Scan Angle off-boresight (deg) | Elevation Angle (deg) | Downlink Freq: 11 GHz | Uplink Freq: 14 GHz |
| 0 | 90 | 34.2 | 36.3 |
| 55 | 35 | 30.9 | 33.0 |

Proposal 1: For the 45x45 phase array element antenna, to proceed with 10 W output power at 90 degree and 35-degree elevation angles, as antenna performances are compliant with FCC and CEPT masks.

# Phase Array Antenna: 78x78 Element

The 78x78 element phased array antenna key parameters are summarized below:

- Element gain: 3.5 dBi

- Array gain: 37.8 dB

- Antenna gain: 42.3 dBi

- Power: 50W

- Ohmic loss: 2 dB

- EIRP: 86.3 dBm (includes ohmic loss)

The 78x78 element array is shown in Figure 7 assuming center-to-center half wavelength at 14.5 GHz.

Figure 7: 78x78 Element Phase Array Antenna.

Following Figure 8 illustrates the 78x78 element phased array antenna gain and EIRP density at 0 deg scan (90 deg elevation angle). The antenna gain is 41 dBi at 14 GHz. With 10 MHz bandwidth and 50 W power, the EIRP density is above the FCC mask.

Following Figure 9 illustrates that the power must be reduced to 18W for FCC mask to be compliant.

Figure 9: 78x78 element @ 0 deg scan: Antenna gain and EIRP density (reduced power).

Proposal 2: We propose that for 78x78 element array, the EIRP be reduced by 4.4 dB at 90 degrees elevation to be FCC mask compliant with 10 MHz bandwidth.

Following Figure 10 illustrates that the bandwidth must be reduced to 30 MHz minimum for FCC mask to be compliant keeping the power to 50 W.

Figure 10: 78x78 element @ 0 deg scan: Antenna gain and EIRP density (increased

Proposal 3: We propose that for 78x78 element array, the bandwidth be increased to 30 MHz at 90 degrees elevation to be FCC mask compliant with 50 W power.

Proposal 4: We propose that for 78x78 element array, the peak EIRP density be 41.6 dBW/MHz at 90 degrees elevation to be FCC mask compliant.

Following Figure 11 illustrates the 78x78 element phased array antenna gain and EIRP density at 55 deg scan (35 deg elevation angle). The antenna gain is 37.5 dBi at 14 GHz. With 10 MHz bandwidth and 50 W power, the EIRP density is above both masks.

Figure 11: 78x78 element @ 55 deg scan: Antenna gain and EIRP density.

Figure 12 illustrates that the power must be reduced to 10W and 20W respectively for the FCC mask and CEPT mask to be compliant with 10 MHz bandwidth.

Figure 12: 78x78 element @ 55 deg scan: Antenna gain and EIRP density (reduced power).

Proposal 5: We propose that for 78x78 element array, the EIRP be reduced by 7 dB at 35 degrees elevation to be FCC mask compliant with 10 MHz bandwidth.

Proposal 6: We propose that for 78x78 element array, the EIRP be reduced by 4 dB at 35 degrees elevation to be CEPT mask compliant with 10 MHz bandwidth.

Figure 13 illustrates that the bandwidth must be increased to 50 MHz and 25 MHz respectively for the FCC mask and CEPT mask to be compliant assuming 50 W power.

Figure 13: 78x78 element @ 55 deg scan: Antenna gain and EIRP density (increased bandwidth).

Proposal 7: We propose that for 78x78 element array, the bandwidth be increased to 50 MHz at 35 degrees elevation to be FCC mask compliant with 50 W power.

Proposal 8: We propose that for 78x78 element array, the bandwidth be increased to 25 MHz at 35 degrees elevation to be CEPT mask compliant with 50 W power.

Proposal 9: We propose that for 78x78 element array, the peak EIRP density be 35.5 dBW/MHz at 35 degrees elevation to be FCC mask compliant.

Proposal 10: We propose that for 78x78 element array, the peak EIRP density be 68.5 dBW/MHz at 35 degrees elevation to be CEPT mask compliant.

Following Table 4 provides the uplink EIRP for 78x78 element phased array antenna at different masks and scan angles.  Without power reduction, the uplink EIRP for 0 and 55 scan angles are respectively 86 dBm and 82.5 dBm.

Table 4: 78x78 uplink EIRP @ 14 GHz for 10 MHz Bandwidth.

|  |  | EIRP (dBm) |  |
| --- | --- | --- | --- |
| Scan Angle off-boresight (deg) | Elevation Angle (deg) | FCC mask | CEPT mask |
| 0 | 90 | 81.6 | 86 |
| 55 | 35 | 75.5 | 78.5 |

Following Table 5 provides the antenna gain for 78x78 element phased array antenna at different operating frequency and scan angles.

Table 5: 78x78 Element Antenna Gain @ 11 GHz and 14 GHz.

|  |  | Antenna Gain (dBi) |  |
| --- | --- | --- | --- |
| Scan Angle off-boresight (deg) | Elevation Angle (deg) | Downlink Freq: 11 GHz | Uplink Freq: 14 GHz |
| 0 | 90 | 38.9 | 41 |
| 55 | 35 | 36.0 | 37.5 |

# Conclusion and Proposals

In this contribution, we have provided the performance of a 45x45 and a 78x78 element phased array antenna at different scan angles, power levels, and bandwidth. Moreover, we have provided the maximum power levels or minimum bandwidth levels that result in compliance with FCC and CEPT masks.

From the analysis provided in this contribution we propose the following:

Proposal 1: For the 45x45 phase array element antenna, to proceed with 10 W output power at 90 degree and 35-degree elevation angles, as antenna performances are compliant with FCC and CEPT masks.

Proposal 2: We propose that for 78x78 element array, the EIRP be reduced by 4.4 dB at 90 degrees elevation to be FCC mask compliant with 10 MHz bandwidth.

Proposal 3: We propose that for 78x78 element array, the bandwidth be increased to 30 MHz at 90 degrees elevation to be FCC mask compliant with 50 W power.

Proposal 4: We propose that for 78x78 element array, the peak EIRP density be 41.6 dBW/MHz at 90 degrees elevation to be FCC mask compliant.

Proposal 5: We propose that for 78x78 element array, the EIRP be reduced by 7 dB at 35 degrees elevation to be FCC mask compliant with 10 MHz bandwidth.

Proposal 6: We propose that for 78x78 element array, the EIRP be reduced by 4 dB at 35 degrees elevation to be CEPT mask compliant with 10 MHz bandwidth.

Proposal 7: We propose that for 78x78 element array, the bandwidth be increased to 50 MHz at 35 degrees elevation to be FCC mask compliant with 50 W power.

Proposal 8: We propose that for 78x78 element array, the bandwidth be increased to 25 MHz at 35 degrees elevation to be CEPT mask compliant with 50 W power.

Proposal 9: We propose that for 78x78 element array, the peak EIRP density be 35.5 dBW/MHz at 35 degrees elevation to be FCC mask compliant.

Proposal 10: We propose that for 78x78 element array, the peak EIRP density be 68.5 dBW/MHz at 35 degrees elevation to be CEPT mask compliant.

To progress with the co-existence simulation work, we strongly recommend selecting one combination from the simulation scenarios presented in this document for 78x78 element antenna.  Recommend power reduction: Proposal 2 (boresight), Proposal 5 (35-deg elevation), and Proposal 6 (35-deg elevation).

We propose to use the simulation plots provided as the basis for the co-existence analysis of the Ku band.

# Technical Annex

## Off-axis EIRP Density Mask

Both FCC and CEPT have defined off-axis EIRP density mask that the terminal must be below at Ku band uplink [3,4,5] compares the FCC and CEPT masks for the same reference bandwidth and power definition. The FCC mask is clearly more stringent compared to the CEPT mask. The FCC and CEPT masks are only applicable for GEO. For LEO constellation, the mask is based on complicated EPFD calculations. The definition for the FCC mask and CEPT mask are provided in the next two sections.

Figure 14: FCC versus CEPT off-axis EIRP density mask.

## FCC mask

The FCC off-axis EIRP density envelopes for FSS earth stations transmitting in Ku band frequency is provided.

## CEPT mask

The CEPT off-axis EIRP density envelopes for FSS earth stations transmitting in Ku band frequency is provided.

# Reference

[1] R4-2504723

[2] R4-2501398 Section 8.

[3] ETSI TR 103 896

[4] ECC Report 271

[5] Rec. ITU-R S.524-6