3GPP TSG-WG RAN4 Meeting #115	R4-2505742

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.26.5

Source: 	Qualcomm Incorporated

Title: 	Discussion on Ambient IoT Testability

Document for:	Approval

# 1. Introduction

In RAN4#114bis, the agreements for A-IoT device OTA test method were captured in [1]. In this paper, we provide further views on the OTA test methodology for A-IoT devices.

# 2  OTA test methdology for A-IoT device

## 2.1 Test setup for A-IoT device testing

As agreed in [1], RAN4 will further discuss the positions of CW antenna

Issue 4-2-1: testing antenna and CW antenna relationship Agreement: Whole sphere scan is supported The position of CW antenna is a set of fixed position(s) as baseline (depending on core requirements discussion). - FFS on relative positions between reader antenna and CW

Two examples of FR1 TRP and TRS OTA test system are presented in TS 38.161 [2] as below figures.

Figure 1: Example of a FR1 TRP TRS OTA test system with combined axis [2]

Figure 2: Example of a FR1 TRP TRS OTA test system with distributed axis

For the test system with distributed axis, one implementation is to use multiple test antennas to support θ range. For this implementation, it is easy to extend to support multiple candidate CW nodes. For example, expect the test antenna, the other test antennas can be used as CW node. While if only one test antenna is equipped in the test systems as shown in Figure 1 and Figure 2, it will increase the complexity of test system very much to support more than one CW node positions.

To support testing FR2 multi-Rx performance, the multi-AoA measurement setup was specified in TR 38.871 in which 2 AoA RRM testing defined in Clause B.2 of 38.508-1or FR2 MIMO testing defined in Clause B.2 of 38.151, can be used as the baseline setups. Multi-AoA measurement setup can naturally support multiple CW positions, but it would be too complicated and expensive for low-cost AIoT device conformance testing.

With that, we have the following proposal for test setup and relative position(s) between reader antenna and CW:

Proposal 1: To reuse the OTA test system for legacy TRP and TRS measurement, only one fixed separation between the reader antenna and CW node should be implemented. The separation value is FFS. RAN4 to check whether the link antenna in Figure 1 and Figure 2 can be used to transmit CW signal.

Another aspect is that the test setup should ensure the isolation between CW node and reader/measurement antenna.

Issue 4-2-2: how to ensure the isolation between CW node and measurement antenna Agreement: - Further discuss power difference between backscatter signal and the CW signal leakage, e.g. 6dB, 12dB, etc. - Encourage TE vendors provide the feasibility of TE extracting D2R power when CW source is transmitting.

The side conditions for CW signal level needs to be discussed case by case. For example, for the Rx sensitivity testing, CW signal level needs to be set as a min value, while for the spurious emission or maximum input level testing, a max value might be needed [4]. If we assume the target SNR of D2R as SNRD2R, and the residual CW is XdB lower than D2R signal level. The actual SNR of D2R with residual CW leakage can be presented as:

SNRD2R_CW = SNRD2R/ (SNRD2R*(1/XLinear) +1)

The isolation XdB can be derived by a target difference between SNRD2R_CW and SNRD2R. For the worst case, i.e., SNRD2R = 1dB assuming min CW signal is set to achieve 1dB SNR at D2R and target different between SNRD2R_CW and SNRD2R is [0.1dB]. Then we can get X is about 17dB. Note that here 17dB is the total isolation including the spatial isolation with the help of measurement antenna gain, isolation due to the distance between measurement antenna and CW node, etc.

Proposal 2:  It is proposed to take [17dB] as the starting point for the isolation between CW node and measurement antenna.

## 2.2 Test methodology for Tx RF requirements of A-IoT device

RAN4 will specify power backscattering loss, Emission requirements, etc., for device 1 as the Tx RF requirements. The baseline test procedure for backscattering loss was agreed in [1] as below:

Issue 4-5-3: Test procedure for Tx RF requirements Agreement: - Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc. - Step 2: Place DUT based on UE positioning guidelines. - Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions. - Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB” - Step 5: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1 - Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid. - Step 7: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP) - Step 8: FFS on performance metric. FFS on how to specify the performance metric.

For Step 4, the input power level of CW signal is still undecided. It could be one value or a range of CW Tx power as agreed in Issue 3-1-1 in [1]. The test configuration for input power level of CW signal should be decided together with core requirement definition for backscattering loss. On top of input power level of CW signal, the power level of R2D signal also needs to be specified as one of test conditions. Therefore, the power level of R2D at device receiver should be set as sensitivity+[X]dB to avoid device failing the test due to not successfully decoding R2D signal. X can be set as 3dB as the starting point.

Proposal 3: In Step 4 of test procedure for backscattering loss, the test configuration for input power level of CW signal should be decided together with core requirement definition for backscattering loss. Additionally, the power level of R2D should be set as sensitivity+[X]dB. Take X = [3] as the starting point.

For Step 7, the performance metric of backscattering loss is another open issue. Per our understanding, taking TRP as the metric is the equivalent to the conductive power which is more reasonable. While it is also well known that EIRP can simplify the test procedure and reduce the testing time. From testing point of view, reducing the testing time is very important for A-IoT device testing since there is no external energy source. Therefore, if EIRP is selected as the performance metric for backscattering loss testing, the test direction should be declared by device to avoid 3D scan to find the peak/best direction.

Proposal 4: If EIRP is selected as the performance metric for backscattering loss testing, the test direction should be declared by device to avoid the 3D scan to find the peak/best direction.

## 2.3 Test methodology for Rx RF requirements of A-IoT device

It was agreed that reference sensitivity shall be specified for A-IoT device. Reference sensitivity is to verify the received performance of R2D link. While different from legacy NR device testing, it is not feasible to measure the reference sensitivity for R2D link via throughput metric, e.g., >95% of maximum throughput. Then the following agreements were reached in [1] regarding the test performance metric and test procedure for REFSENS:

Issue 3-4-1: Reference sensitivity Agreement: - Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies - Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory - At least to define maximum gain direction sensitivity requirements - FFS on how to different sensitivity requirements on different orientations

Issue 4-5-1: testing performance metric of reference sensitivity Agreement: - Use following as starting point: - Measuring device’s correct response and correct respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna. - FFS on how to set the side condition including CW signal power.

Issue 4-5-2: Test procedure for receiver RF requirements Agreement: - Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc. - Step 2: Place DUT based on UE positioning guidelines. - Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions. - Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level. - Step 5: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min - Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid. - Step 7: FFS on performance metric. FFS on how to specify the performance metric.

In our companion paper [4], it is proposed to use known PagingID and CFRA in the REFSENS test procedure. An example of REFSENS test procedure with CFRA is described in Figure 3.

Figure 3: Example of REFSENS test procedure with CFRA

And the timing relationship between R2D and D2R is given by Figure 4 where the nominal requirement is TR2D. Then if the impact SFO is further considered, the device’s correct response for Msg 1 in inventory procedure should be received by reader within the timing rage of Tmin and Tmax. Regarding how to define Tmin and Tmax can be found in [4].

Figure 4: Example of timing relationship between R2D and D2R

With above, the Step 4 - Step 7 of test procedure for receiver RF requirements can be further modified as below:

- …

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. TE send A-IoT Paging with Paing ID in CFRA mode. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 5: Determine whether TE can receive the correct response in D2R channel within timing window, Tmin and Tmax.

- Step 6: Repeat Step 3-5 until complete all the tests.

- Step 7: Determine whether the device can pass the REFSENS requirements such as 90 % success rate and 10% miss detection rate.

Proposal 5: The Step 4- setup 7 of test procedure of reference sensitivity for A-IoT device 1 can be modifed as below:

- …

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. TE send A-IoT Paging with Paing ID in CFRA mode. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 5: Determine whether TE can receive the correct response in D2R channel within timing window, Tmin and Tmax.

- Step 6: Repeat Step 3-5 until complete all the tests.

- Step 7: Determine whether the device can pass the REFSENS requirements such as 90 % success rate and 10% miss detection rate.

Similar as performance metric of backscattering testing, to simplify the test procedure, If EIS is selected as the performance metric for backscattering loss testing, the test direction should be declared by device to avoid the 3D scan to find the peak/best direction.

Proposal 6: If EIS is selected as the performance metric for backscattering loss testing, the test direction should be declared by device to avoid the 3D scan to find the peak/best direction.

## 2.4 Energy source and energy harvesting

In [1], the following agreements were reached for energy source and energy harvesting:

Issue 4-6-1: energy source and energy harvesting Agreement: - Device declaration on the required energy conditions. - External energy source directly connected to the device with cable and/or battery is excluded.

In Step 3 of test procedure for both Tx and Rx RF requirements, it is mentioned that device should have enough energy to complete the test according to device declaration on the required energy conditions.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

While it is not very clear about what would the required energy conditions declared by device. As captured in TR 38.769, the ON-OFF pattern for the device should be known during the testing as there is no specified device behavior regarding when device wakes up after fully charging and how long the device can be in ON status after starting transmission.

From AIoT testing PoV, at least the time to complete charging, and the duration the device can remain ON after charging should be declared. FFS on other needed device declaration information.

Proposal 7: From AIoT testing PoV, at least the time to complete charging, and the duration the device can remain ON after charging should be declared. FFS on other needed device declaration information.

# 3  Conclusion

In this paper, we provide our view on the remaining issues for A-IoT testability, and the following proposals are made:

Proposal 1: To reuse the OTA test system for legacy TRP and TRS measurement, only one fixed separation between the reader antenna and CW node should be implemented. The separation value is FFS. RAN4 to check whether the link antenna in Figure 1 and Figure 2 can be used to transmit CW signal.

Proposal 2:  It is proposed to take [17dB] as the starting point for the isolation between CW node and measurement antenna.

Proposal 3: In Step 4 of test procedure for backscattering loss, the test configuration for input power level of CW signal should be decided together with core requirement definition for backscattering loss. Additionally, the power level of R2D should be set as sensitivity+[X]dB. Take X = [3] as the starting point.

Proposal 4: If EIRP is selected as the performance metric for backscattering loss testing, the test direction should be declared by device to avoid the 3D scan to find the peak/best direction.

Proposal 5: The Step 4- setup 7 of test procedure of reference sensitivity for A-IoT device 1 can be modifed as below:

- …

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. TE send A-IoT Paging with Paing ID in CFRA mode. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 5: Determine whether TE can receive the correct response in D2R channel within timing window, Tmin and Tmax.

- Step 6: Repeat Step 3-5 until complete all the tests.

- Step 7: Determine whether the device can pass the REFSENS requirements such as 90 % success rate and 10% miss detection rate.

Proposal 6: If EIS is selected as the performance metric for backscattering loss testing, the test direction should be declared by device to avoid the 3D scan to find the peak/best direction.

Proposal 7: From AIoT testing PoV, at least the time to complete charging, and the duration the device can remain ON after charging should be declared. FFS on other needed device declaration information.

# 4. References

- R4-2505230, “WF on A-IOT device requirements”, CMCC, April, 2025

- TS 38.161

- TS 38.871

- R4-2506162, ‘REFSENS procedure, SFO and timing aspects’, Qualcomm, May, 2025