# 3GPP TSG-RAN WG4 Meeting #115	R4-2505743

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.14.3.2

Source: 		Qualcomm Incorporated

Title: 	On FR1 NTN devices OTA

Document for:	Approval

# Introduction

In RAN4#114bis, the agreements and open issues for FR1 NTN OTA test method are captured in WF [1]. In this paper, we provide further views on OTA test method and performance metric for FR1 NR NTN and IoT NTN devices.

# 2. Discussion

The usage scenario and OTA performance metric for NTN handheld UE has been discussed in several previous RAN4 meetings and the following agreements were reached in RAN4#114bis [1].

Issue 2-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: o For IoT-NTN and NR-NTN, typical usage scenarios are: § Scenario 1: both head+hand mode and hand only mode. § Scenario 2: hand only mode targeted for specific UE orientation usage § Scenario 3: hand only mode targeted for arbitrary UE orientation usage o UE shall declare support for only one scenario per band. The declaration details of each scenario should be further discussed. o For IoT-NTN and NR-NTN handheld UEs declared Scenario 2, the performance metrics are partial sphere metric. FFS details o For IoT-NTN and NR-NTN handheld UEs declared Scenario 3, the performance metrics are full sphere TRP and TRS

Satellite operators’ preference on the usage of NR-NTN/IoT NTN handheld UE is to make sure NTN UE to be close to normal NR UE. Therefore, for UE supporting Scenario 1, it was agreed to use the legacy full sphere TRP and TRS as the performance metric for NR-NTN and IoT NTN handheld UE. Hence there should not be any limitations on the usage for scenario.

For UE only supporting Scenario 2, UE needs to be guided with the help of Pointing App to point toward the satellite to ensure successful data transmission. And the Pointing App will help users orient the devices. It is understood that for Scenario 2, there should be a product manual to guide users on how to use Pointing App or similar software.

Observation 1: For Scenario 2, there should be a product manual to guide users on how to use Pointing App or similar software to point toward the satellite to ensure successful data transmission.

Different from Scenario 2, since Scenario 3 is to use in hand only mode targeting for arbitrary UE orientation usage, there is no such Pointing App.

In order to ensure that the DUT's declaration of usage scenarios in OTA conformance testing is consistent with its actual usage scenarios, the following are the recommendations for declaration:

- DUT shall declare to support only one scenario

- The declaration of scenario applicability shall be consistent with product manual usage scenario

- The declaration of scenario shall be clearly stated in the test report

Proposal 1: RAN4 to adopt the following rules for scenario declaration in NTN OTA testing:

- DUT shall declare to support only one scenario

- The declaration of scenario applicability shall be consistent with product manual usage scenario

- The declaration of scenario shall be clearly stated in the test report

For Scenario 2, since Pointing App is used to guide users to point the device toward satellite, therefore the device should be able to align its maximum direction as closely as possible to the satellite. Hence, the performance metric for Scenario 2 could use a conical range centered on the maximum EIRP/EIS direction as the metric. The maximum direction of EIRP/EIS can be based on the measurement or UE declaration.

Proposal 2: RAN4 should consider defining the partial TRP/TRS with a conical range centred on the maximum EIRP/EIS direction for NTN UE supporting Scenario 2. The maximum direction of EIRP/EIS can be based on the measurement or UE declaration.

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 NTN devices. The following observation and proposals are made:

Observation 1: For Scenario 2, there should be a product manual to guide users on how to use Pointing App or similar software to point toward the satellite to ensure successful data transmission.

Proposal 1: RAN4 to adopt the following rules for scenario declaration in NTN OTA testing:

- DUT shall declare to support only one scenario

- The declaration of scenario applicability shall be consistent with product manual usage scenario

- The declaration of scenario shall be clearly stated in the test report

Proposal 2: RAN4 should consider defining the partial TRP/TRS with a conical range centred on the maximum EIRP/EIS direction for NTN UE supporting Scenario 2. The maximum direction of EIRP/EIS can be based on the measurement or UE declaration.

# 4.	References

- R4-2504763, WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3, vivo