# 3GPP TSG-RAN WG4 Meeting #115		  R4-2505744

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.15.3

Source:	Qualcomm Incorporated

Title:	On remaining issue for FR2 OTA test method on STxMP

Document for:	Approval

# 1  Introduction

The WF on FR2 OTA test method on STxMP was agreed in [1]. The only remaining issue is how to define the additional Test Tolerance due to limited choices for AoA1&AoA2. In this paper, we provide our views on this issue for STxMP testing.

Issue 2-1-1: How to accommodate the difference from the core requirement derivation and test system restrictions including finite resolution. - Agreement: - The value for the uncertainty source of systematic error related to beam peak search is set as 0. The impact due to finite testing directions including limited choices for AoA1&AoA2 test directions can be handled in an additional Test Tolerance, TTFiniteRes. This additional TT should be determined in RAN4 and then communicated to RAN5 as recommended value to be added to the TT determined as a function of MU, i.e., TTTotal = x*MUTotal+ TTFiniteRes where x is determined in RAN5.

# 2  Discussion

It was confirmed that test setup for multi-Rx specified in [2] can be reused for STxMP testing. For minimum peak EIRP testing, there are the following two options on how to select AoA separation and UE orientation:

- Option 1: AoA1 direction is reused from legacy beam peak direction for the declared UE orientation. AoA2 direction is offset from AoA1 declared by UE selecting from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system.

- Option 2: Alternative direction of AoA1 is declared by UE. AoA2 direction is to use AoA+ or AoA- by UE declaration selecting from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system.

For both Option 1 and Option 2, it can be observed that the limitation on the AoA pair selection applies. For example, for Option 1, if AoA1 direction is reused from legacy beam peak direction, then AoA2 direction can only be selected from potential AoA separation set of {30deg, 60deg, 90deg, 120deg, 150deg}. Similar for Option 2, when AoA1 is declared by UE, AoA2 direction can be selected only from the set of 30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system.

Observation 1: The additional Test Tolerance, TTFiniteRes, applies for both Option 1 and Option 2 AoA separation selection.

In [1], it was agreed that RAN4 will determine the TTFiniteRes by evaluation results and analysis. In theory, the impact on EIRP and TRP due to limited AoA pair should be different. For EIRPmax and TRPmax, based on test method defined in TR38.771 [3], the two sets of AoA separation and UE orientation needs to be tested:

-	The AoA separation and UE orientation declared by UE for EIRPPUMAX,f,c,k testing.

-	One of AoA is at the beam peak direction of single CC operation without STxMP enabled, the other AoA is at the direction decided by UE declared orientation (listed in TS 38.101-2) and 30deg AoA separation.

If there is no limitation on the AoA1 and AoA2, a smaller AoA separation than 30deg could be selected and it might lead to higher aggregated EIRP and TRP values for EIRPmax and TRPmax testing.

Proposal 1: RAN4 to future study the applicability rule of TTFiniteRes for min peak EIRP per TCI, EIRPmax and TRPmax. RAN4 can revisit the details when the STxMP RF core requirements are completed.

It is known that, for both Option 1 and Option 2, AoA1 is selected through either legacy beam peak direction or UE declaration. Therefore, the additional TT shall only apply for either AoA1 or AoA2. With that, we have the following two proposals:

Proposal 2: The additional Test Tolerance, TTFiniteRes, shall only apply for one AoA EIRP requirements.

# 3  Conclusion

In summary, we have following observation and proposals:

Observation 1: The additional Test Tolerance, TTFiniteRes, applies for both Option 1 and Option 2 AoA separation selection.

Proposal 1: RAN4 to future study the applicability rule of TTFiniteRes for min peak EIRP per TCI, EIRPmax and TRPmax. RAN4 can revisit the details when the STxMP RF core requirements are completed.

Proposal 2: The additional Test Tolerance, TTFiniteRes, shall only apply for one AoA EIRP requirements.

# 4  References

[1] R4-2504727, ‘WF for [114bis][329] NR_FR2_OTA’, Qualcomm

[2] 3GPP TR 38.871

[2] 3GPP TR 38.771