3GPP TSG-RAN WG4 Meeting #115			                                R4-2505750

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.14.3.2

Source:	Xiaomi

Title:	Discussion on test methodology and radiated performance metric for FR1 NTN devices

Document for:	Discussion

- Introduction

The test methodology and radiated performance metric for FR1 NTN devices has been extensively discussed in previous meetings. The latest progress was captured in the WF [1]. This contribution continues to provide our views on these remaining issues.

Issue 2-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: o For IoT-NTN and NR-NTN, typical usage scenarios are: § Scenario 1: both head+hand mode and hand only mode. § Scenario 2: hand only mode targeted for specific UE orientation usage § Scenario 3: hand only mode targeted for arbitrary UE orientation usage o UE shall declare support for only one scenario per band. The declaration details of each scenario should be further discussed. o For IoT-NTN and NR-NTN handheld UEs declared Scenario 2, the performance metrics are partial sphere metric. FFS details o For IoT-NTN and NR-NTN handheld UEs declared Scenario 3, the performance metrics are full sphere TRP and TRS

- Discussion

Performance metric for scenario 2

For those UEs supporting scenario 2, RAN4 agreed to define partial sphere metrics as the performance metric, and FFS the detail. From our perspective, the directional antenna design is more likely to be deployed in order to guarantee the UL link-budget, thus, it is reasonable to consider a certain angular range which is declared by OEMs. And for the data processing approach for the performance metric, two options were proposed as the performance metric in [2], and our preference is option 2, e.g. Outage value corresponding to the Nth percentile of the CDF of EIRP/EIS over the declared range of angles.

Proposal 1: RAN4 to define partial sphere metrics within a certain angular range which is declared by OEMs as the performance metric.

Proposal 2: For the data processing approach for the performance metric, RAN4 to define the outage value corresponding to the Nth percentile of the CDF of EIRP/EIS over the declared range of angles.

3	Conclusions

In this paper, we provided our views on the remaining issues on test methodology and radiated performance metric for FR1 NTN devices and our proposal as follows:

Proposal 1: RAN4 to define partial sphere metrics within a certain angular range which is declared by OEMs as the performance metric.

Proposal 2: For the data processing approach for the performance metric, RAN4 to define the outage value corresponding to the Nth percentile of the CDF of EIRP/EIS over the declared range of angles.

4 References

[1] R4-2504763, WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3, Vivo

[2] R4-2500323, On NTN TRP TRS test methodology, Apple